3GPP TSG-RAN WG4 Meeting #116bis	R4-2513954

Prague, Czech, 13th – 17th October 2025

Agenda Item:	7.5.4

Source:	Ericsson

Title:	On demodulation requirements for 6Rx reception with interference mitigation

Document for:	Discussion

# 1	Introduction

In RAN#109 meeting, a new WID on NR demodulation performance: Phase 6 was approved [1]. According to the WID, following are the objectives for UE part:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

With Rel-19 introducing UE RF enhancements supporting 6Rx reception, a consistent methodology for modeling both inter-cell interference and intra-cell inter-user interference becomes increasingly important. Existing Rel-17 and Rel-18 test models (INR-based interference representation) provide a mature framework for benchmarking but were defined mainly for 2/4/8Rx UE categories. It is natural to extend the interference mitigation solution to 6Rx UE/devices.

In this contribution, we provided our views on the scope of demodulation and CSI reporting requirements for two different interference scenarios, together with the discussion on the interference model and detailed parameter assumptions.

# 2	Inter-cell interference

In this section, we discuss the scenario of inter-cell interference cancellation with 6Rx reception.

## 2.1	Test scope

Regarding the test scope, following the guidance of the WID, RAN4 will define PDSCH demodulation for the inter-cell interference scenario. As for the related CQI reporting requirements, a conclusion from WI Rel-19 NR demodulation performance: Phase 5 is needed that is whether to define CQI reporting requirements for 6Rx (without interference). In this case, for the test scope, we propose the following:

Proposal 1: Define PDSCH demodulation requirements for inter-cell interference scenario

Proposal 2: Postpone the discussion on the CQI reporting requirements for inter-cell interference scenario

## 2.2	Inter-cell interference model

As for the inter-cell interference model, we propose to reuse the model used for Rel-17/18 NR performance enhancement that contains two interfering cells for initial evaluation purposes.

In Rel-17 and Rel-18, the interference model that RAN4 agreed to use was defined in B.6 in TS38.101-4 [2], where the interference to noise ratio (INR) is defined as follows:

Each interfering cell involved in PDSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value: INRi=j=1NRXIor(i)(j)j=1NRXNoc(j) where Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc is the average power spectral density of a white noise source consistent with definition provided in section [4.4.3/4.5.3].

Reuse the INR method that considered back in Rel-17 to determine the interference profile. Following exact INR values for two different interference cells can be reused for performing initial evaluation:

| Parameter | Unit | Interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| INR | dB | 7.77 | 2.29 |

Specifically, as described in B.6.2 in TS38.101-4 [2], the interference model can be assumed as that in each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. Transmitted physical channels shall include SSB and TRS/CSI-RS as specified in the requirements section.

Regarding the transmission rank for the interference cells, legacy configurations are shown as follows:

| Parameter | Unit | interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| Transmission rank | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |

In Rel-19 with 8Rx reception, we propose to also consider rank 1 and rank 2 mixed with probabilities as specified above for the transmission rank of the interfering cells since higher rank interference tend to appear as white noise.

As for the precoding of interfering PDSCH, we propose to reuse the method that is to assume random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

We also propose to use 16QAM randomly modulated symbols for the modulation order of interfering PDSCH.

Proposal 3: Assume the following for the interference model:

- Number of interference cell: 2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- INR: 7.77 for interference cell 1, 2.29 for interference cell 2

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

## 2.3	PDSCH requirement and parameters

Following the objectives, we propose to take the legacy Rel-17 inter-cell interference parameter configurations as baseline and do the evaluation. Then, make adjustment accordingly to fit more for defining PDSCH requirement with inter-cell interference for 6Rx reception.

Following the previous approach, we propose to define the requirements for both FDD SCS=15kHz and TDD SCS=30kHz.

Proposal 4: Consider following parameter assumptions for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

As for the rank, antenna, MCS and channel model, RAN4 can take the following existing requirements as a reference:

Table 2.3-1 Existing requirements in TS38.101-4

|  | Rank | MCS | Antenna | Propagation |
| --- | --- | --- | --- | --- |
| Rel-17 PDSCH requirements with inter-cell interference, 2/4Rx | 1 | 13 | 2x2, 2x4 ULA Low | TDLC300-100 |
|  | 1 | 13 | 2x2, 2x4 ULA Low | TDLA30-10 |
| Rel-19 PDSCH requirements with interference, 8Rx | 2 | 17 | 2x8, ULA low | TDLA30-10 |
|  | 4 | 13 | 4x8, ULA low | TDLA30-10 |
| Rel-19 PDSCH requirements, 6Rx | 4 | 17 | 4x6, Correlation TBD | TDLA30-10 |

In the meantime, we would prefer to consider only one case to cover both handheld UEs and FWA devices as RAN4 usually consider ULA low correlation for interference mitigation requirements.

Based on the above existing requirements and one-case principle, we propose the following cases for evaluation purpose, down-selection could be applied depending on the performance and further discussions.

Proposal 5: Consider following cases for initial evaluations:

| Eva. num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 1 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 2 | 2 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 3 | 4 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 4x6, low | 70 |  |

For the other parameter assumptions, we propose the following:

Proposal 6: Other PDSCH parameters:

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interference

UE capability and test applicability can be discussed and decided (if necessary) after RAN4 reaches agreements on the scope and basic parameter assumptions.

# 3	Intra-cell inter-user interference

In this section, we discuss the scenario of intra-cell inter-user interference cancellation with 6Rx reception.

## 3.1	Inter-user interference model

As for the intra-cell inter-user interference model, we propose to first reconsider the model used for Rel-17 NR performance enhancement and do some evaluations. Following aspects are within the pool of priority that need to be discussed so that to determine the overall interference model.

Number of paired UEs

Reuse the legacy pairing UE model to set up one target UE and one co-scheduled (interfering) UE. Since we are taking MMSE-IRC receiver as the baseline receiver, the target UE does not need to know the number of co-scheduled UEs and their ranks when it is processing in demodulation. Based on this, we propose to consider only 1 co-scheduled UE but with different layers in different test cases.

Proposal 7: Consider only 1 co-schedule UE in the interference model

Rank for target and interference PDSCH

According to the WID, the rank for each user should be no more than 2, which means that at most RAN4 can only consider a rank combination of 2+2. Considering the real network deployment scenario, it’s also likely to configure a hybrid rank combination for target and interfering UE(s) if the Network can hardly find two paired UEs with the same rank configuration. Thus, we propose to evaluate following possible rank combinations and further check the results before making any down-selection for the purpose of introducing requirements.

Proposal 8: Consider following rank configuration for initial evaluations

- For a total of 2 layers

  - Target UE + interference UE:

    - 1+1

- For a total of 4 layers

  - Target UE + interference UE:

    - 2+2

    - 2+1

    - 1+2

Antenna configuration

As indicated in the objectives of the WID, new PDSCH requirement will be introduced for 6Rx UEs/devices. Similar to the antenna configuration used in Rel-19 UE RF enh WI, we propose to consider 2Tx and 4Tx so that the test case design is suggested to be based on 2T6R and 4T6R.

Proposal 9: Consider following antenna configurations:

- 2T6R

  - For rank 1+1

- 4T6R

  - For rank 2+2, 2+1, 1+2

Codebook type

As for the codebook type, we prefer to reuse the assumption of Rel-17 intra-cell interference cancellation that is Type I single panel codebook so that the precoding configuration can be assumed as randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [3].

Proposal 10: Consider Type I Single Panel only

PMI selection and precoding matrix generation

We think the precoding method used for Rel-18 NR demodulation performance evolution WI can be reconsidered, which is stated as follows:

- Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Proposal 11: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

PRB bundling size and precoding granularity

As for the PRB bundling size and precoding granularity, we prefer to 2PRBs for frequency domain and per slot for time domain, which is the same assumption as the existing UE demodulation requirements.

Proposal 12: 2PRBs for PRB bundling size and precoding granularity

MCS for interfering PDSCH

We propose to reuse the assumption for Rel-17 NR demodulation performance evolution WI which is the following:

- Random 16QAM symbols

Proposal 13: Assume Random 16QAM symbols for interfering PDSCH

## 3.2	PDSCH requirement and parameters

Following the objective, we propose to take the legacy Rel-17 intra-cell inter-user interference parameter as the baseline and do the evaluation, then update it with proper parameter assumptions if needed for the purpose of introducing PDSCH requirement.

For the antenna and channel model, following the principal of one-case covering both handheld UEs and FWA devices, we propose to consider the following:

Proposal 14: Consider following antenna and channel model assumptions

- Antenna configuration

  - 2T6R ULA low for rank combination 1+1

  - 4T6R ULA low for rank combination 1+2, 2+1 and 2+2

- MCS for target UE for evaluation

  - MCS17, MCS13

For other PDSCH parameters for intra-cell inter-user interference scenario, we propose to consider following assumptions:

Proposal 15: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- MCS for target UE for evaluation

  - MCS17, MCS13

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

# 4	Summary

In this contribution, we shared our views on the test scope, interference model and parameter assumptions for inter-cell interference and intra-cell inter-user interference scenarios. Our proposals are summarized as follows:

Proposal 1: Define PDSCH demodulation requirements for inter-cell interference scenario

Proposal 2: Postpone the discussion on the CQI reporting requirements for inter-cell interference scenario

Proposal 3: Assume the following for the interference model:

- Number of interference cell: 2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- INR: 7.77 for interference cell 1, 2.29 for interference cell 2

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

Proposal 4: Consider following parameter assumptions for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

Proposal 5: Consider following cases for initial evaluations:

| Eva. num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 1 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 2 | 2 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 3 | 4 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 4x6, low | 70 |  |

Proposal 6: Other PDSCH parameters:

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interference

Proposal 7: Consider only 1 co-schedule UE in the interference model

Proposal 8: Consider following rank configuration for initial evaluations

- For a total of 2 layers

  - Target UE + interference UE:

    - 1+1

- For a total of 4 layers

  - Target UE + interference UE:

    - 2+2

    - 2+1

    - 1+2

Proposal 9: Consider following antenna configurations:

- 2T6R

  - For rank 1+1

- 4T6R

  - For rank 2+2, 2+1, 1+2

Proposal 10: Consider Type I Single Panel only

Proposal 11: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

Proposal 12: 2PRBs for PRB bundling size and precoding granularity

Proposal 13: Assume Random 16QAM symbols for interfering PDSCH

Proposal 14: Consider following antenna and channel model assumptions

- Antenna configuration

  - 2T6R ULA low for rank combination 1+1

  - 4T6R ULA low for rank combination 1+2, 2+1 and 2+2

- MCS for target UE for evaluation

  - MCS17, MCS13

Proposal 15: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- MCS for target UE for evaluation

  - MCS17, MCS13

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

# References

[1]	RP-252957: WID on NR demodulation performance: Phase 6, China Telecom, BT, RAN#109, Beijing, China

[2]	TS38.101-4

[3]	TS38.214