3GPP TSG-RAN WG4 Meeting #116bis	R4-2513957

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.1.2

Source:	Ericsson

Title:	Simulation results for 6Rx SDR requirements

Document for:	Information

# 1	Introduction

In RAN4 #114 meeting, companies agreed to introduce 6Rx SDR requirements with 2-layers. In RAN4 #115, it’s been agreed to determine the MCS value for 1024QAM, 256QAM and 64QAM with scaling factor 1. In this contribution, we provided our simulation results for the purpose of facilitating those MCS values.

# 2	Simulation results

In this section, we first exhibit the parameter assumptions that we used for the SDR simulations. Then, we provided our simulation results showing the SNR point for different MCS values.

## 2.1	Simulation assumption

We followed table 5.5A-1 in TS38.101-4 and changed the antenna to 2T6R, 4T6R and 8T6R for simulations. Corresponding static channel precoding we have used the agreed matrixes in the WF [1].

## 2.2	Simulation results

As agreed in RAN4 #116 meeting in Bengaluru, RAN4 should determine the following TBD values for each modulation order.

Issue 3-1-4: SDR requirements - Agreement - Decide the MCS values as per the agreed static channel matrix in Issue 3-1-1 and parameters in Table 5.5A-1/2/3 of TS 38.101-4 in the next meeting - QPSK/16QAM/64QAM/256QAM with 6 MIMO layers as below table Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6; 8; 1; TBD. 6; 8; 0.8; 23. 6; 8; 0.75; 22. 6; 8; 0.4; 12. 6; 6; 1; TBD. 6; 6; 0.8; 24. 6; 6; 0.75; 23. 6; 6; 0.4; 14. 6; 4; 1; 16. 6; 4; 0.8; 16. 6; 4; 0.75; 16. 6; 4; 0.4; 11. 6; 2; 1; 9. 6; 2; 0.8; 9. 6; 2; 0.75; 9. 6; 2; 0.4; 5. Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12]. - 1024QAM with 2 MIMO layers as below table Supported RX antenna ports; Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6RX; 2; 10; 1; TBD. 2; 10; 0.8; 21. 2; 10; 0.75; 19. 2; 10; 0.4; 9. Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5.

For the purpose of determining the MCS value for 64QAM, 256QAM and 1024QAM, we have conveyed the following simulation results.

Table 2.2-1 Simulation results for 64QAM/256QAM/1024QAM

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 8Tx6Rx | 6 | 6 (64QAM) | Table 5.1.3.1-1: MCS index table 1 | 28 | 1 | 22.9 |
|  |  |  |  | 27 |  | 20.8 |
|  |  |  |  | 26 |  | 18.7 |
|  |  |  |  | 25 |  | 17.7 |
|  | 6 | 8 (256QAM) | Table 5.1.3.1-2: MCS index table 2 | 27 |  | N/A |
|  |  |  |  | 26 |  | N/A |
|  |  |  |  | 25 |  | 25.3 |
|  |  |  |  | 24 |  | 23.6 |
|  |  |  |  | 23 |  | 21.8 |
| 2Tx6Rx | 2 | 10 (1024QAM) | Table 5.1.3.1-4: MCS index table 4 | 24 |  | 25.9 |
|  |  |  |  | 23 |  | 23.8 |

# 3	Evaluation for 1024QAM with 4-layer

In this section, we mainly evaluate the performance of 1024QAM with 4 MIMO layers.

Figure 3.1-1 PDSCH performance of 1024QAM with 4 MIMO layers

# 4	Summary

In this contribution, we shared the simulation results for PDSCH demodulation and SDR for 6Rx UE demodulation.

# References

[1]	R4-2512617: Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #116, Bengaluru, India