3GPP TSG-RAN WG4 Meeting #116bis	R4-2513975

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on the 2-sided CSI

Document for:	Discussion

# Introduction

RAN#109 approved the revision of WID on AI/ML for NR air interface Phase 2 [1]. According to the core part objectives, RAN4 should focus on Fully defined/specified reference model in Inter-vendor training collaboration where RAN plenary requires feedback from RAN4 by RAN#110. This objective also includes Specification of standardized dataset format/content plus dataset exchange. However, this objective is led by RAN1, and RAN1 has already started to specify the dataset format/content for inter-vendor training collaboration.

This contribution mainly discusses 1) the specification of standardization encoder model structure in the inter-vendor training collaboration and 2) interoperability and testability of the two-sided CSI compression feature.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

# Inter-vendor training collaboration

We first summarize the difference/similarity of ‘inter-vendor training collaboration’ discussed in RAN1 and ‘interoperability’ discussed in RAN4.

## RAN1 Direction A and Direction C

In Rel-20 WI, RAN1 are discussing three options to specify the two-sided CSI compression features:

- Direction C

- Direction A sub-option 3a-1

- Direction A sub-option 4-1

In our understanding, the purpose of these directions is how the UE-side develops their own encoder used in the commercial network.

Direction C:

Direction C is the fully standardized reference model, which includes both model structure (backbone) and parameters. There are three sub-options for fully standardized reference models depending on whether encoder and/or decoder is specified:

- 1-1: Only reference encoder is specified.

  - NW-side and/or UE-side may train their actual CSI generation part (encoding) and actual CSI-reconstruction part (decoding) separately compatible to the reference encoder.

- 1-2: Only reference decoder is specified.

  - NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

- 1-3: Both reference encoder and reference decoder are specified.

  - NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible with the reference decoder/encoder.

Direction A sub-option 3a-1

Direction A sub-option 3a-1 specifies the standardized encoder model structure. It is assumed the encoder parameters are exchanged from NW-side to UE-side. The Rel-20 WID states that the standardized encoder structure is the same as in Direction C (without parameters).

This is one example of how UE-side develops their encoder (Alt-1):

- NW-side first trains a pair of nominal encoder and NW decoder,

- NW-side shares the nominal encoder parameters,

- UE-side develops a UE test decoder against the exchanged nominal encoder,

- UE-side develops UE encoder against the UE test decoder.

RAN1 is also considering another way to develop the UE-side encoder (Alt-2):

- NW-side first trains a pair of nominal encoder and NW decoder,

- NW-side shares the nominal encoder parameters,

- UE-side develops UE encoder directly based on the nominal encoder input/output.

Direction A sub-option 4-1

Direction A sub-option 4-1 specifies the standardized dataset format/contents and it is assumed the training dataset is exchanged from NW-side to UE-side. RAN1 has agreed that the dataset consists of at least ‘target CSI’ and ‘CSI feedback’, where the ‘target CSI’ corresponds to the channel estimates before encoding and after decoding, and the ‘CSI feedback’ corresponds to the output of the encoder that will be transmitted from UE with CSI reporting.

This is an example of how UE-side develops their encoder (Alt-1):

- NW-side first trains a pair of NW encoder and NW decoder,

- NW-side shares the training dataset including target CSI and CSI feedback to UE-side,

  - NW-side also shares the performance target, pairing ID, etc.,

- UE-side develops the UE test decoder from the shared dataset,

- UE-side develops the UE encoder based on the UE test decoder and shared dataset.

RAN1 is also considering another way to develop the UE-side encoder (Alt-2):

- NW-side first trains a pair of NW encoder and NW decoder,

- NW-side shares the training dataset including target CSI and CSI feedback to UE-side,

  - NW-side also shares the performance target, pairing ID, etc.,

- UE-side develops the UE encoder directly based on the target CSI and CSI feedback in the shared dataset.

In Rel-20 WI, RAN1 discusses the inter-vendor training collaboration, where RAN1 focuses on how to specify the two-sided CSI compression feature in the specification. As mentioned in TR [2], RAN1’s discussion has an assumption that UE vendors and NW vendors can exchange model parameters and datasets.

Observation 1: RAN1’s inter-vendor training collaboration discussion assumes that UE vendors and NW vendors can exchange model parameters and datasets.

## RAN4 Option 3 and Option 4

Table 1 summarizes our understanding of what to be captured in 3GPP to ensure the interoperability and testability of two-sided CSI compression function, although it has not been agreed in RAN4 yet.

Table 1	Summary of what to be captured in 3GPP for testability and interoperability of two-sided CSI compression functions.

|  | Option 3 | Option 4a | Option 4b |
| --- | --- | --- | --- |
| What is standardized | Test decoder | Dataset with the latent space | Reference encoder |
| What to be ‘captured’ | - Standard test decoder (structure and parameters) - Dataset | - Reference encoder (structure and parameters) - Dataset with the latent space - Reference decoder (structure) | - Reference encoder (structure and parameters) - Dataset - Reference decoder (structure) |
| TE vendor | - Use the standard test decoder to decode CSI from the UE encoder. | - Train both their test decoder and a reference encoder with the dataset (with latent space). - Use their test decoder to decode CSI from the UE encoder. | - Train their test decoder towards the standard reference encoder with the dataset. - Use their test decoder to decode CSI from the UE encoder. |

In Rel-19 SI, since the main task of RAN4 was to ensure testability, RAN4 mainly discussed the feasibility of testability of two-sided CSI compression feature, that is, how TE vendors develop their own test decoder without exchanging information between TE vendors and UE vendors. For this purpose, RAN4’s Options 3 and 4 need to capture more information (e.g., dataset, parameters) compared with RAN1 Directions.

Observation 2: The purpose of RAN4’s testability and interoperability discussed in Rel-19 SI was to make TE vendors develop their own test decoder without exchanging information between TE vendors and UE vendors.

## Fully defined/specified reference model

According to the WID objective of inter-vendor training collaboration, RAN4 needs to send feedback on fully defined/specified reference model (“Direction C”). In our understanding, RAN1 Direction C Reference decoder is equivalent to the test decoder discussed in RAN4 Option 3.

Observation 3: RAN1 Direction C Reference decoder (structure and parameters) is equivalent to the test decoder discussed in RAN4 Option 3.

Proposal 1: RAN4 should confirm the reference decoder discussed in RAN1 Direction C Reference decoder is equivalent to the test decoder (structure and parameters) discussed in RAN4 Option 3.

Moreover, RAN1 Direction A sub-option 3a-1 ‘standardized encoder model structure’ is equivalent to the reference encoder structure discussed in RAN4 Option 4a/4b. However, we should emphasize that RAN4 option 4a/4b also requires the dataset to train the reference encoder and TE-side decoder.

Observation 4: RAN1 Direction A sub-option 3a-1 ‘standardized encoder model structure’ is equivalent to the reference encoder structure (without parameters) discussed in RAN4 Option 4a/4b.

Proposal 2: RAN4 should confirm the encoder model structure discussed in RAN1 Direction A sub-option 3a-1 is equivalent to the reference encoder structure (without parameters) discussed in RAN4 Option 4a/4b.

The WID objective mentions RAN4 should consider RAN1 scalability study outcome. In our understanding, the model scalability means the CSI compression model works for any Tx antenna port configuration, subband size configuration, and encoder output size configuration (e.g. latent space size, latent quantization size). However, the testability study conducted in Rel-19 RAN4 SI only considered the fixed Tx antenna configuration (32 ports), fixed subband size (i.e. 13 subbands), and fixed latent data format. We think it is important to ensure the one reference encoder and test decoder models captured in 3GPP support the scalability.

Observation 5: Rel-19 SI testability study in RAN4 did not consider the model scalability, that is, RAN4 fixed the number of Tx ports, subband size, and latest space data format.

Proposal 3: RAN4 should continue the CSI compression testability study considering model scalability in Rel-20 WI.

Moreover, according to the WID [1], Direction A, sub-option 3a-1 should leverage the defined/reference model of Direction C. Hence, at least an encoder (structure and parameters) needs to be specified. Furthermore, the WID describes Direction C as “Fully defined/specified reference model”, which we interpret at both the encoder and decoder to be specified.

## Specification of standardized dataset format/content

As we discussed in Direction A, sub option 4-1, RAN1 has already started the discussion, and they have agreed that the dataset consists of at least target CSI and CSI feedback. We should also emphasize that the dataset format/contents discussed in RAN1 is not necessarily same as the dataset discussing in RAN4 Option 3 and Option 4 because the purpose of the dataset format/contents discussed in RAN1 is for inter-vendor collaboration, although the dataset discussed in RAN4 is used for TE test decider development. However, coordination between RAN1 and RAN4 may be beneficial to avoid double work. RAN4 can reuse the data format, although the content could be different.

Proposal 4: Dataset content used for RAN4 testability/interoperability (Option 3/4a/4b) is not necessarily same as the dataset content to be specified in RAN1 for inter-vendor collaboration.

Proposal 5: RAN4 should reuse the dataset data format discussed in RAN1.

In Rel-19 SI, RAN4 created the training dataset with one configuration such as 32 ports, fixed subband size, etc. In Rel-20 WI, when RAN4 captures the dataset in 3GPP, it should contain more data to ensure the scalability.

Proposal 6: Dataset captured in 3GPP should contain more data to ensure the scalability.

# Interoperability and testability

In Rel-19 SI, RAN4 mainly discussed the testability and interoperability issues for two-sided CSI compression, and RAN4#116 agreed with the guidance for Rel-20 WI discussion based on the Rel-19 SI discussion as follows [3].

- Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

## What/how to capture the model/dataset in 3GPP

As we summarized in Table 1, in our understanding, 3GPP needs to capture the encoder, decoder and dataset, so that TE vendors can deploy and verify a test decoder, possibly their own developed test decoder, without exchanging information between TE vendors and UE vendors.

One way to capture the model/dataset is to store the model/dataset themselves (binary data) in 3GPP database (e.g., FTP server, 3GPP Forge). In Rel-19 SI, for example, RAN4 has used ONNX (Open Neural Network Exchange) to exchange reference models.

We understand the ONNX is an open format to present machine learning models used in AI/ML area in 2025. However, since ONNX is developed by an open-source community, it is not ensured the community exists throughout the entire lifecycle of 5G.

Even if the community exists, 3GPP needs to be very careful to point towards the correct version and the version needs to not become obsolete the captured/standardized reference models. In case the ONNX community does not recommend using the current data format, 3GPP needs to update all the encoder/decoder models using new format, but it is not clear who is responsible for maintaining the models after the WI is complete.

Another way is to capture the model structure and dataset in the TS or TS. It could be possible to specify the model structure in the document (e.g., JSON-based serialized text data or data flow graph). On the other hand, it is not realistic to specify the model parameters (i.e., weights) and dataset in TS or in TR (MS Word document at least in NR specification) because it could be huge data. The parameters and dataset would then need to be stored in a separate file.

Table 2, Table 3, and Table 4 summarize our current views how to capture the models, model parameters, and datasets, respectively, in 3GPP.

Note the ‘3GPP server’ refers to FTP server or 3GPP Forge, for example.

Table 2: Options on how to capture the reference model structures

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like ONNX | 3GPP server |
| Option 2 | Text data (e.g., JSON or data-flow graph) | TS or TR |

Table 3: Options on how to capture the reference model parameters

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like ONNX (Together with model structure) | 3GPP server |
| Option 2 | Parameter data file (e.g., binary data or text data) | Parameter data format is specified in TS or TR Parameter data files are stored in 3GPP server |

Table 4: Options on how to capture the dataset

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like NumPy | 3GPP server |
| Option 2 | Dataset data file (e.g., binary data or test data) | Dataset data format/content are specified in TS or TR Dateset data files are stored in 3GPP server |

Proposal 7: RAN4 should discuss how to capture the reference encoder and reference decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model).

Proposal 8: RAN4 should discuss how to capture the reference encoder and reference decoder parameters (i.e., model weights).

Proposal 9: RAN4 should discuss the dataset format and content for testability and interoperability purposes.

# Summary

Observations:

Observation 1: RAN1’s inter-vendor training collaboration discussion assumes that UE vendors and NW vendors can exchange model parameters and datasets.

Observation 2: The purpose of RAN4’s testability and interoperability discussed in Rel-19 SI was to make TE vendors develop their own test decoder without exchanging information between TE vendors and UE vendors.

Observation 3: RAN1 Direction C Reference decoder (structure and parameters) is equivalent to the test decoder discussed in RAN4 Option 3.

Observation 4: RAN1 Direction A sub-option 3a-1 ‘standardized encoder model structure’ is equivalent to the reference encoder structure (without parameters) discussed in RAN4 Option 4a/4b.

Observation 5: Rel-19 SI testability study in RAN4 did not consider the model scalability, that is, RAN4 fixed the number of Tx ports, subband size, and latest space data format.

Proposals:

Proposal 1: RAN4 should confirm the reference decoder discussed in RAN1 Direction C Reference decoder is equivalent to the test decoder (structure and parameters) discussed in RAN4 Option 3.

Proposal 2: RAN4 should confirm the encoder model structure discussed in RAN1 Direction A sub-option 3a-1 is equivalent to the reference encoder structure (without parameters) discussed in RAN4 Option 4a/4b.

Proposal 3: RAN4 should continue the CSI compression testability study considering model scalability in Rel-20 WI.

Proposal 4: Dataset content used for RAN4 testability/interoperability (Option 3/4a/4b) is not necessarily same as the dataset content to be specified in RAN1 for inter-vendor collaboration.

Proposal 5: RAN4 should reuse the dataset data format discussed in RAN1.

Proposal 6: Dataset captured in 3GPP should contain more data to ensure the scalability.

Proposal 7: RAN4 should discuss how to capture the reference encoder and reference decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model).

Proposal 8: RAN4 should discuss how to capture the reference encoder and reference decoder parameters (i.e., model weights).

Proposal 9: RAN4 should discuss the dataset format and content for testability and interoperability purposes.

# References

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm.

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP.

- R4-2511849, “WF on [116][132] NR_AIML_air_part2”, Ericsson.

- Open Neural Network Exchange, https://onnx.ai/