3GPP TSG-RAN4 Meeting #116bis	R4-2513995

# Prague, Czech Republic 13th – 17th October 2025

Title:	Views on BS RF requirements for US4.9

Source:	Qualcomm Incorporated

Agenda item:	5.11.3

Document for:	Approval

# Introduction

To support public safety use of the 4940–4990 MHz range within the scope of 5G NR and in alignment with the FCC’s framework, the initiative required the definition of a new NR TDD operating band [1]. Through this paper we highlight the considerations from the 3GPP RAN4 BS towards the regulations in the FCC

# General discussion

In FCC document CFR Title 47 Part 90.210(m) and Part 90.1215, the SEM and maximum conducted output power are detailed. It is important to ensure that 3GPP requirements need to meet those values.

The SEM in CFR Title 47 Part 90.210(m) is specified as below:

Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows: (1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB. (2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB. (3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB. (4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB. (5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB. (6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation. (7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

The SEM limits of 3GPP spec are specified in TS 38.101-1 6.5.2.2, as reproduced below:

Table 6.5.2.2-1: General NR spectrum emission mask ΔfOOB (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 3; 5; 10, 15, 20, 25, 30, 35, 40, 45; 50, 60, 70, 80, 90, 100. ± 0-1; -13; -13; -13; 1 % of channel BW. ± 0-1; -24; 30 kHz. ± 1-5; -10; -10; -10; 1 MHz. ± 5-6; -25; -13. ± 6-10; -25. ± 5-BWChannel; -13. ± BWChannel-(BWChannel+5); -25.

As the measurement bandwidths and ΔfOOB are different for FCC and 3GPP, a comparison is not straightforward, hence in Figure 1, the SEM of a 20MHz CBW for a PC3 is plotted for FCC and 3GPP requirements. It can be observed that the FCC is significantly more stringent and a regular SEM requirement would not be enough, therefore a new network signalling is needed to ensure the FCC SEM requirements.

Figure 1 SEM evaluation: (left) 3GPP requirements, (right) FCC requirements

Similar to the UE, the BS OBUE is also absolute, as given in Section 6.6.4 in TS 38.104 and thus, mapping from the FCC mask and the BS OBUE needs to be carried out in RAN4.

Proposal 1: RAN4 to investigate the possibility of having a new OBUE specifically for the US4.9 band to incorporate the FCC requirements given in CFR Title 47 Part 90.210(m).

# Conclusion

In this contribution, our proposal for the BS RF for 4.9US new band is as follows:

Proposal 1: RAN4 to investigate the possibility of having a new OBUE specifically for the US4.9 band to incorporate the FCC requirements given in CFR Title 47 Part 90.210(m).

# References

[1] RP-251887, “New WID on Introduction of NR TDD 4.9GHz Band for US Operation”, Fujitsu, 3GPP TSG RAN Meeting #108, Prague, Czech Republic.

[2] R4-2510412, “On adjacent spectrum protection requirements for the 4940-4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India.

[3] R4-2510429, “Coexistence study for introduction of US 4.9GHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India.