3GPP TSG RAN WG4 Meeting #116bis	R4-2514008

Prague, CZ, 13th – 17th, Oct. 2025

# Source:	ZTE Corporation, Sanechips

# Title:	TP for PRD 01 to update band combination notation in clause 5.1

# Agenda Item:	5.2.6

# Document for:	Approval

- Introduction

The contents related to band combination notation in clause 5.1 in PRD [1] were based on TR38.862 [2]. However, when we comparing these two TRs, we can found the notations are not fully aligned. Since RAN4 will use PRD for the rules and guidelines and ways for band combination, we think this part should be updated according to keep alignment with the latest TR38.862.

In addition to that, to update the descriptions related to LTE-DC and NR-DC to align with the followed examples.

- Reference

- [1] Permanent Reference Document (PRD);Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, v0.4.0

- [2] TR38.862 Study on band combination handling in RAN4, v17.2.0

- Text Proposal

----- Start of TP -----

# 5	Guidelines of specifying band combinations

## 5.1	General

The notation of the band combinations in the TS 38.101 specifications as well as the combination request lists by the operators and the basket WIDs can have significant impact, as it can become unclear what combination is really meant in the request or specification in cases of errors. Then the combination may be misinterpreted, not taken into account when implementing combinations in BS or UEs or even removed from specs or requests. Also automated processing of the tables will become difficult with too many bugs.

Generally, the notation of band combinations starts with the type of the configuration (mainly CA or DC), followed by one list (either LTE or NR) or two lists (first LTE, then NR) of bands with bandwidths.  The following types of band combinations are defined in 3GPP specifications:

Carrier Aggregation: Starts with “CA_” as the first three characters. Then either a list of LTE or NR carriers is following, where the carriers or bands are always separated by “-”. LTE and NR carriers cannot be combined, that would be a DC combination. Examples:

-	CA_1A-2A (LTE)

-	CA_n1A-n2A (NR)

-	Examples for wrong notations: CA_1A_2A (“_” instead of “-” between the carriers/bands), CA-1A-2A (no “_” but “-” after “CA”, CA_1A_n2A (this would need to be a DC combination)

-	NOTE: LTE examples are provided for information only

Dual Connectivity: Starts with “DC_” as the first three characters, then for EN-DC configurations followed by the list of LTE carriers, a “_” as separation between the LTE and NR carriers and then the list of NR carriers. For NE-DC configurations the list begins with NR carriers, a “_” as separation between the NR and LTE carriers, and then the list of LTE carriers. There are exceptions for SUL EN-DC combinations, which separate the LTE and NR carriers by “_SUL_” instead of “_”, and contiguous intra-band EN-DC combinations using “(n)” instead of “_” and the “n” of the first NR band in the list. DC configurations within LTE or NR just list the carriers after “DC_”, where the carriers are always separated by “-”. For EN-DC combinations for V2X the “DC_” at the beginning is replaced by “V2X_”, even if it is still an EN-DC combination. Examples:

-	DC_1A_n2A (EN-DC)

-	DC_1A-2A (LTE-DC)

-	DC_n1A-n2A (NR-DC)

-	DC_(n)1AA (EN-DC with contiguous intra-band LTE and NR carriers)

-	DC_1(n)AA (NE-DC with contiguous intra-band NR and LTE carriers)

-	DC_1A-(n)2AA (EN-DC with one LTE carrier followed by contiguous intra-band LTE and NR carriers)

-	DC_2(n)AA-1A (NE-DC with one LTE carrier followed by contiguous intra-band NR and LTE carriers)

-	DC_n78A_1A-3A (NE-DC)

-	Examples for wrong notations: DC_1A-n2A (“-” instead of “_” between the LTE and NR carriers/bands for EN-DC combinations), DC-1A-2A (no “_” but “-” after “DC”, DC_n1A_n2A (“_” instead of “-” between the NR carriers/bands for NR-DC combinations)

Supplementary UL: An NR SA configuration including one SUL carrier without CA is marked starting with “SUL_” as the first four characters. If it is within an EN-DC combination there is a “_SUL_” between the LTE and the NR part instead of the “SUL_” at the beginning, using the usual “DC_” as the three characters. An NR SA configuration with CA including one SUL carrier is marked starting with “CA_” as the first three characters. For intra-band CA with one SUL carrier, the list begins with intra-band CA, followed by “-” as separation, then the SUL carrier. For inter-band CA with one SUL carrier, the list begins with NR carriers without SUL, followed by “_” as separation, then the NR carrier with SUL. For uplink EN-DC configuration, if TDM operation of uplink sharing from UE perspective (ULSUP) is chosen, the notation of “_ULSUP-TDM_” is used. The FDM operation of uplink sharing from UE perspective is not supported in current specifications.  Examples:

-	SUL_n2A-n80A (n80 being the SUL band)

-	CA_n41C-n80A (n80 being the SUL band): CA is between n41A and SUL_n41A-n80A

-	CA_n78(2A)-n86A (n86 being the SUL band): CA is between n78A and SUL_n78A-n86A

-	CA_n1A_n78A-n81A (n81 being the SUL band): CA is between n1A and SUL_n78A-n81A

-	DC_1A_SUL_n2A-n80A (n80 being the SUL band)

-	DC_3A_n80A_ULSUP-TDM_n78A (TDM operation on SUL_n78-n80)

In summary the following types and notations are defined:

-	CA_ …: A Carrier Aggregation configuration followed by the list of either LTE or NR carriers, or or NR carriers with one SUL carrier.

-	DC_ …: A Dual Connectivity configuration followed by the list of either LTE carriers for LTE-DC or NR carriers for NR-DC or for EN-DC first LTE carriers, then “_” and the NR carriers or for NE-DC first NR carriers, then “_” and the LTE carriers. In case of a DC combination for V2X, the “DC_” is replaced with “V2X_”.

-	SUL_ …: An NR SA configuration including one SUL  carrier without CA. In case of a DC configuration with SUL, the “SUL_” is shifted behind the “_” separating the LTE and NR carriers and the configuration starts with “DC_” as usual for DC configurations. In case of an uplink EN-DC configuration with SUL, “_ULSUP-TDM_” is applied for TDM operation for uplink sharing from UE perspective.

----- End of TP -----