3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514029

Prague, CZ, 13th – 17th October 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN conformance testing requirements for Iot-NTN TDD band

# Agenda item:	6.21.4

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of introduction of IoT-NTN TDD mode with the following objective. In this contribution, we want to share some views about the NB-IoT TDD operating NTN band from SAN conformance testing requirements perspective.

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

# 2 Discussion

We make the following proposals for SAN conformance testing requirements for IoT-NTN TDD band 249.

Table 2.1 General requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| - 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | No specification impact. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | Need to add band 249 in Table 4.4-1. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | The carrier bandwidth in section 4.7.3.1 needs to be updated. |
| 4.8 Applicability of requirements | Need to revise the applicability Transmit ON/OFF power in Table 4.8.2-1. Need to add “Transmit ON/OFF power (only applied for NB-IoT TDD SAN)” in Table 4.8.3-1. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Requirements for contiguous and non-contiguous spectrum | No specification impact. |
| 4.11 Requirements for SAN capable of multi-band operation | No specification impact. |
| 4.12 Reference coordinate system | No specification impact. |
| 4.13 Format and interpretation of tests | No specification impact. |

Table 2.2 Conducted Tx requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| - 6 Conducted transmitter characteristics |  |
| 6.1 General | No specification impact. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3 Output power dynamics |  |
| 6.3.1 General | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | No specification impact. |
| 6.4 Transmit ON/OFF power | Need to update transmit ON/OFF power requirement for NB-IoT TDD SAN. |
| 6.5 Transmitted signal quality |  |
| 6.5.1 General | No specification impact. |
| 6.5.2 Frequency error | No specification impact. |
| 6.5.3 Modulation quality | No specification impact. |
| 6.5.4 Time alignment error | No specification impact. |
| 6.5.5 DL RS power | No specification impact. |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | No specification impact. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |

Table 2.3 Conducted Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 7 Conducted receiver characteristics |  |
| 7.1 General | No specification impact. |
| 7.2 Reference sensitivity level | Need to add the following REFSENS requirement for IoT-NTN TDD band. Table 7.2.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band in TDD operating bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -126.1. |
| 7.3 Dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 7.3.5-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in Annex A.15; -105.1; -101.7; AWGN. |
| 7.4 In-band selectivity and blocking |  |
| 7.4.1 Adjacent channel selectivity (ACS) | Need to add the following ACS requirement for IoT-NTN TDD band. Table 7.4.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB (Note); LEO SAN class: -85.5; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | No specification impact. |

Table 2.4 Radiated Tx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN output power | No specification impact. |
| 9.4 OTA output power dynamics | No specification impact. |
| 9.5 OTA transmit ON/OFF power | Need to update OTA transmit ON/OFF power requirement for NB-IoT TDD SAN. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7 OTA unwanted emissions | No specification impact. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

Table 2.5 Radiated Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 10 Radiated receiver characteristics |  |
| 10.1 General | No specification impact. |
| 10.2 OTA sensitivity | Need to add the following OTA sensitivity requirement for IoT-NTN TDD band. Table10.2.5-3: SAN LEO class EIS levels for standalone NB-IoT operation and NB-IoT operation in NTN NR in-band SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel (NOTE); OTA sensitivity level, EIS (dBm). 200; 15; A14-3 in Annex A.14; Declared minimum EIS + 1.6. NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth.. |
| 10.3 OTA reference sensitivity level | Need to add the following OTA reference sensitivity level requirement for IoT-NTN TDD band. Table10.3.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band for TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -125.8 - ΔOTAREFSENS. |
| 10.4 OTA dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 10.4.5.2-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in annex A.15; -105.1 - ΔOTAREFSENS; -101.7 - ΔOTAREFSENS; AWGN. |
| 10.5 OTA in-band selectivity and blocking | Need to add the following ACS requirement for IoT-NTN TDD band. Table 10.5.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD frequency bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB - ΔOTAREFSENS(Note); LEO SAN class: -85.5- ΔOTAREFSENS; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 10.5.1 OTA adjacent channel selectivity | No specification impact. |
| 10.5.2 OTA in-band blocking | No specification impact. |
| 10.6 OTA out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA in-channel selectivity | No specification impact. |

Table 2.6 Annex

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| Annex A (normative): Reference measurement channel | Need to add new FRC A14-3 in Table A.14-1 for IoT-NTN TDD band. Need to add new FRC A15-3 in Table A.15-1 for IoT-NTN TDD band. |
| Annex B (informative): Environmental requirements for SAN equipment | No specification impact. |
| Annex C (informative): Test tolerances and derivation of test requirements | No specification impact. |
| Annex D (informative): Measurement system set-up | No specification impact. |
| Annex E (normative): Characteristics of the interfering signals | No specification impact. |
| Annex F (normative): Propagation conditions | No specification impact. |
| Annex G (normative): In-channel Tx test | Need to revise averaging length for the transmission time to 8ms in section G.7.3 Averaged EVM. |

Proposal 1: We make the proposals as Table 2.1, 2.2, 2.3, 2.4, 2.5 and 2.6 for SAN conformance testing requirements for IoT-NTN TDD band 249.

# 3 Conclusion

In this contribution, we shared some views on SAN conformance testing requirements for IoT-NTN TDD band and the proposals are made as follows:

Proposal 1: We make the proposals as Table 2.1, 2.2, 2.3, 2.4, 2.5 and 2.6 for SAN conformance testing requirements for IoT-NTN TDD band 249.

# 4 Reference

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC