3GPP TSG-RAN4 Meeting # 116bis	R4-2514032

Prague, CZ, 13th – 17th, Oct. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for HD-FDD for Ku band

# Agenda item:	7.6.4.1

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on introduction of HD-FDD for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about HD-FDD for NR NTN from UE RF requirements perspective.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

# 2 Discussion

  - 2.1 A new VSAT type

The objective of this WID states that introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type. Hence, we need to introduce a new mobile VSAT type 7 with electronic steering antenna. However, there are two issues need to be clarified. The first issue is that the WID doesn’t specify which satellite type the new VSAT type with small antenna is communicating with. The second is there already has mobile VSAT type 5 and type 6 with electronic steering antenna, and the difference between the new VSAT type 7 with VSAT type 5/6 is antenna size. It needs to add a note to clarify the new VSAT type 7 is mobile VSAT with small antennas (e.g., 20x20cm).

Proposal 1: To introduce a new mobile VSAT type 7 with electronic steering antenna.

Proposal 2: To clarify which satellite type the new VSAT type with small antennas is communicating with.

Proposal 3: Add a note to clarify the new VSAT type 7 is mobile VSAT with small antennas (e.g., 20x20cm).

  - 2.2 UE RF requirements for VSAT type 2, 3, 5, 6

In Phase 1, we need to introduce related UE RF requirements for HD-FDD for Ku band with FR1 numerology. For VSAT type 2, 3, 5, 6, OTA reference sensitivity level needs to be checked for HD-FDD. Usually, REFSENS for HD-FDD is bigger tightening value than FDD due to lack of Tx self-interference and less insertion loss in the front-end architecture for RedCap UE. However, for Ku band, insertion loss for FDD is usually very small, only about 0.2dB. Compared with large OTA reference sensitivity level with less than -122dBm, EIS tightening due to insertion loss difference between HD-FDD and FDD can be negligible. Therefore, OTA reference sensitivity level for FDD shall be reused for HD-FDD for VSAT type 2, 3, 5, 6.

Proposal 4: OTA reference sensitivity level for FDD shall be reused for HD-FDD for VSAT type 2, 3, 5, 6.

  - 2.3 UE RF requirements for new VSAT type 7

For the new VSAT type 7, the following UE RF impacts need to be checked for new VSAT type 7.

Table 2.3 UE RF impacts for new VSAT type 7

| NR-NTN UE Tx/Rx requirement | Proposals for new VSAT type 7 |
| --- | --- |
| 9.1 General |  |
| 9.2 Transmitted power |  |
| 9.2.1 NTN VSAT maximum output power | The following requirements need to be defined for the new VSAT type 7: - Minimum peak EIRP for Fixed VSAT - Maximum output power limits for Fixed VSAT - Minimum peak EIRP for Mobile VSAT - Maximum output power limits for Mobile VSAT |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | This requirement shall be checked whether it is applicable to small antenna size. |
| 9.2.3 Configured transmitted power | No specification impact. |
| 9.3 Output power dynamics | No specification impact. |
| 9.4 Transmitter signal quality | No specification impact. |
| 9.5 Output RF spectrum emissions | No specification impact. |
| 9.5.1 Occupied bandwidth | No specification impact. |
| 9.5.2 Out of Band Emissions | No specification impact. |
| 9.5.2.1 General |  |
| 9.5.2.2 Spectrum emission mask | No specification impact. |
| 9.5.2.3 Adjacent channel leakage ratio | No specification impact. |
| 9.5.3 Spurious Emissions | This requirement shall be checked whether it is applicable to small antenna size. |
| 9.6 Antenna pointing accuracy and performance | No specification impact. |
| 9.7 Additional regional requirements indicated by NS | No specification impact. |
| 10.1 General |  |
| 10.2 Polarization characteristics | No specification impact. |
| 10.3 OTA reference sensitivity level | This requirement shall be defined for the new VSAT type. |
| 10.4 Maximum input level | This requirement shall be defined for the new VSAT type. |
| 10.5 Adjacent channel selectivity | No specification impact. |
| 10.6 Blocking characteristics | No specification impact. |
| 10.7 Spurious emissions | Not applicable. |
| 10.8 Receiver antenna off-axis performance | No specification impact. |

Proposal 5: To use the proposals in Table 2.3 for UE RF impacts for new VSAT type 7.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: To introduce a new mobile VSAT type 7 with electronic steering antenna.

Proposal 2: To clarify which satellite type the new VSAT type with small antennas is communicating with.

Proposal 3: Add a note to clarify the new VSAT type 7 is mobile VSAT with small antennas (e.g., 20x20cm).

Proposal 4: OTA reference sensitivity level for FDD shall be reused for HD-FDD for VSAT type 2, 3, 5, 6.

Proposal 5: To use the proposals in Table 2.3 for UE RF impacts for new VSAT type 7.

# 4 Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)