3GPP TSG-RAN WG4 Meeting #116bis	R4-2514065

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on RRM core requirements on FR2-1 L3 measurement delay

Agenda Item:	4.15.1

Document for:	Discussion

# Introduction

At the last 3GPP RAN WG4 Meeting #116, the outcomes of the consensuses and the unresolved issues were documented in WF [1] and [2]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic: L3 measurement delay reduction by optimizing CSSF for CSSF outside gap in CA/DC

Issue 1-2-1: applicability limitation for solution 3

Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

We discussed this issue extensively in the last meeting. At least one company concerned that searcher source is challenging in the case of inter-RAT MOs without MG and inter-frequency MOs without MG. In the end, it was agreed to remove inter-RAT MOs without MG from the applicable configurations. Regarding the left case of inter-frequency MOs without MG, since it is an advanced function and the UE normally has powerful capability to support it, we expect such UE can apply the solution in the case of inter-frequency MOs without MG. Besides, from a broader applicability perspective, the solution is expected to be applied to the case of inter-frequency MOs without MG.

Proposal 1: Supportive to applying enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG.

Another underlying issue concerns the applicability of CSSF Solution 1 to inter-frequency layer measurements, which may have an intra-band relationship with the serving cells, as duplicated in the below from Big CR:

#### 9.2.3.2 Requirements for FR2 For UE supporting CSSF enhancement for one serving CC measurement per-band, if UE receives network indication via enable one serving carrier measurement in a band to enable one serving carrier measurement for an FR2 band, For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in the band; or - PSCC, when UE is configured with EN-DC with PSCC in the band; or - PSCC, when UE is configured with NR-DC with PSCC in the band; or - an SCC, if such SCC is the only one SCC on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band; or - an SCC, indicated by network via enable one serving carrier measurement in a band, if such SCC is one of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band; - otherwise - one SCC determined by UE implementation. UE is not required to perform SS-RSRP, SS-RSRQ, and SS-SINR measurements on both serving and neighbour cell for each of the other intra-frequency layer(s) in the same band.

We understand that, following the agreements, measurements of neighbor cells that are intra-frequency with respect to the serving cells may follow CSSF Solution 1. That is, neighbor cell measurements are actually covered by CSSF solution 1, which assumes that the UE receives reference signals with same quality from the neighbour cells and from ‘the serving cell on a same frequency layer and the serving cells on different frequency layer but in a same band’.

As demonstrated in the below figure, with respect to agreements, neighbour cell 1 (intra-frequency with SCC1) isn’t measured if the PCC in the same band is already measured.

Figure 1 CCs in a same band

This raises the question: for an inter-frequency cell that is intra-band relationship with the serving cells, but not intra-frequency with the serving cells, can the measurements of the inter-frequency cell also follow CSSF Solution 1?  For instance, neighbour cell 2 in Figure 1 is such an inter-frequency cell.

From a tech. perspective, we understand CSSF solution 1 is applicable to all carriers (serving or neighbor) in a same band.

Proposal 2: RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency neighbor cell (without measurement gap) that is intra-band but not intra-frequency with the serving cells.

## Topic: L3 measurement delay reduction by optimizing Rx beam sweeping factor

Issue 1-1-2: Re-evaluation mechanism

Agreement: FFS following options in the maintenance stage: - Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be: - Alt 1: Use the values for T1 as a starting point. - Alt 2: 5 s, which is predefined in RAN4 spec. - Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS. - Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS. Agreement in Friday online: - No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

In previous meetings, we supported Option 1, by the solution, the UE should re-enter FBS in a controllable manner. However, some companies opposed introducing a second timer for the non-FBS duration before re-evaluating the activation condition for FBS, as they argued there are some scenarios in which the UE does not need to repeat FBS.

That said, leaving the decision entirely to the UE regarding when to re-enter FBS after quitting FBS makes the feature become a one-time type of feature. In worst case, the network configures FBS relevant signaling, but the UE only follows it once. This is not a rational design from either a signaling load or network configuration perspective.

Observation 1: Having not a timer to re-evaluate FBM is not a rational design from either a signaling load or network configuration perspective to design a signaling that UE only mandatorily follows once.

Taking both perspectives into account, a compromise solution is proposed: After the UE exits FBS, it shall measure the serving cell quality.

- If the FBS activation criteria is met, it is up to the UE implementation to decide when to re-enter FBS.

- If the FBS activation criteria is not met, the UE shall re-evaluate the activation criteria and may re-enter FBS mode after timer T2 expires.

To our understanding, since the UE is always required to measure the serving cell quality, it should not be a significant issue for the UE to continuously compare the measurement results against the threshold.

Proposal 3: After the UE exits FBS, it shall measure the serving cell quality and evaluate the FBS activation condition:

- If the FBS activation criteria is met, it is up to the UE implementation to decide when to re-enter FBS.

- If the FBS activation criteria is not met, the UE shall re-evaluate the activation condition and may re-enter FBS mode after a timer T2 expires.

Issue 1-1-4: Requirements when UE reports different BSF values for different bands

Way forward: Further discuss this issue in the maintenance stage: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

To answer the question in this issue, first of all, we need to align the FBS applicable scenarios.

Recall the applicability of requirements in Big CR: R4-2512348, as duplicated in the below:

The requirements related to the support of Rel-19 L3 fast Rx beam sweeping are applicable when - For intra-frequency measurements, the carrier configured for L3 SSB measurement in the FR2-1 band is the serving carrier, and - For inter-frequency measurements, in addition to the serving carrier, at most one carrier in the FR2-1 band is configured for L3 SSB measurement, and - UE is not configured with CA or DC, and - L3 fast Rx beam sweeping operation is activated. L3 fast Rx beam sweeping operation is activated for intra-frequency measurements and inter-frequency measurements with measurement gap provided that - UE is configured with triggering conditions indicated by fbs-ThresholdP-r19 and/or fbs-ThresholdQ-r19, and the configured triggering conditions are satisfied. - The time since L3 fast Rx beam sweeping operation is activated is not larger than 90 s. L3 fast Rx beam sweeping operation is activated when UE is indicated to perform handover or RRC Connection Release with Redirection to NR cell procedure.

Literally, there are two applicable scenarios based on the aforementioned applicability:

- One serving cell MO and the neighbour cells in the MO can apply FBS.

- One serving cell MO and one target MO in a different band, only the neighbour cells in the target cell MO can apply FBS.

This issue actually extends the applicable scenarios to cases where multiple MOs in different bands are configured.

Proposal 4: Regarding the issue concerning different bands, RAN4 should check the applicable scenarios of FBS to include the cases with multiple MOs configured.

Issue 1-1-5: Revision of the agreement on applicability requirement

Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Agreement in Thursday online: When the target and serving carrier frequency are different, - FBS requirement does not apply to inter-frequency measurement without measurement gap. - FFS inter-frequency measurement with per-FR measurement gap. - Note: inter-RAT is not in the scope.

For the case that the target and serving carrier frequency are on different, the design target is to speed up the target carrier, it doesn’t harm if the serving carrier is faster, even the faster measurement on the serving carrier doesn’t facilitate handover essentially.

Proposal 5: We’re open to speeding up the serving carrier by FBS in the case that the target and serving carrier frequency applying FBS are on different.

# Conclusion

Observation 1: Having not a timer to re-evaluate FBM is not a rational design from either a signaling load or network configuration perspective to design a signaling that UE only mandatorily follows once.

Proposal 1: Supportive to applying enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG.

Proposal 2: RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency neighbor cell (without measurement gap) that is intra-band but not intra-frequency with the serving cells.

Proposal 3: After the UE exits FBS, it shall measure the serving cell quality and evaluate the FBS activation condition:

- If the FBS activation criteria is met, it is up to the UE implementation to decide when to re-enter FBS.

- If the FBS activation criteria is not met, the UE shall re-evaluate the activation condition and may re-enter FBS mode after a timer T2 expires.

Proposal 4: Regarding the issue concerning different bands, RAN4 should check the applicable scenarios of FBS to include the cases with multiple MOs configured.

Proposal 5: We’re open to speeding up the serving carrier by FBS in the case that the target and serving carrier frequency applying FBS are on different.

# References

- R4-2512124, WF for NR_RRM_Ph5_part1, Apple

- R4-2512125, WF for NR_RRM_Ph5_part2, CATT