3GPP TSG-RAN WG4 Meeting #116bis	R4-2514066

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	6.10.2.2

Document for:	Discussion

# Introduction

In this contribution, we herein present our views and proposals on the remaining open issues relevant to performance requirements.

# Discussion

### Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

Issue 2-2-2: sub-TC for CSSF optimization solution 1 with network indication

- Option 1(Apple): for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting. - Option 2 (Nokia): To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band. - [Way forward] - Continue discussion in RAN4#116bis.

Since the NW indication is newly introduced, we propose defining a test to verify the measurement delay with enhanced CSSF when the network specifies the carrier to be measured in the FR2 band.

The test shall ensure that the UE measures the corresponding CC as indicated by the network, rather than other CC(s) within the same band. The test procedure, where the UE is configured with at least two CCs, may include the following steps:

- One first CC is configured to be indicated by NW.

- The first CC is measured by UE.

- One second CC is configured to be indicated by NW.

- The second CC is measured by UE.

Proposal 1: RAN4 to define a test to verify the measurement delay with enhanced CSSF when the network indicates the CC to be measured in the FR2 band.

- In the test, the UE is able to measure the CC indicated by the network, among multiple CC(s) within the same band.

# Conclusion

Proposal 1: RAN4 to define a test to verify the measurement delay with enhanced CSSF when the network indicates the CC to be measured in the FR2 band.

- In the test, the UE is able to measure the CC indicated by the network, among multiple CC(s) within the same band.

# References

- R4-2512124, WF for NR_RRM_Ph5, Apple