# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2514093

# Prague, Czech, 13th – 17th October, 2025

Source:  ZTE Corporation, Sanechips

Title:   Discussion on the RRM requirements aspects of AI/ML positioning

Agenda Item:	6.11.5

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some views on the interoperability and testability for AI positioning from the general test framework perspective.

- Discussion

In previous meeting, RAN4 has reached the related agreements for AI positioning as below:

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

We observed that RAN4 has already defined reporting delay includes measurement delay, inference delay and the time needed until the UE send the report. However, the number of measurement samples have not been defined for the measurement. In this meeting, we shall specify concrete measurement samples.

In legacy positioning study, we have already agreed the Nsample as below:

is the number of PRS RSTD measurement samples, where - = 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions: - PRS bandwidth is within the active BWP and - Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB. - = 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions: - PRS bandwidth is within the active BWP and - Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB. - = 4 otherwise.

In positioning accuracy enhancement, UE performers measurement on DL PRS and then the measurement results will be the input of model inference. It is believed that the legacy measurement samples shall be reused since the measurement behaviour is the same. Nsample defined in legacy positioning equals to 1,2,4 based on UE capability and LMF requests. Hence, the measurement samples in positioning accuracy enhancement shall be 1,2,4 based on different UE capabilities and LMF requests.

Observation 1: Nsample defined in legacy positioning equals to 1,2,4 based on UE capability and LMF requests.

Observation 2: UE performers measurement on DL PRS and then the measurement results will be the input of model inference. The measurement behaviour is the same.

Proposal 1: The measurement samples in positioning accuracy enhancement shall be 1,2,4 based on different UE capabilities and LMF requests which same as legacy positioning.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML positioning:

Observation 1: Nsample defined in legacy positioning equals to 1,2,4 based on UE capability and LMF requests.

Observation 2: UE performers measurement on DL PRS and then the measurement results will be the input of model inference. The measurement behaviour is the same.

Proposal 1: The measurement samples in positioning accuracy enhancement shall be 1,2,4 based on different UE capabilities and LMF requests which same as legacy positioning.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2505105, “WF on requirements for AI/ML air interface normative work”