# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514098

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LP-WUS

Agenda item:	4.22.3

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

LP-WUR operation with SDT

In last meeting, one company believed the SDT requirements apply also for UE supporting LP-WUS and the way forward could be seen as below:

- Proposals - P1: SDT requirements apply also for UE supporting LP-WUS. - P1-1: The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading. (Nokia) Recommendations: Discuss the proposal P1-1

In legacy, UE with MR could perform small data transmission with some thresholds including RSRP threshold, data volume etc which defined in TS 38.321:

RRC configures the following parameters for SDT procedure: - sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT; - sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT; - mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT; - cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

For a UE that utilizes LP-RSRP measurements for condition evaluation, the network shall configure new evaluation threshold for LR. If new SDT evaluation thresholds are configured for LR, the following two cases regarding the RRM measurement offloading state should be clarified:

- If the LP-RSRP measurement results are above the threshold, the MR is awakened to perform SDT operation.

- If the LP-RSRP measurement results are below the threshold, the MR is awakened to initial a RACH procedure.

- In case 2, when small data are available for transmission and the LP-RSRP is below the configured threshold, the UE wakes up the MR and performs a RACH procedure to enter the connected state, after which data transmission occurs.

- In case 1, when small data are available for transmission and the LP-RSRP is above the configured threshold, the UE wakes up the MR, which then performs the SDT operation.

- Technically, we consider case 1 acceptable, while case 2 involves requirements related to the connected state, which RAN4 has already agreed no requirements in Rel-19. However, this threshold shall be considered in RAN2 not in RAN4. For RAN4 discussion, only TA validation and RSRP difference are considered for SDT requirements. If RAN2 defines the evaluation threshold, RAN4 shall consider reusing legacy requirements and wake up delay, otherwise, no SDT requirements for UE supporting LP-WUS.

Observation 1: For a UE that utilizes LP-RSRP measurements for condition evaluation, the network shall configure new evaluation threshold for LR. If new SDT evaluation thresholds are configured for LR, the following two cases regarding the RRM measurement offloading state should be clarified:

- If the LP-RSRP measurement results are above the threshold, the MR is awakened to perform SDT operation.

- If the LP-RSRP measurement results are below the threshold, the MR is awakened to initial a RACH procedure.

- Proposal 1: The SDT evaluation threshold shall be defined in RAN2. RAN4 shall wait for RAN2’s conclusion.

Low mobility criteria

- For this issue, we received one view as below:

- Proposals - P1: In core maintenance phase, RAN4 to discuss the case when low mobility criteria is configured for UE power saving but not met in the LP-WUR based RRM case#1 and case#3.

- In R16 UE power saving, RAN2 introduced both low mobility and not at cell edge which are MR-based power saving criteria. In LP-WUS discussion, actually, RAN2 has some agreements and assumptions as below:

- RAN2 #127bis meeting: - RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. - RAN2 #131 meeting: - RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode. - LR measurement is not used for R16/R17 RRM relaxation condition.

- We know that only LR measurement exists in case 1 (RRM measurement offloading), if considering low mobility criteria, we have two directions as below in case 1:

- LR based measurement towards low mobility criteria;

- MR based measurement towards low mobility criteria.

- The legacy R16 power saving standards are based on MR measurement not LR. If direction 1 is adopted, RAN2 shall define a new criteria for LR based low mobility measurement. However, RAN2 has already agreed that LR measurement is not used for R16/R17 RRM relaxation condition. Consequently, direction 1 does not exist. For direction 2, RAN2 assumes UE low mobility criteria is not included for R19 LP-WUS RRM relaxation (case 3)/ offloading (case 1) mode. Although this is not the final agreement, we believe it has large possibility that MR based low mobility criteria will not be introduced in LPWUS discussion. Hence, RAN4 shall not consider relaxed measurement based on low mobility in LPWUS study. If companies have some concern on this, we could wait for RAN2’s final conclusion.

- Proposal 2: For low mobility criteria, we have following two options:

- Option 1: RAN4 shall not consider relaxed requirements based on low mobility criteria for UE supporting LP-WUS.

- Option 2: RAN4 shall wait for RAN2’s final conclusion.

Maximum interruption in paging reception

For maximum interruption in paging reception, all companies deem that this requirements shall be reused. However, there is no discussion on it, so we just would like to clarify the requirements shall be reused in LP-WUS.

Proposal 3: The maximum interruption in paging reception requirements shall be reused in LP-WUS.

Conclusion

In this paper we provided our views on LPWUS.

Observation 1: For a UE that utilizes LP-RSRP measurements for condition evaluation, the network shall configure new evaluation threshold for LR. If new SDT evaluation thresholds are configured for LR, the following two cases regarding the RRM measurement offloading state should be clarified:

- If the LP-RSRP measurement results are above the threshold, the MR is awakened to perform SDT operation.

- If the LP-RSRP measurement results are below the threshold, the MR is awakened to initial a RACH procedure.

- Proposal 1: The SDT evaluation threshold shall be defined in RAN2. RAN4 shall wait for RAN2’s conclusion.

- Proposal 2: For low mobility criteria, we have following two options:

- Option 1: RAN4 shall not consider relaxed requirements based on low mobility criteria for UE supporting LP-WUS.

- Option 2: RAN4 shall wait for RAN2’s final conclusion.

Proposal 3: The maximum interruption in paging reception requirements shall be reused in LP-WUS.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869

[3] R4-2503116