# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514101

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for RedCap NTN

Agenda item:	6.19.3.1

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on NR NTN phase 3.

Discussion

In this section, we would like to provide our initial views on performance requirements on two different topics, which are redcap UEs in NTN scenario and downlink coverage enhancement. In this meeting, we mainly study general scope of these two topics.

  - 2.1 RedCap UEs in NTN scenario

In previous meetings, we have spent lots of time on discussing core requirements on redcap UEs in NTN scenario. To our understanding, the core requirements are quite stable so far and the performance requirements can be discussed in this meeting.

2.1.1 Measurement Accuracy Requirements

In last meeting, RAN4 only discussed the measurement accuracy requirements in performance part. The following agreement could be seen:

Agreement: - FFS: RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP ` - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location - For L1/L3 measurement accuracy, RAN4 to follow the agreements reached in RAN4 #114-bis meeting. - For measurement accuracy for NW verified location, RAN4 to follow the agreements reached in RAN4 #114 meeting. - RAN4 to define new sections for measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN. - Further to discuss the side conditions.

We observed that RAN4 has not confirmed what kind of accuracy requirements shall be defined yet. If defined, we shall continue to discuss the side conditions for accuracy requirements. Consequently, the first step is to determine what kind of accuracy requirements shall be defined.

To our understanding, the five sub bullets shall be all considered since those requirements have defined for both legacy NTN scenario and RedCap UE in TN scenario. Thus, we could conclude the following proposal:

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN:

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

In next step, we shall discuss how to define the concrete side conditions for the above sub bullets. Actually, different scenarios have different side conditions for accuracy requirements. From our perspective, we deem that the legacy side conditions for normal UE in NTN scenario and RedCap UE in TN scenario shall be both considered as baseline.

Proposal 2: The side conditions for RedCap UE in NTN scenario shall require joint consideration of normal UE in NTN scenario and RedCap UE in TN scenario.

2.1.2 Test principle and setups

For this part, the previous way forward could be seen as below:

- <Way Forward>: - Proposal 1: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference. - Proposal 2: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

Clause A.3.35 and clause A.3.36 are test principle and setups refer to RedCap and Satellite access respectively. However, only FR1 is discussed in NR NTN Phase 3. Consequently, we deem that , for test principle and setups, the testing related to satellite access and RedCap defined in A.3.36 and A.3.35 with FR1 can be a reference.

Proposal 3: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.36 and A.3.35 with FR1 can be a reference.

2.1.3 How and where to capture the test case for RedCap UE with FR1-NTN

For this part, the way forward is as below:

<Way Forward>: - Proposal 1: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN. - Proposal 1a: RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on. - Proposal 1b: Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 2: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements. - Proposal 2a: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs. - Proposal 3: - RAN4 to define new sections for test cases with suffix ‘A.X’. - Proposal 4: For RedCap UEs in NTN scenario, - To incorporate clauses for RedCap in NTN within Clause A.14, - Some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

For those above proposals, different companies have different view about where to capture the test cases for RedCap UE with FR1-NTN. To our understanding, we shall not totally either introduce a new suffix or add some additional clarifications in existing clause. If we totally introduce a new suffix, the whole spec will be complicated and redundant. If we totally add some additional clarifications in current spec, the whole spec will be more vague and not clear enough.

Above all, we could apply the same logic as was used for the less than 5MHz TN scenario. That is, since the legacy handover test cases for the TN scenario are already defined, a new subclause was introduced when the less than 5MHz TN case was considered. We propose introducing the new sub clause ‘A.14.X’ under clause A.14.

About how to capture test cases, we agree with use the legacy NTN test procedures or others as baseline. For the sake of spec simplification, we shall only capture the differences in new sub clause ‘A.14.X’ like test case defined for less than 5M in TN scenario (clause A.6.3.1.18). Thus, proposal 2 is more proper.

Propose 4: RAN4 shall introduce new sub clauses ‘A.14.X’ under clause A.14 instead of the total new suffix.

Proposal 5: The existing test cases defined for FR1 NTN in A.14 can be reused as starting point for RedCap UEs with FR1-NTN.

Proposal 6: For the sake of spec simplification, RAN4 shall only capture the main differences compared to legacy NTN test cases in new sub clause ‘A.14.X’. The duplicated parts could use ‘refer to’.

2.1.4 Test configuration and parameters for RedCap UE with FR1-NTN

For test configuration, we shall consider GSO and NGSO, FDD and HD-FDD factor for RedCap UE with FR1-NTN. Two points shall be noted as below:

- In legacy NTN test configuration, GSO and NGSO shall be both configured. If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- In legacy RedCap test configuration, both FDD and HD-FDD duplex are configured, but the UE is only required to be tested in one of the supported test configurations.

We shall consider above noted points in final test configuration. The final test configuration could be seen as below:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Additionally, it is evident that the test configuration for 1Rx and 2Rx is same even if test cases are separated. For this reason, it is our view that the test configuration shall be same for 1Rx and 2Rx RedCap UE with FR1-NTN.

Proposal 7: The test configuration shall be same for 1Rx and 2Rx RedCap UE with FR1-NTN.

Proposal 8: The final test configuration could be:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

2.1.5 Test Scope

In core part discussion, we have the agreement as below:

<Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location

From our perspective, first of all, the test cases for legacy normal UE in NR NTN scenario could be as the baseline. Secondly, we know that the some legacy core requirements are impacted by redcap UEs, thus, we shall only recognize the new requirements compared to legacy requirements and analyze the relevant test cases.

For cell re-selection requirements, we have normal cell-re-selection requirements and enhanced measurement requirements, we find that both two requirements are applied for 1Rx and 2Rx UEs. We find that only the UE capability and additional margin are different for 1Rx and 2Rx UEs, which are the same as legacy defined in RedCap WI. Because of this, the new test cases shall be defined in NTN scenario. We also observe that RAN4 will reuse the enhanced cell re-selection requirements for normal UE except for Tdetect being extended by [2] DRX cycles. Thus, we can only have the test case on this part for cell re-selection requirements.

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |

For handover requirements, we have RACH-based handover and RACH-less handover in NTN scenario. For RACH-based handover requirements, the legacy requirements for 2Rx UEs are reused, thus, the test cases for RACH-based handover could be reused. For 1Rx RACH-based handover requirements, RAN4 shall define the related test cases. For RACH-less handover, only 1Rx UEs shall be defined in core part discussion. Consequently, we would like to define the test cases for 1Rx UE.

| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
| --- | --- |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |

For conditional handover, we have time-/location-based CHO without/with L3 requirements. Based on the discussion on core part, we find that only the test cases for time-/location-based CHO with/without L3 measurement for 1Rx UE shall be defined.

| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
| --- | --- |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |

For satellite switching, RAN4 defined the new requirements for 1Rx RedCap UEs with FR1-NTN bands. We shall only consider the relevant test cases on it.

| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| --- | --- |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |

For RRC re-establishment requirements, RAN4 has defined the new requirements for 1Rx RedCap UEs with FR1 NTN bands. However, the requirements for 2Rx RedCap UEs are still under discussion since the existing NTN requirements may need be revised depends on the supported UE capabilities. Consequently, we deem that the new test cases for 1Rx UEs shall be considered. Whether to define the new test cases for 2Rx UE could be discussed.

| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
| --- | --- |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |

The same story for RRC Connection Release with redirection requirements, the potential test cases can be seen as below:

| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
| --- | --- |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |

For RA requirements, the following test cases shall be considered based on core part discussion:

| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
| --- | --- |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |

The same story as RRC re-establishment for RLM/BFD/CBD and L1/L3 measurement. The following test cases could be considered:

| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
| --- | --- |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

For timing requirements, we observe that the existing timing requirements shall be reused without differentiating 1Rx/2Rx. Thus, we deem that the legacy test cases for timing can be reused without defining any test cases for timing requirements.

In core part discussion, for signaling characteristics for 1Rx/2Rx redcap UEs, the agreement can be seen as below:

< Agreement >: - The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands: - Active BWP switch delay, - active TCI state switch delay - UE specific CBW change requirements - Pathloss reference signal switching delay - The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

For the above feature, no new test cases is needed since the legacy test cases could be reused.

Based on above, we propose the following test cases shall be considered in this meeting:

Proposal 9: RAN4 shall define the following test cases for redcap UEs in NTN scenario:

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |
| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |
| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |
| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |
| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

2.1.6 Whether to define separate test cases for 1Rx and 2Rx RedCap UE with NTN

When defining the specific test cases for RedCap UEs in NTN scenario, the existing NTN test cases can be used as a baseline. Since many test procedure and parameters defined for NTN can be reused for RedCap UEs in NTN, so we think RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs. Refer to legacy RedCap UE in TN scenario, 1Rx and 2Rx UEs have different test cases. We shall follow the same logic, RAN4 shall define separate test cases for 1Rx and 2Rx RedCap UE with NTN.

Proposal 10: RAN4 shall define separate test cases for 1Rx and 2Rx RedCap UE with FR1-NTN.

  - 2.2 Downlink coverage enhancement

In previous meetings, we have spent lots of time on discussing core requirements on downlink coverage enhancement. To our understanding, the core requirements are quite stable so far and the performance requirements can be discussed in this meeting.

For downlink coverage enhancement, according to previous agreements discussed in core part, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 11: For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

In core part discussion, only the following requirements are under discussion:

- RRC Re-establishment - RRC Connection release with redirection to NR - HO - CHO - Satellite switch with re-sync - Cell re-selection

For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, we observe that the existing requirements are not changed. Thus, we deem that the current test cases can be reused for these cases.

Proposal 12: For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

For RRC Re-establishment, the new core requirements are defined in the last meeting. Thus, we think the new test case shall be considered for RRC re-establishment requirements.

Proposal 13: For RRC Re-establishment, the new test case shall be considered.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN phase 3:

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN:

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

Proposal 2: The side conditions for RedCap UE in NTN scenario shall require joint consideration of normal UE in NTN scenario and RedCap UE in TN scenario.

Proposal 3: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.36 and A.3.35 with FR1 can be a reference.

Propose 4: RAN4 shall introduce new sub clauses ‘A.14.X’ under clause A.14 instead of the total new suffix.

Proposal 5: The existing test cases defined for FR1 NTN in A.14 can be reused as starting point for RedCap UEs with FR1-NTN.

Proposal 6: For the sake of spec simplification, RAN4 shall only capture the main differences compared to legacy NTN test cases in new sub clause ‘A.14.X’. The duplicated parts could use ‘refer to’.

Proposal 7: The test configuration shall be same for 1Rx and 2Rx RedCap UE with FR1-NTN.

Proposal 8: The final test configuration could be:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Proposal 9: RAN4 shall define the following test cases for redcap UEs in NTN scenario:

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |
| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |
| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |
| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |
| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

Proposal 10: RAN4 shall define separate test cases for 1Rx and 2Rx RedCap UE with FR1-NTN.

Proposal 11: For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 12: For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

Proposal 13: For RRC Re-establishment, the new test case shall be considered.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133 Clause A.14.