# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514141

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.2

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

In the following section, we will provide the detailed discussions for 6Rx UE demodulation requirements.

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have some open issues[2]. Next, we would like to address in the following for 6Rx UE demodulation requirements.

General

Test applicability rule for 6Rx test cases

- Proposals - Option 1: Introduce the test applicability rules by following core part agreement. - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices. - Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signalling finally confirmed by RAN2 - Option 2: Postpone the discussion until RAN2 concludes the UE capability signalling discussion

Regarding test applicability rule, this will affect further testing options for 6Rx UE, which has different devices to support different layers. According to the conclusion of RAN2, the new explicit signalling to support 6 layers for FWA UE has been agreed in the following.

Considering the explicit signalling is only agreed for FWA UEs. RAN4 maybe need to define the demodulation requirements only for FWA UEs with 6 MIMO layers. Therefore, from the perspective of demodulation, test applicable rules are needed.

Proposal 1.  Introduce the test applicability rules by following core part agreement.

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Specification structure

- Proposals - Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification - Section 5.2.3 4Rx requirements - Section 5.2.4 6Rx requirements - Section 5.2.5 8Rx requirements

In legacy specification, RAN4 created new sections for 2Rx, 4Rx, and 8Rx. We prefer to follow the same approach for 6Rx.

Proposal 2. Propose to modify the current section and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

PDSCH requirements for 6Rx

Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE

- Proposals - Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Firstly, the test cases of 4 MIMO layers for FWA and Handle UE should be discussed. Currently, the above options for MIMO correlation α = 0, β = 0, and α = 0, β = 0.07179 are very closely, and in the last meeting, some companies confirmed that the performance difference between different correlation may be only 1dB gap or less. From the requirements definition view, we think that the results are within the margin of error. Therefore, we don’t think it is necessary to define separate test cases for both FWA and Handle UE.

Proposal 3. Propose to define one single test case for both FWA and Handle UE.

Test applicability rule for FWA

- Proposals - Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests - Option 2: Not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests.

The purpose of introducing test applicability in the legacy procedure is to reduce the amount of testing effort. Currently, RAN4 plans to introduce the 4 MIMO layers and 6 MIMO layers for FWA device. From our understanding, if the UE can pass the 6 MIMO layers test, definitely the 4 MIMO layers test case shall be passed. Therefore, to introduce the test applicable rule for FWA is reasonable.

Proposal 4. Propose to consider the related test applicable rule that FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests.

DMRS configuration for 6 MIMO layers case

- Proposals - Option 1: Rel-15 Type 1 DMRS 2+2 - DMRS ports {1000-1003, 1004, 1006} - Option 2: Rel-18 eType1 DMRS 1+1

In the previous feasibility phase, RAN4 evaluated different DMRS configurations, which can bring different system throughput gain. And now, from the perspective of the testing, Rel-18 DMRS configuration is a optional feature, not all UEs could support this default configuration. In RAN4 legacy requirements, DMRS 2+2 is also used in Rel-18 8Rx requirements definition. Hence, we prefer to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 5. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

SDR requirements for 6Rx

SDR requirements for 1024QAM with 4 layers

- Proposals - Option 1: Not Define SDR requirements for 1024QAM with 4 layers - Option 2: Consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers

Currently, for SDR 1024QAM requirements, only defined 2 MIIMO layers test cases for 2Rx, 4Rx, and 8Rx. Especially, when RAN4 defined 8Rx demodulation requirements, it did not define the 8Rx SDR requirements with 1024QAM and 4layers. Considering the 6Rx requirement, we don’t observe the strong view to define this requirement. On the other hand, we also evaluated the performance of 1024QAM with 4 layers, as shown in the following. Currently, for the lowest MCS23, the ideal SNR is about 27.4dB, if we consider the margin and additional margin for 1024QAM, the SNR point will be close to 30dB. Also, for the other higher MCS, the SNR point will exceed 30dB, which will a challenge for testing environment. Therefore, we propose to do not define the SDR requirements for 1024QAM with 4 layers.

Table 1: Simulation results for SDR test with 1024QAM/4 layers

| MCS | Target SNR(dB)@85% of maxTP |
| --- | --- |
| 23 | 27.4 |
| 24 | 28.7 |
| 25 | 29.6 |
| 26 | 31.4 |

Figure 1. Simulation for SDR test with 1024QAM/4 layers

Observation 1. Currently, for the lowest MCS23, the ideal SNR is about 26.8dB, if we consider the margin and additional margin for 1024QAM, the SNR point will be close to 30dB. Also, for the other higher MCS, the SNR point will exceed 30dB.

Proposal 6. Propose to do not define the SDR requirements for 1024QAM with 4 layers.

CSI requirements for 6Rx

CQI test for 6Rx

- Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel

It is clearly mentioned in WID[1] that RAN4 needs to specify CSI requirements to support 6Rx, from our point of view, it is better to define CSI requirements in this item to follow the RAN plenary decision.

Proposal 7. Propose to define one CQI requirements with rank4 under static channel.

RI test

- Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

Regarding RI test, which is used to verify that the reported rank indicator accurately represents the channel rank. For FWA and handled UE 6Rx, we did not find it necessary to define requirements, since we also did not define requirements for 8Rx.

Proposal 8. Propose to do not define RI requirements for 6Rx.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx requirements, the conclusions are:

Observation 1. Currently, for the lowest MCS23, the ideal SNR is about 26.8dB, if we consider the margin and additional margin for 1024QAM, the SNR point will be close to 30dB. Also, for the other higher MCS, the SNR point will exceed 30dB.

Proposal 1.  Introduce the test applicability rules by following core part agreement.

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Proposal 2. Propose to modify the current section and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Proposal 3. Propose to define one single test case for both FWA and Handle UE.

Proposal 4. Propose to consider the related test applicable rule that FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests.

Proposal 5. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 6. Propose to do not define the SDR requirements for 1024QAM with 4 layers.

Proposal 7. Propose to define one CQI requirements with rank4 under static channel.

Proposal 8. Propose to do not define RI requirements for 6Rx.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2508727, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon.