3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514165

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for other SBFD impacts

Source: 	Huawei, HiSilicon

Agenda item:	6.13.3.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#116, and the outcomes are captured in WF [1]. Based on the work plan, RAN4 should continue the performance part of the WI in this meeting.

In this paper, we will provide our views on RRM performance requirements for other SBFD impacts.

Discussion

Accuracy requirements

Based on the core part discussion, for L1 and L3 measurements, existing accuracy requirements should apply except for one case, i.e. L1-RSRP and L1-SINR, for which RAN4 will define relaxed accuracy for DUD Case 3, where different cases for DUD are copied below.

- Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

In RAN4#115 following is agreed for L1 CSI-RS measurement.

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

In [2] the simulation assumption for evaluating L1-RSRP and L1-SINR accuracy performance with 24 RB is approved. In [3] we provide our simulation results, and we observe that the performance difference between 24 RB and 48 RB is around 0.5dB, and for some configuration of L1-SINR there is even no degradation. Therefore, we suggest to define the accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

In RAN4#116 the applicability of the existing and relaxed accuracy requirements for case 3 is discussed but without conclusion.

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply.

The main controversial part is whether UE should meet existing accuracy (48 RB) or relaxed accuracy (24 RB) when it indicates CSI processing time scaled by 2. Our view in RAN4#116 was that UE indicates CSI processing time scaled by 2 only when it intends to measure CSI-RS across 2 DL subbands. On the other hand, some companies commented that UE may measure CQI/PMI/RI across 2 DL subbands but measures L1-RSRP/L1-SINR still in 1 DL subband. In our view, this is a possible case, so we can compromise to option 2, i.e. UE is always required to meet relaxed accuracy (24 RB) in Case 3.

Proposal 1: Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB), and the relaxed accuracy requirements apply for Case 3 regardless of the measurement period.

Test cases

The impacted core requirements by the SBFD include

- CSI-RS based RLM

- CSI-RS based BFR

- CSI-RS based L1-RSRP measurement

- CSI-RS based L1-SINR measurement

- CSI-RS based L3 measurement

We suggest to define test cases for all except 4) and 5). For 4) the procedure itself and the impact of SBFD for L1-SINR are very similar to L1-RSRP, so it is sufficient to test only 3). For 5) our understanding is that UE behavior and requirements are same as legacy, but it is up to NW to ensure proper resource configuration so there is no need to test UE performance. In addition, RAN4 would define accuracy requirements for L1-SRS-RSRP, L1-CLI-RSSI and L1-RSRP, which also need to be tested.

Based on above, we suggest to define the following test cases for other impacts of SBFD as in Proposal 2. It is noted that the list starts from TC9 as TC1-8 are for L1 CLI measurement as in our companion paper.

Proposal 2: RAN4 to define the following list of test cases for other impacts of SBFD.

TC9-10: CSI-RS based RLM OOS FR1/FR2

TC11-12: CSI-RS based RLM IS FR1/FR2

TC13-14: CSI-RS based BFR FR1/FR2

TC15-16: CSI-RS based L1-RSRP measurement delay FR1/FR2

TC17-18: CSI-RS based L1-RSRP measurement accuracy FR1/FR2 (DUD Case 3)

Conclusions

In this paper we provided our views on RRM performance requirements for other SBFD impacts.

Proposal 1: Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB), and the relaxed accuracy requirements apply for Case 3 regardless of the measurement period.

Proposal 2: RAN4 to define the following list of test cases for other impacts of SBFD.

TC9-10: CSI-RS based RLM OOS FR1/FR2

TC11-12: CSI-RS based RLM IS FR1/FR2

TC13-14: CSI-RS based BFR FR1/FR2

TC15-16: CSI-RS based L1-RSRP measurement delay FR1/FR2

TC17-18: CSI-RS based L1-RSRP measurement accuracy FR1/FR2 (DUD Case 3)

Reference

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2514166, Simulation results for L1-RSRP and L1-SINR accuracy, Huawei, HiSilicon