3GPP TSG-RAN WG4 Meeting #116-bis 	R4-2514243

Prague, CZ, Oct 13th – 17th, 2025

Agenda Item: 6.3.2

Source: MediaTek inc.

Title: Discussion on performance requirements for low-band CA switching

Document for: Discussion

# Discussion

The main objectives of the work item is to introduce the physical procedures and requirements to enable low band CA via switching, detailed below [1]:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

RAN4 has already completed RRM aspects related to the low band CA operation and the agreements have been captured in the WF [2]. According to the work plan, RAN4 shall start discussing defining test cases.

## Discussion on test cases

The WI impacts the following RRM aspects:

| Impacted sections | Reason for update | Whether TC is needed |
| --- | --- | --- |
| 8.1.2.2 | Update the P scaling factor. | Yes |
| 8.1.3.2 | Update the P scaling factor. | No, redundant |
| 8.2.2.2.3 | Adding interruption ratio requirements | Yes |
| 8.3.2 | Adding the UE processing delay for activating the LBCA-SwitchingPattern | Yes |
| 8.3.3 | Adding the UE processing delay for deactivating the LBCA-SwitchingPattern | No, redundant |
| 8.3.4 | Adding the UE processing delay for activating the LBCA-SwitchingPattern | No, redundant |
| 8.5.2.2 | Update the P scaling factor. | No, redundant |
| 8.5.3.2 | Update the P scaling factor. | No, redundant |
| 8.5.5.2 | Update the P scaling factor. | No, redundant |
| 8.5.6.2 | Update the P scaling factor. | No, redundant |
| 8.10.3 | Update the note for first SSB for TCI state | No need |
| 8.10.5 | Update the note for first SSB for TCI state | No need |
| 8.14.3 | Adding Kp scaling factor for Path Loss requirements | Yes |
| 8.15.3 | Update the note for first SSB for TCI state | No need |
| 9.2.5 | Updating the Kp scaling factor for Intra-f | No, redundant |
| 9.5.4 | Updating the P scaling factor for L1-RSRP |  |
| 9.8.4 | Updating the P scaling factor for L1-SINR |  |

Based on the changes to the RRM requirements, a single test case can be defined for each of the following:

- A test case to cover the impact on P scaling factor.

- A test case to cover the measurement on deactivated SCell (SDL).

- A test case to cover the processing delay for activation/deactivation.

- A test case to cover the impact on the updated Kp scaling factor.

- RRM group to define test cases in LB-CA for the following RRM aspects: (i) a test case to cover the impact on P scaling factor, (ii) a test case to cover the measurement on deactivated SCell (SDL), (iii) a test case to cover the processing delay for activation/deactivation, and (iv) test case to cover the impact on the updated Kp scaling factor.

# Summary

In summary:

Proposal 1: RRM group to define test cases in LB-CA for the following RRM aspects: (i) a test case to cover the impact on P scaling factor, (ii) a test case to cover the measurement on deactivated SCell (SDL), (iii) a test case to cover the processing delay for activation/deactivation, and (iv) test case to cover the impact on the updated Kp scaling factor.

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2512119, “WF on RRM requirements for NR_LBCA_Sw”, Apple, Bengaluru, India, Aug 2025.