3GPP TSG RAN WG4 Meeting #116bis	R4-2514248

Prague, Czech Republic, 13th – 17th October 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	6.11.4

Document for: 	Discussion

# Introduction

In the last RAN4 meeting (#116) AI/ML BM testability aspects were treated, and the WF in [17] included the following agreements:

Issue 2-6: Test system setup

Agreement:

Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing)

Single AoA tests can also be performed in the multi AoA based RRM test setup

Further analyze what spatial channel models can be emulated with this test setup

  - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup

Single AoA tests are not precluded

Issue 2-7: Test system channel model

Agreement:

Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber.

Take CDL-C Umi as one example

Other channel models can be further discussed

We continue elaborating in this document our view already presented in previous contributions [8 to 14], with special attention to the definition of the multi-AoA baseline system, a respective candidate simplified channel to fit in the baseline system, and the required next steps to analyze whether the simplified channel model is suitable for AI/ML based beam management.

# Discussion

# feIFF Setup

As agreed in the last WF in [17], the multi-AoA and eIFF based system with 4 probes is considered as baseline test setup for further evaluation for AI/ML BM testing. This system reflects basically most of current commercial implementations of the RRM eIFF test systems (TS 38.508-1 [7], Annex B.2.6, with 2 simultaneous active probes with relative angular offsets of 30°, 60°, 90°, 120° and 150°), by deploying 4x CATRs in a single plane with the layout (or an equivalent) as shown in Figure 1 and Figure 2. How to define the concrete probe layout and the alignment with the coordinate system to be used for testing can be further discussed.

Figure 1 – Example of a commercial implementation of eIFF system with 4 IFF probes / CATRs

Figure 2 – Example of a commercial implementation of eIFF system supporting all (5) the required angular offsets with the minimal number (4) of static CATRs

For multi-AoA CDL emulation, the current eIFF system is further enhanced to enable the simultaneous activation of all the 4 available probes / CATRs. For the purpose of AI/ML BM testability discussion, this system with 4 static CATRs placed in a single plane and separated by 30º, 60º and 60º respectively, as well as supporting 4 active cross-polarized probes,  can be called “feIFF” (further enhanced Indirect Far Field).

Proposal 1: (feIFF Setup) The multi-AoA IFF based test setup with 4x active cross-polarized probes (CATRs) placed in a single plane and separated by 30º, 60º and 60º respectively, is defined as feIFF (further enhanced IFF) test setup. The concrete probe layout and the alignment with the coordinate system to be used for testing is FFS.

As already treated in the previous contributions, the technical capabilities and advantages of the feIFF test setup are summarized in Observation 1.

Observation 1: (feIFF Setup) The feIFF test setup supports 4 active cross-polarized probes with large distinctive angular separation of up to 150°, overall big quiet zone of at least 30 cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

In addition, the feIFF test setup has practical advantages as a superset of the eIFF setup, as summarized in Observation 2.

Observation 2: (feIFF Setup) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

# Simplified channel

The next step as per the WF in [17], is to define a simplified CDL based channel model supported by the feIFF test setup. Focus at this stage is matching OTA characteristics between the simplified channel and the setup.

For this purpose, as already presented in our previous contributions in [13] and [16], using the UMi CDL-C channel model Table 7.2.2-3 from TR 38.827 [3] as a reference, one option to implement further simplifications are as follows:

- The clusters that have the same AoAs can be merged, following Observation 11 in [15]. To do so, the delay for the first tap is kept and the power of the three taps is combined, resulting in +3dB for clusters #2 and #6.

- The weak clusters falling outside the UE’s spherical coverage (i.e. <-10dB) have a limited impact on the beam management  and can be removed, following Observation 8 to Observation 10 in [15], in order to reduce the number of clusters.

Implementing #1 results in a power scaling of all clusters. This is done by adjusting the power for the strongest cluster (i.e. #6) to 0dB and reducing the remaining clusters by the difference to cluster #6. With this scaling, there are more clusters falling below -10dB from the strongest cluster and then they are also removed. The resulting channel model parameters are presented in Table 1.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz 
after merging clusters with similar AoAs and removing weak clusters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The maximum angular spread required to emulate the down selected clusters in Table 1 is close to 150°, which can be covered by the probe layout of the proposed setup feIFF shown in previous section.

Observation 3: (Simplified Channel) CDL channel model can be simplified by merging the clusters that have the same AoA and removing the weak clusters falling outside the UE’s spherical coverage (i.e. < -10dB) resulting in the simplified channel model in Table 1.

Observation 4: (Simplified Channel) The required total AoA spread of 150° of the simplified channel in Table 1 can be emulated with the feIFF probe layout.

Looking at further channel model simplifications to fit to the feIFF system, there is one clear change to be implemented: the elevation spread (i.e. ZoA) and the intra-cluster elevation angle spread (i.e. CZSA) must be flattened, to fit the probe layout placed in a single plane. Further discussion could determine whether a flat 90º (i.e. as done for FR1 MIMO OTA as described in TR 38.827 [3]) should be used, or it could be possible to select the weighted average of the ZoA for the remaining clusters (i.e. 74.51º). In Table 2, the weighted average is used for further discussion. It can be decided later whether this single elevation is tested in the chamber by selecting the appropriate DUT testing conditions/orientations.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Related to how this channel model can be implemented in the candidate test system feIFF, we see 3 possible options:

- Option A: Using the simplified channel model in Table 2, the strongest cluster (i.e. cluster #6) is aligned to one of the available probes (i.e. 30º, Figure 3). In this approach, like done for MIMO OTA, the test system calculates the weights to simulate the AoAs in between the probes (i.e. reflectors). With this option, there is no need to further modify the channel model table. On the other hand, the main drawback is that there are several clusters falling outside the probe spread.

- Option B: Using the simplified channel model in Table 2, the mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes (i.e. 75º, Figure 3). With this option, there is no need to further modify the channel model table, and all clusters (nearly) fall into the probe spread.  The main drawback is that the strongest cluster fall between probes separated 60º apart.

- Option C: Using Option B as starting point, AoAs in the channel model parameter table are modified to align with the closest probe location. Here, every cluster will be aligned with one probe location and allows the intra-cluster angle spread (i.e. CASA) to be set to 0º for a very much simplified channel model. This results in the simplified channel model in Table 3 (probe alignment in Figure 3) .

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Observation 5: (Simplified Channel) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

In all 3 cases, and even though intra-cluster angle spread and/or intra-cluster elevation angle spread must be reduced, the doppler per cluster will be emulated by the test system.

Observation 6: (Simplified Channel) The simplified channel model parameters for UMi CDL-C do not affect the doppler per cluster, i.e. the latter is still emulated by the test system.

Proposal 2: (Simplified Channel) Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

The resulting alignment of the clusters with respect to the example probe layout for feIFF shown in Figure 1 is presented in Figure 3 . Please note that in the figure the channel model cluster AoAs have been transformed to the chamber probe coordinates, and that the strongest cluster (i.e. cluster #6) has been highlighted with a different marker (▲).

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF

# Channel evaluation

As presented in our previous contributions, we propose to base the assessment of the suitability of a simplified channel for AI/ML BM testing on AI/ML BM system simulations. This means comparing the UE prediction performance between the full (not simplified) CDL channel model (which is taken as the reference), with the prediction performance when using the simplified channel.

The training model and the prediction performance metric should remain the same for both cases. The evaluation of the channel will then be judged based on the performance difference, quantified by the agreed metric.

Proposal 3: (Channel Evaluation) Evaluate the suitability of the simplified channel for AI/ML BM testing through system level simulations. Compare the prediction performance between simulations with the full channel and the simplified channel, when using the same training model and performance metric.

Comprehensive simulations need to be conducted by UE/chipset vendors in order to include also UE related performance aspects like BB errors, RF impairments, eventually UE Rx beam sweeping and, above all, the implemented AI/ML model.

As a preliminary evaluation of the simplified channel proposed in section 2.2, we have conducted our own simulations (involving a “theoretical UE”), as described in detail in section 2.3.3 of our previous contribution in [15]. The results of these simulations are summarized below in Table 4 and Figure 4.

Table 4 – Prediction accuracy probability

|  | CNN 24 Clusters | CNN 6 Cl. (A/B) | CNN 6 Cl. (C) | DNN 24 Clusters | DNN 6 Cl. (A/B) | DNN 6 Cl. (C) |
| --- | --- | --- | --- | --- | --- | --- |
| Top 3 Set A Beams | 93,14% | 93,71% | 92,99% | 91,70% | 91,99% | 91,80% |
| Top 1 Set A Beam | 69,83% | 69,69% | 69,48% | 66,25% | 67,12% | 66,93% |

Figure 4 – Prediction accuracy probability

The results show that the prediction accuracy probability varies at a neglectable scale (approx. 1%) between cases with the full UMi CDL-C channel with 24 clusters and simplified UMi CDL-C channel with 6 clusters, in both variants described in previous section: Option A/B as per Table 2 and Option C as per Table 3.

Observation 6: (Channel Evaluation) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (deviation of max ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Observation 7: (Channel Evaluation) Based on preliminary simulation results, the simplified channel models for feIFF in Table 2 and Table 3 can be considered for further evaluation for AI/ML BM testing.

As mentioned before, for finalizing the evaluation of the channel for the AI/ML testing, further comprehensive system simulations are required by UE/chipset vendors, in order to include also real UE performance aspects.

Proposal 4: (Channel Evaluation) RAN4 to start the evaluation of CDL-C simplified channel models for feIFF with 6 Clusters in Table 2 and Table 3 through AI/ML BM system level simulations. Evaluate the channel suitability as per the approach described in Proposal 3.

# Conclusions

In this contribution, the following observations and proposals are made:

Proposal 1: (feIFF Setup) The multi-AoA IFF based test setup with 4x active cross-polarized probes (CATRs) placed in a single plane and separated by 30º, 60º and 60º respectively, is defined as feIFF (further enhanced IFF) test setup. The concrete probe layout and the alignment with the coordinate system to be used for testing is FFS.

Observation 1: (feIFF Setup) The feIFF test setup supports 4 active cross-polarized probes with large distinctive angular separation of up to 150°, overall big quiet zone of at least 30 cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

Observation 2: (feIFF Setup) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Observation 3: (Simplified Channel) CDL channel model can be simplified by merging the clusters that have the same AoA and removing the weak clusters falling outside the UE’s spherical coverage (i.e. < -10dB) resulting in the simplified channel model in Table 1.

Observation 4: (Simplified Channel) The required total AoA spread of 150° of the simplified channel in Table 1 can be emulated with the feIFF probe layout.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz 
after merging clusters with similar AoAs and removing weak clusters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

Observation 5: (Simplified Channel) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF

Observation 6: (Simplified Channel) The simplified channel model parameters for UMi CDL-C do not affect the doppler per cluster, i.e. the latter is still emulated by the test system.

Proposal 2: (Simplified Channel) Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

Proposal 3: (Channel Evaluation) Evaluate the suitability of the simplified channel for AI/ML BM testing through system level simulations. Compare the prediction performance between simulations with the full channel and the simplified channel, when using the same training model and performance metric.

Observation 7: (Channel Evaluation) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (deviation of max ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Observation 8: (Channel Evaluation) Based on preliminary simulation results, the simplified channel models for feIFF in Table 2 and Table 3 can be considered for further evaluation for AI/ML BM testing.

Proposal 4: (Channel Evaluation) RAN4 to start the evaluation of CDL-C simplified channel models for feIFF with 6 Clusters in Table 2 and Table 3 through AI/ML BM system level simulations. Evaluate the channel suitability as per the approach described in Proposal 3.

# References

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V19.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.4.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.101-4 V18.8.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.133 V19.1.0, “NR; Requirements for support of radio resource management”

- 3GPP TS 38.508-1 V19.0.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2418941, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2501726, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2504268, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114bis, April 2025

- R4-2506981, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #115, May 2025

- R4-2511597, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #116, August 2025

- R4-2500403, “Testability for AI/ML beam management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2511890, “WF on AI/ML for air interface [116][131]”, Qualcomm, 3GPP TSG RAN WG4 Meeting #116, August 2025

- R4-2508081, “Updated simulation assumptions for AI/ML BM”, Vivo, etc, 3GPP TSG RAN WG4 Meeting #115, May 2025

- R4-2508081, “WF on updated simulation assumption for AI based beam management”, Vivo, etc, 3GPP TSG RAN WG4 Meeting #116, August 2025