3GPP TSG-RAN WG4 Meeting #116bis	R4-2514292

Prague, Czech Republic, 13-17 October, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on UE Performance requirements for FR1 SU-MIMO with SCM

Agenda item:	 7.5.3

Document for:	Agreement

- Introduction

According to the WI in RP-252957, the objective for the performance part of the WI on UE Performance requirements for FR1 SU-MIMO with SCM is the following:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - Using rCDL-C1 channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

According to the objectives above, we submit with this document our company’s proposals for the work to be done in RAN4.

# Demodulation Requirements Scope

According to the objectives included in the WID, we support the introduction of PDSCH demodulation requirements

Proposal 1: RAN4 to define PDSCH demodulation simulation assumptions and collect simulation results for the goal of defining PDSCH requirements for the case of 8T8R and 4T4R, if the results are sufficiently aligned.

Proposal 2: For the purposes of performance requirements, RAN4 should consider only CDL approaches, in particular rCDL model as defined in TR 38.753.

Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

As observed in TR 38.753, the spatial channel models studied in RAN4 “have fixed angular properties and non-uniform PMI statistics, hence a fixed precoder may perform better than a channel adaptive precoder found by PMI selection algorithm.”. We believe that this can be an issue for the definition of PMI requirements and RAN4 should first identify and discuss countermeasures.

Proposal 4: RAN4 to discuss the impact of fixed angular properties of spatial channel modeling and identify countermeasures before evaluating the introduction of PMI reporting requirements.

# Channel modeling for rCDL

## Channel Models in TR 38.753 to be updated

The final version of the rCDL channel model in TR38.753 has included multiple revision to the procedures followd to generate the rCDL-C1 channel model used for the purpose of the Rel.19 Study Item. In particular, the cluster reduction procedure has been further detailed, and it has been agreed to update the angular scaling procedure in Section 5.1.3.1 to reflect the latest RAN1 TR 38.901 (v19).

Both of these procedures may result in minor deviations in the final rCDL tables compared to the existing rCDL-C1 model.

Observation 1: RAN4 has updated the cluster reduction procedure used to generate rCDL-C1 in TR 38.753 V2.0.0.

Observation 2: RAN4 has agreed to use the updated angular scaling procedure (referenced to TR38.901, version 19.0.0) in future releases of TR 38.753.

Creating new channel model tables and capturing them in TR38.753 will ensure that the procedures remain current and robust in light of ongoing standardization efforts.

Proposal 5: RAN4 to generate and include new rCDL channel tables in TR 38.753 using the updated cluster reduction and TR 38.901 v19 angular scaling procedure, for the purpose of using them for the definition of performance requirements in this work item.

- Conclusions

Proposal 1: RAN4 to define PDSCH demodulation simulation assumptions and collect simulation results for the goal of defining PDSCH requirements for the case of 8T8R and 4T4R, if the results are sufficiently aligned.

Proposal 2: For the purposes of performance requirements, RAN4 should consider only CDL approaches, in particular rCDL model as defined in TR 38.753.

Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

Proposal 4: RAN4 to discuss the impact of fixed angular properties of spatial channel modeling and identify countermeasures before evaluating the introduction of PMI reporting requirements.

Observation 1: RAN4 has updated the cluster reduction procedure used to generate rCDL-C1 in TR 38.753 V2.0.0.

Observation 2: RAN4 has agreed to use the updated angular scaling procedure (referenced to TR38.901, version 19.0.0) in future releases of TR 38.753.

Proposal 5: RAN4 to generate and include new rCDL channel tables in TR 38.753 using the updated cluster reduction and TR 38.901 v19 angular scaling procedure, for the purpose of using them for the definition of performance requirements in this work item.