3GPP TSG-RAN WG4 Meeting #116bis 	R4-2514306

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.18.2

Source: 	Ericsson

Title:	On RRM performance issues for XR

Document for:	Approval

# Introduction

The WI on WID on XR (eXtended Reality) for NR Phase 3 includes both core and performance parts [1]. While the core part has been finalized, the performance issues have not been yet discussed much, which is the scope of this contribution.

The only agreement for the performance part is on the accuracy requirements applicability [1]:

<Agreement> RAN4 confirms the agreement from RAN4 #113 in R4-2420103:

- Reuse existing measurement accuracy requirements for UE under XR operation.

# Discussion: RRM test cases for UE under XR operation

According to the RAN4 agreements and the agreed work split [3], the following requirements have been defined for XR in Rel-19:

| 9.2.6 | Intra-freq measurements with gaps |
| --- | --- |
| 9.3.4 | Inter-freq measurements with gaps |
| 9.4.2 | Inter-RAT E-UTRAN FDD measurements |
| 9.4.3 | Inter-RAT E-UTRAN TDD measurements |
| 9.15.5 | Inter-frequency L1-RSRP with MG |

It is also worth noting that the new requirements are defined for both FR1 and FR2-1, with and without DRX.

Furthermore, RAN4 agreed that the new requirements are specified for Type 1 and Type 2 MGs, per-UE gaps and per-FR gaps.

Defining test cases for all combinations of the above would result in a big set of test cases.

- Proposal 1: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

- Proposal 2: Test cases with and without DRX are specified.

- Proposal 3: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

- Proposal 4: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

- Proposal 5: Test cases are specified only for per-UE MGs.

- Proposal 6: For measurement procedure, define test cases without SSB time index detection.

Even if there will be the same accuracy requirements with XR as in legacy, there is still a need to verify whether accuracy can be met in the new XR scenarios since the measurement period is defined differently for XR and the time between samples can become very non-uniform.

- Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Proposal 8: The minimum set of measurement procedure TCs for XR is, e.g., as in Table 1 below:

Table 1: Minimum test case list for measurement delay requirements (NR-SA) under XR operation

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gap | A.6.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR1 | All cells in FR1 | DRX | Per-UE, Type 1 |
|  | A.7.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR2-1 | PCell in FR1, target cell in FR2-1 | DRX | Per-UE, Type 1 |
| Inter-RAT E-UTRAN FDD/TDD measurements | A.6.6.3.x SA NR - E-UTRAN event-triggered reporting in DRX under XR operation with FR1 serving cell | PCell in FR1, target cell in E-UTRAN FDD/TDD | DRX | Per-UE, Type 1 |

- Proposal 9: The minimum set of measurement accuracy TCs for XR is, e.g., as in Table 2 below:

Table 2: Minimum test case list for measurement accuracy requirements (NR-SA) under XR operation

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |

# Summary

- Proposal 1: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

- Proposal 2: Test cases with and without DRX are specified.

- Proposal 3: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

- Proposal 4: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

- Proposal 5: Test cases are specified only for per-UE MGs.

- Proposal 6: For measurement procedure, define test cases without SSB time index detection.

- Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Proposal 8: The minimum set of measurement procedure TCs for XR is, e.g., as in the table below:

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gap | A.6.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR1 | All cells in FR1 | DRX | Per-UE, Type 1 |
|  | A.7.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR2-1 | PCell in FR1, target cell in FR2-1 | DRX | Per-UE, Type 1 |
| Inter-RAT E-UTRAN FDD/TDD measurements | A.6.6.3.x SA NR - E-UTRAN event-triggered reporting in DRX under XR operation with FR1 serving cell | PCell in FR1, target cell in E-UTRAN FDD/TDD | DRX | Per-UE, Type 1 |

- Proposal 9: The minimum set of measurement accuracy TCs for XR is, e.g., as in the table below:

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2512144, WF on RRM requirements for NR_XR_Ph3, Nokia, Aug. 2025.

[3] R4-2504911, WF on RRM requirements for XR_Ph3, Nokia, Apr. 2025.