3GPP TSG-RAN4 Meeting #116bis	        R4-2514355 Prague, Czech Republic, 13th October – 17th October 2025

Source: 	Qualcomm Inc.

Title: 	Views on PC1.5 with 2 Tx for FDD bands for handheld and FWA UE

Agenda Item:	7.1.4

Document for:	Discussion

WI Code:	NR_UE_RF_Ph5-Core

# Introduction

RAN has agreed a work item for FR1 NR support to include PC1.5 with 2 Tx for FDD bands n1 and n25 for handheld and FWA UE [1]. Key parts of the work item objectives for RAN4 are to specify UE RF requirements and band specific requirements. In this contribution our views on UE RF and band specific requirements are made.

# Discussion

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

The WI objectives which are relevant to RAN4 UE RF work are copied below.

From the WI objectives it can be observed that while the RAN4 requirements are intended to cover PC1.5 with 2 Tx for a handheld and FWA UE in FDD bands, its requested only for bands n1 and n25.

Observation 1: Support for 2x2 UL MIMO and Tx Diversity is in WI scope of PC 1.5 with 2 Tx for FDD bands.

Proposal 1: Maximum output power (MOP) to support 2x2 UL MIMO and Tx Diversity shall be 29 dBm (i.e 2x26 dBm) for both handheld and FWA UE in FDD bands n1 and n25.

Maximum power reduction (MPR) for PC 1.5 with 2 Tx for a handheld UE is already defined in the standards as shown below in Table 1.

Table 1: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Proposal 2: Analyze the applicability of existing MPR table for PC 1.5 with 2 Tx for a FDD handheld UE.

Maximum power reduction (MPR) for PC 1.5 with 2 Tx for a FWA UE is already defined in the standards as shown below in Table 2.

Table 2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Proposal 3: Analyze the applicability of existing MPR table for PC1.5 with 2 Tx for a FDD FWA UE.

It can be seen from Table 3 below that there are multiple network signalling (NS) labels applicable to bands n1 and n25. Their corresponding A-MPR details are provided in Table 4.

Table 3: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |

Table 4: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 or txDiversity2Tx-r18 [TS 38.306], the additional relaxation of [2] dB is applicable. |  |  |  |  |  |

Proposal 4: Evaluate A-MPRs for all the applicable NS labels for bands n1 and n25 for PC1.5 with 2 Tx for FDD handheld and FWA UEs.

Observation 2: For a PC2 UE with Tx Diversity additional relaxation is provided in Note 7 of Table 6.2.3.1-1.

Proposal 5: Evaluate whether further relaxation in A-MPR for PC1.5 with 2 Tx FDD architecture is required for a UE indicating Tx Diversity.

Proposal 6: Update the modified MPR behavior table L.1-1 for bands n1 and n25 to support PC1.5 with 2 Tx for FDD handheld and FWA UEs.

UTRA ACLR requirement is applicable when the network signalling value NS_03U, NS_05U or NS_100 is signalled by the network in the field additionalSpectrumEmission and is as presented below in Table 5.

Table 5: UTRA ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB |

Observation 3: UTRA ACLR requirement is applicable for NS labels NS_03U, NS_05U and NS_100.

Proposal 7: Evaluate whether new UTRA ACLR is required for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signalled.

# Conclusions

In this contribution to support PC1.5 with 2 Tx for FDD bands n1 and n25 following observations and proposals are made:

Observation 1: Support for 2x2 UL MIMO and Tx Diversity is in WI scope of PC 1.5 with 2 Tx for FDD bands.

Proposal 1: Maximum output power (MOP) to support 2x2 UL MIMO and Tx Diversity shall be 29 dBm (i.e 2x26 dBm) for both handheld and FWA UE in FDD bands n1 and n25.

Proposal 2: Analyze the applicability of existing MPR table for PC 1.5 with 2 Tx for a FDD handheld UE.

Proposal 3: Analyze the applicability of existing MPR table for PC1.5 with 2 Tx for a FDD FWA UE.

Proposal 4: Evaluate A-MPRs for all the applicable NS labels for bands n1 and n25 for PC1.5 with 2 Tx for FDD handheld and FWA UEs.

Observation 2: For a PC2 UE with Tx Diversity additional relaxation is provided in Note 7 of Table 6.2.3.1-1.

Proposal 5: Evaluate whether further relaxation in A-MPR for PC1.5 with 2 Tx FDD architecture is required for a UE indicating Tx Diversity.

Proposal 6: Update the modified MPR behavior table L.1-1 for bands n1 and n25 to support PC1.5 with 2 Tx for FDD handheld and FWA UEs.

Observation 3: UTRA ACLR requirement is applicable for NS labels NS_03U, NS_05U and NS_100.

Proposal 7: Evaluate whether new UTRA ACLR is required for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signalled.

# References

[1]	RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, 3GPP TSG-RAN Meeting #109, Beijing, China, 15th – 18th September, 2025.