3GPP TSG-RAN WG4 Meeting # 116						R4-2514368

Prague,Czech Republic,  October 13 - 17, 2025

Source:		Ericsson

Title:			 LS reply on the A-IoT BS power

Agenda item:		7.11.2

Document for:		Approval

# Introduction

In this paper, we present our views on Rel-20 A-IoT coexisting simulation assumptions.

# Discussion

In LS [1], it is stated below and ask RAN4 feedback.

For the Rel-20 study, RAN1 discussed BS Tx power assumption for coverage evaluation in A-IoT and made the following agreements:

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Agreement For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

The in-band operation is only NR base station, and therefore, the NR BS output is relevant here. In Rel-19, the NR outdoor base station power is chosen to be 46 dBm. And in 38.922, the TRP power is quoted below and it can be observed that the same TRP for Rural/macro sub-urban/macro Urban. Though final TRP power used in coexisting simulation study is pending discussion, 46 dBm can be used as a number to reply to RAN1 if this would be agreed in coexisting simulation for NR macro BS output power. For in-band operation, considering the 10MHz bandwidth for NR BS and 1RB to 4 RB transmission bandwidth of R2D signal, the PSD should be 28.8 dBm/1 RB [46 -10log(52)], 31.8 dBm/2 RB, 33.6 dBm/3 RB and 34.8 dBm/4RB.

Table 4.2.1.5-2[2]: Total Radiated Power

| Parameter | Rural | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 46 | 37 |

In TR 38.922, the maximum BS output power is quoted below. Considering the TRP power of 46 dBm, the maximum antenna gain can be assumed to be 26.2 dB.  For the RAN1 LS 38 dBm over R2D transmission bandwidth, if transmission bandwidth is 1 RB and NR BS bandwidth is 10 MHz (52 RB), the TRP will be 55 dBm. Adding the 26.2 dB antenna gain will lead to 81.3 dBm E.I.R.P which exceeding the maximum BS output power.

The 38 dBm PSD over 1 RB transmission bandwidth exceeds the maximum E.I.R.P of BS output power.

Table 4.2.1.5-1: Maximum BS output power in 1710 to 4990 MHz

|  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| Maximum base station output power/sector (e.i.r.p.) (dBm) | 72.2 | 72.2 | 72.2 | 61.5 | N/A |

Depending on the R2D transmission bandwidth, the PSD is 28.8 dBm for 1 RB transmission bandwidth , 31.8 dBm for 2 RB, 33.6 dBm for 3 RB and 34.8 dBm for 4 RB, assuming the BS output power is 46 dBm with 10MHz bandwidth.

The standalone  BS power can be assumed to be 46 dBm.

The 38 dBm is not a reasonable PSD assumption for in-band operation.

# Conclusions

In this contribution, we present our overview LS from RAN1 with below observation and proposals:

Observation 1 The 38 dBm PSD over 1 RB transmission bandwidth exceeds the maximum E.I.R.P of BS output power.

Observation 2 The 38 dBm is not a reasonable PSD assumption for in-band operation.

Proposal-1: Depending on the R2D transmission bandwidth, the PSD is 28.8 dBm for 1 RB transmission bandwidth , 31.8 dBm for 2 RB, 33.6 dBm for 3 RB and 34.8 dBm for 4 RB, assuming the BS output power is 46 dBm with 10MHz bandwidth.

Proposal-2:The standalone BS power can be assumed to be 46 dBm.

# References

- R4-2513009,	LS on BS Tx power assumption for coverage evaluation in A-IoT

- TR 38.922