# 3GPP TSG RAN WG4 Meeting#116bis	R4-2514482

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements of LP-WUS/WUR for NR

Agenda item:	6.16.4

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for LP-WUS/WUR including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issues and provide simulation results for defining demodulation performance requirements.

2. Testability Aspects

2.1 Whether to define a test mode for LP-WUS/WUR

Whether to define a test mode for LP-WUS/WUR

- Agreement: Interested companies can provide an analysis of the expected test durations for the different modes next meeting. Further discuss if a test mode is required based on the provided analysis.

In the previous meeting, it was discussed whether a test mode is necessary to introduce for LP-WUS. The way forward is outlined below.

We observe that the LP-WUS periodicity typically aligns with the DRX periodicity. In idle mode, the DRX cycle is 1.28 seconds, which could result in test durations potentially couple of hours to achieve 1% MDR. While such test durations are expected in idle mode scenarios, we note that introducing a dedicated test mode for MDR testing would necessitate a specific implementation solely for RAN4 testing purposes. From a UE implementation standpoint, this is not desirable, as it introduces additional complexity and development overhead.

Observation 1: Introducing a test mode solely for RAN4 testing is not preferred, as it would require a dedicated implementation on the UE side.

Proposal 1: Do not introduce a test mode for LP-WUS/WUR testing.

2.2 Whether to define separate test cases for idle/inactive and connected modes

Whether to define separate test cases for idle/inactive and connected modes

- Proposals

  - Option 1: Define separate test cases procedure for the modulator in idle and connected modes.

The WF is outlined below.

The issue of whether to define separate test cases for IDLE/INACTIVE and CONNECTED modes should be revisited once discussions around simulation parameters have progressed further, particularly to determine if distinct test assumptions are needed for each mode.

Proposal 2: Defer this discussion whether separate test cases for idle/inactive and connected modes are needed.

3. Demodulation Requirements

3.1 Whether to introduce FAR test

Whether to introduce FAR test

- WF

  - Option 1: Yes.

  - Option 2: No

The WF is outlined below.

In our view, FAR primarily affects power-saving performance and does not reflect any demodulation aspects of LP-WUS/WUR. Therefore, we believe such a test should not be introduced as part of the demodulation performance requirements.

Observation 2: The WUR might incorrectly indicate the presence of a WUS when no transmission has occurred, resulting in a false alarm. A high FAR primarily impacts the power saving performance.

Proposal 3: Support option 2.

3.2 Antenna Configuration

Antenna configuration

- Agreement:

  - 1T1R for FR1

  - FFS: 1T1R for FR2

The WF is outlined below.

In our view, 1T1R antenna configuration could be a practical and feasible option for FR2.

Proposal 4: 1T1R for FR2.

3.3 Whether to define single requirements for Type 1 and Type 2 receivers

Whether to define single requirement for Type 1 and Type 2 receivers

- Proposals

  - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined.

  - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

The WF is outlined below.

In our view, this topic should be revisited after simulation results from both receivers have been analyzed. Depending on the chosen simulation assumptions, it may or may not be feasible to define a unified requirement for both receiver types. Therefore, we propose postponing this discussion until a meaningful comparison can be made.

Proposal 5: Support option 1.

3.4 Whether to define capability rules for Type 1 and Type 2 receivers

Whether to define single requirement for Type 1 and Type 2 receivers

- Proposals

  - Option 1: Don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

The WF is outlined below.

The term "capability rules" in this context is unclear. If the intent was to refer to "applicability rules," our position is that such rules should only be considered after a thorough analysis of the results. This evaluation will help determine whether defining an applicability rule is warranted.

Proposal 6: Defer the discussion of applicability rules until simulation results have been analyzed for both receivers.

3.5 Inclusion of the requirements in the specification

- Proposals

  - Option 1:

    - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

    - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

The WF is outlined below.

Given that LP-WUS requirements represent a new category within the specification and considering the nature of these requirements, it may be appropriate to introduce dedicated subclauses specifically for LP-WUS requirement.

Proposal 7: Support option 1.

4. Simulation results

In the previous meeting, we agreed on several simulation assumptions. In this section, we provide our initial simulation results based on the following assumptions as presented in Table 1.

Table 1: Simulation assumptions for OFDM receiver

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| M | 4 | 1 |
| Wake-up information length (K) | 5 bits | 2 bits |
| Number of coded bits after rate matching | 32 for 5 info bits | [12, 16] for 2 info bits |
| LP-WUS waveform | OOK-4 | OOK-1 |
| Channel model | TDLC300-100 | TDLA30-10 |
| Frequency range and SCS/BW | 30kHz(40MHz) | 120kHz(100MHz) |
| Antenna configuration | 1x1 | 1x1 |
| Time offsets | 1us | 5us |
| Freq offsets | 5ppm | 5ppm |
| FAR | 1% | 1% |
| MDR target | 1% | 1% |

Based on the above simulation assumptions, we provide SNR requirements to achieve 1% MDR for both FR1 and FR2 in Table 2.

Table 2: Simulation results for MDR requirements for OFDM receiver

| Tests | SNR@1% MDR |
| --- | --- |
| FR1 (M=4, number of coded bits after rate matching=32) | 1.5 |
| FR2 (M=1, number of coded bits after rate matching=16) | -3.3 |
| FR2 (M=1, number of coded bits after rate matching=12) | -2.5 |

The results presented in Table 2 indicate that the FR1 test case requires a comparatively higher SNR to achieve the same MDR target of 1% as FR2. In addition to the more challenging channel conditions, FR1 also utilizes a larger M value, which shortens the ZC sequence length. This reduction in complexity comes at the cost of inferior synchronization performance. Consequently, the selection of a lower timing offset for larger M values is well justified here.

Furthermore, it is observed for FR2 test cases that an increase in the number of coded bits (post rate matching) correlates with a reduction in the required SNR to achieve the same MDR target of 1%. This improvement can be attributed to the higher number of repetitions associated with a larger coded bit count (post rate matching).

5. Conclusions

In this paper, we share our perspectives on the remaining open issues and also provide initial set of simulation results for LP-WUS/WUR. The following proposals have been made.

Observation 1: Introducing a test mode solely for RAN4 testing is not preferred, as it would require a dedicated implementation on the UE side.

Proposal 1: Do not introduce a test mode for LP-WUS/WUR testing.

Proposal 2: Defer this discussion whether separate test cases for idle/inactive and connected modes are needed.

Observation 2: The WUR might incorrectly indicate the presence of a WUS when no transmission has occurred, resulting in a false alarm. A high FAR primarily impacts the power saving performance.

Proposal 3: Do not introduce an FAR test for LP-WUS/WUR.

Proposal 4: 1T1R for FR2.

Proposal 5: Option 1.

Proposal 6: Defer the discussion of applicability rules until simulation results have been analyzed for both receivers.

Proposal 7: Support option 1.

6. References

[1] R4-2512526, “WF for [116][330] NR_LPWUS_Demod,” 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.