3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514486

Prague, CZ, October 13 – 17, 2025

Source:	NTT DOCOMO, INC.

Title:	Initial views on Rel-20 HPUE requirements

Agenda item:	7.1.4

Document for:	Approval

- Introduction

At the last RAN#109 meeting, Rel-20 UE RF enhancements WI was approved [1]. One of the objectives in the WI is to study the new possibility and specify the UE RF requirements of HPUE for NR single carrier. This paper provides our initial view on this topic.

- Background

RAN#109 approved Rel-20 UE RF enhancements WI [1]. One of the objectives in the WI is to specify the new HPUE requirements for single carrier with NR FR1 bands. The objectives for HPUE requirements are shown below:

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

- Discussion on higher power class with 4Tx for CPE/FWA

In Rel-18, 4Tx on single carrier for CPE/FWA was discussed [2], and the requirement for PC1.5 was introduced, but not for the higher power classes than it. RAN4 already has PC1 which is a power class with higher maximum output power than PC1.5. Therefore, as stated in the WID, RAN4 needs to discuss whether existing PC1 can be reused, or new power class should be introduced. PC1 was originally specified for public safety. This differs from the implementation assumptions (e.g., PA configurations) in the Rel-20 discussions, and RAN4 should consider its impact. In addition, for PC1, the maximum output power is specified as 31dBm. 32dBm transmission is within the tolerance of PC1 requirement, but it is recommended that 32dBm is redefined as a new maximum output power based on the regularity of existing power classes (i.e., PC3, PC2, and PC1.5) increasing in 3dB increments. Renewing the existing PC1 requirements in this release is not reasonable as well, so it is preferable to introduce new power class.

Observation 1: RAN4 should consider the differences in UE implementation assumptions and the regularity of existing power classes increasing in 3dB.

Proposal 1: RAN4 can introduce new power class with 4Tx for TDD bands for CPE/FWA.

- Conclusion

In this contribution, we provide our initial views as follows.

Observation 1: RAN4 should consider the differences in UE implementation assumptions and the regularity of existing power classes increasing in 3dB.

Proposal 1: RAN4 can introduce new power class with 4Tx for TDD bands for CPE/FWA.

References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, RAN#109

- RP-233186, “Revised WID: Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei, HiSilicon, NTT DOCOMO, RAN#102