3GPP TSG-WG4 Meeting #116bis	R4-2514497

# Prague, Czech Republic, 13 - 17 Oct. 2025

# Title:	On removing redundant dual-UL IMD MSD requirements

# Agenda Item:	4.6.1.1	High power UE (HPUE) for CA in terrestrial network (TN)	[NR_ENDC_RF_Ph4-Core]

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

RAN4 accomplished a major milestone at meeting#116 by endorsing the simplification of HPUE inter-band CA MSD requirements using Look-Up Tables (LUTs) [1]. To ensure consistency across band combinations and compliance with the RAN4 PRD rules, it was pointed out in [2] that RAN4 should ensure a maximum of two dual-uplink (UL) MSD requirements per downlink (DL) affected band and is specified in the legacy MSD tables so that the calculated HPUE MSD via LUTs also “creates” a maximum of two MSD requirements. This rule has been followed since its introduction. However, there are several legacy MSD requirements which do not follow this rule. This contribution explains the changes that are intended to be filed at meeting #117 for Rel 19.3.0.

- Discussion

Figure 1 below reminds the RAN4 PRD rules on MSD requirements due to dual-UL IMD interference in case a DL band is affected by multiple IMD products. In summary:

- A maximum of 2 MSD requirements is allowed in case the DL band is affected by both even and odd order IMD products, and these requirements are specified for the lowest odd and the lowest even order IMD product;

- 1 MSD requirement is allowed in case the DL band is affected by several odd order or several even order IMD products, and this requirement is specified for the lowest IMD order (even/odd).

Figure 1: RAN4 PRD rules on handling multiple dual-UL IMD order MSD requirements

The PRD rules have been followed since its introduction. However for legacy NR CA or EN-DC band combinations, we can find MSD requirements which remain specified for multiple odd order or for multiple even order dual-UL IMD products. One example is shown in Figure 2 where the band n1 PC3 MSD requirements are specified for both IMD2 and IMD4.

In this example, it is necessary to remove the redundant Band n1 IMD4 MSD requirement.

Figure 2: Example of multiple PC3 2UL IMD MSD test points affecting DL band n1 in CA_n1-n77

The number of requirements impacted by this change is relatively small. We provide in both Annex1 and Annex 2 a list based on parsing the Rel 19.2.0 TS 38.101-1 and TS 38.101-3.

Proposal: Bring clean-up CR to remove the redundant 2UL IMD MSD requirements from MSD tables in TS 38.101-1 and TS 38.101-3. Exceptions to these clean-ups are MSD requirements which cannot be verified due to regional frequency range restrictions – see the Annex for a detailed list of changes.

- Conclusion

This contribution proposes to parse the dual-UL IMD requirements specified in legacy MSD tables to ensure that the RAN4 PRD rules are correctly implemented so that future HPUE MSD requirements calculated with LUT do not introduce inconsistent MSD requirements.

Proposal: Bring clean-up CR to remove the redundant 2UL IMD MSD requirements from MSD tables in TS 38.101-1 and TS 38.101-3. Exceptions to these clean-ups are MSD requirements which cannot be verified due to regional frequency range restrictions – see the Annex for a detailed list of changes.

# References

- R4-2511752, [NR_ENDC_RF_Ph4-Core] Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Skyworks Solutions, Inc., Nokia, T-Mobile USA, ZTE, Oppo, Samsung, Murata Manufacturing Co Ltd., MediaTek, Vivo, Huawei, Apple, Qualcomm France.

- R4-2511623 Remaining items on HPUE MSD Simplification, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Skyworks Solutions, Inc.

# Annex 1– TS 38.101-1 Rel 19.2.0 Clean-up overview: removal of redundant 2UL IMD4, IMD5 MSD test points

# Table 7.3A.5-1: Removal of following MSD for DL band / IMD order.

- CA_n1-n77: n1 IMD4

- CA_n2-n66: n66 IMD5

- CA_n2-n77: n2 IMD4

- CA_n3-n77: n3 IMD4

- CA_n3-n78: n3 IMD4

- CA_n20-n41: n20 IMD5

- CA_n25-n66: n66 IMD5

- CA_n25-n77: n25 IMD4

- CA_n28-n50: n28 IMD4

- CA_n28-n74: n28 IMD4

Table 7.3A.5-1a: Removal of following MSD for DL band / IMD order.

- CA_n1-n77: n1 IMD4

- CA_n2-n77: n2 IMD4

- CA_n3-n77: n3 IMD4

- CA_n3-n78: n3 IMD4

- CA_n25-n66: n66 IMD5

- CA_n25-n77: n25 IMD4

Table 7.3A.5-1b: Removal of following MSD for DL band / IMD order.

- CA_n2-n77: n2 IMD4

- CA_n3-n78: n3 IMD4

- CA_n25-n77: n25 IMD4

Table 7.3A.5-2: Removal of following MSD for DL band / IMD order.

- CA_n1-n3-n78: 		n78 IMD4

- CA_n2-n66-n77: 		n66 IMD4

- CA_n2-n66-n77: 		n77 IMD4

- CA_n3-n5-n78: 		n78 IMD5

- CA_n3-n8-n78: 		n78 IMD5

- CA_n3-n26-n78: 		n78 IMD5

- CA_n5-n7-n78: 		n78 IMD4

- CA_n5-n66-n77: 		n78 IMD5

- CA_n7-n28-n78: 		n78 IMD4

- CA_n20-n41-n78:	n78 IMD4

- CA_n25-n41-n77: 	n77 IMD5

- CA_n25-n41-n78: 	n78 IMD5

- CA_n25-n66-n77: 	n66 IMD4

- CA_n25-n66-n77: 	n77 IMD4

- CA_n26-n66-n77: 	n77 IMD5

- CA_n41-n71-n77: 	n77 IMD4

- CA_n41-n71-n78: 	n77 IMD4

- CA_n66-n70-n78: 	n70 IMD4

- CA_n70-n71-n77: 	n77 IMD5

The following redundant test points cannot be removed due to regional frequency range restrictions.

- CA_n2-n66-n77: 	n2 IMD4

- CA_n25-n66-n77: 	n25 IMD4

- CA_n26-n70-n77: 	n77 IMD5

Table 7.3A.5-2a: Removal of following MSD for DL band / IMD order.

- CA_n25-n41-n77: 	n77 IMD5

- CA_n25-n66-n77: 	n66 IMD4

- CA_n41-n71-n77: 	n77 IMD4

The following redundant test point cannot be removed due to regional frequency range restrictions.

- CA_n25-n66-n77: 	n25 IMD4

Table 7.3A.5-2b: Removal of following MSD for DL band / IMD order.

- CA_n25-n41-n77: 	n77 IMD5

- CA_n41-n71-n77: 	n77 IMD4

# Annex 2 – TS 38.101-3 Rel 19.2.0 Clean-up overview: removal of redundant 2UL IMD4, IMD5 MSD test points

# Table 7.3B.2.3.5.1 (PC3 – two bands): Removal of following MSD requirements / IMD order.

- DC_1A_n77A: B1 IMD4

- DC_2A_n77A: B2 IMD4

- DC_2A_n78A: B2 IMD4

- DC_3A_n77A: B3 IMD4

- DC_3A_n78A: B3 IMD4

- DC_20A-n41A: B20 IMD5

- DC_25A_n77A: B25 IMD4

- DC_25A_n78A: B25 IMD4

- DC_28A_n50A: B28 IMD4

- DC_42A_n3A: n3 IMD4

- DC_66A_n25A: B66 IMD5

Table 7.3B.2.3.5.1-1a (PC2 two-bands): Removal of following MSD requirements / IMD order.

- DC_1A_n77A: B1 IMD4

- DC_3A_n78A: B3 IMD4

- DC_2A_n77A: B2 IMD4

- DC_2A_n78A: B2 IMD4

- DC_3A_n77A: B3 IMD4

- DC_20A-n41A: B20 IMD5

- DC_25A_n78A: B25 IMD4

Table 7.3B.2.3.5.2-1 (PC3 three-bands): Removal of following MSD for DL band / IMD order.

- DC_1A-3A_n77A:	B3 IMD4

- DC_1A_n3A-n77A:	n77 IMD4, B3 IMD4

- DC_1A-3A_n77A:	B3 IMD4 – Duplicated test point.

- DC_2A_n2A-n77A:	n2 IMD4

- DC_2A-4A_n78A:	B2 IMD4

- DC_2A_n25A-n66A:	n25 IMD5

- DC_2A-66A_n25A:	B66 IMD5

- DC_2A-66A_n77A:	B66 IMD4 + Duplicated test point.

- DC_2A-66A_n78A:	B2 IMD4

- DC_2A_n66A-n77A:	n66 IMD4

- DC_2A_n66A-n78A:	n78 IMD4

- DC_3A_n26A-n78A:	n78 IMD5

- DC_5A_n3A-n78A:	n78 IMD5

- DC_7A_n26A-n78A:	n78 IMD4

- DC_7A_n66A-n77A:	n77 IMD5

- DC_8A_n3A-n78A:	n78 IMD5

- DC_12A-66A_n25A:	B66 IMD5

- DC_25A-66A_n77A:	B66 IMD4

- DC_25A-66A_n78A:	B25 IMD4

- DC_66A_n2A-n78A:	n2 IMD4, n78 IMD4

- DC_66A_n25A-n66A:	n66 IMD5

The following redundant test points cannot be removed due to regional frequency range restrictions.

- DC_25A-66A_n77A:	B25 IMD4

Table 7.3B.2.3.5.2-1a (PC2 three-bands): Removal of following MSD for DL band / IMD order.

- DC_1A-3A_n77A:	B3 IMD4

- DC_2A_n2A-n77A:	n2 IMD4

- DC_2A-66A_n77A:	B66 IMD4 + B2 IMD4

- DC_2A-66A_n78A:	B2 IMD4

- DC_66A_n2A-n77A:	n2 IMD4