3GPP TSG-RAN WG4 Meeting #117	R4-2520022

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.19.4

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for Rel-19 NR NTN Phase 3

Document for:	Discussion

# Introduction

Based on the WF [1], we provide simulation results and our perspectives on the unresolved issues for UE demodulation requirements for NTN RedCap and eRedCap.

- Discussion

There were discussions about whether to apply Rel-19 NGSO channel modelling for NTN RedCap/eRedCap UE demodulation requirements.

Issue 3-2-1: Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirement Way Forward - Option 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model - Option 2: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario - Companies are encouraged to provide simulation results in the next meeting with fixed frequency offset 200Hz and timing offset=[29Tc] to derive the additional margin

NTN RedCap/eRedCap UE requirements and NGSO time varying channel model are parallel WIs. It is against the RAN4 working principle for RAN4 to address two interdependent WIs in parallel. Therefore, we prefer to define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model.

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model

We provide our simulation results based on the agreement at RAN4#116bis where DMRS 1+1 are used for the case of MCS4.

Issue 3-1-1: FRC for HD-FDD RedCap/eRedCap demodulation Agreement: - Use DMRS 1+1 for NTN RedCap and eRedCap UE requirements and introduce new FRCs for cases of MCS4/52PRB

Table 1: Simulation results for RedCap with 1RX

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | 1.8 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | 0.5 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | 8.5 |

Table 2: Simulation Results for eRedCap - without reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 1.8 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 8.5 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | - |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 5.0 |

Table 3: Simulation Results for eRedCap - with reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 2.2 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 8.0 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -1.6 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 4.5 |

# Conclusion

In this contribution, we provide simulation results for NTN RedCap/eRedCap with NTN channel model. Also, our proposal is:

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model

# Reference

- R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, Samsung