3GPP TSG-RAN WG4 Meeting #117		   				 		  							R4-2520033

Dallas, USA, 17th – 21th Nov., 2025

Agenda Item:	6.6.3.2

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for NGSO testing

Document for:	Discussion

# Introduction

The RRM test cases with dynamic channel model are addressed in this paper.

# Dynamic channel model for RRM test cases

According to WID RP-241281, the scope is to specify UE RRM performance test of uplink timing in the FR1-NTN bands, and other RRM tests are not included.

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

#### Issue 2-1-1: R19 UL timing tests and test requirement impact

Agreement

- Apply the new FR1-NTN channel model for R19 UL transmit timing test cases

  - No impacts for the test procedure or test requirements; apply the following adaptation

    - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

    - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts.

    - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

  - Do not add new additional margins in the test case. Test tolerance is left for RAN5

- FFS applicability of the timing advance test and its additional margin

Regarding the uplink timing tests, whether to include timing advance test was discussed in the last meeting.

In our view, a dynamic channel model is necessary to simulate these dynamic changes (such as Doppler and timing shifts) that occur due to UE movement and ephemeris data realistically, but a single comprehensive test run can sufficiently cover the intended verification.

The purpose of Timing Advance Tests is to ensure the UE can correctly adjust its timing in response to commands (TAC) from the network. This mechanism is largely does not significantly depend on the channel dynamics, so using a dynamic channel model offers little additional value for this test.

Proposal 1: Apply the dynamic channel to R19 UL transmit timing test cases and do not apply it to the timing advance test.

# Summary

Our observations and proposals in this paper are summarized as below:

Proposal 1: Apply the dynamic channel to R19 UL transmit timing test cases and do not apply it to the timing advance test.

# References

[1] R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.), RAN4#116bis, Oct. 2025.