3GPP TSG-RAN WG4 Meeting #117		   				 		  							R4-2510034

Dallas, USA, 17th – 21th Nov., 2025

Agenda Item:	5.10.2

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for NR NTN Ku bands

Document for:	Discussion

# Introduction

This outlines our view on outstanding issues of RRM performance requirements for the NR NTN Ku band, as discussed in the WF [1].

Issue 1-1-1: Measurement accuracy with new band

…

- FFS the band group to include Ku band

RRM performance requirements for NR NTN Ku bands

While the minimum Io/SSB_RP of SAN FR1 (as in Table 10.1.2C.1.1-1) is set with respect to NR operating band groups, for VSAT OTA using Ka band/FR2-NTN as a reference, the Minimum Io (Minimum SSB_RP) can be derived as in B.2.1.7 in TS.38.133 (as in Table 10.1.3C.1.1-1in 38.133)

- Minimum SSB_RP = Reference sensitivity UE_Type, n512, 50 MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n

where the OTA reference sensitivity can be derived from the vendor declared EIS, as in clause 10.3 of TS 38.101-5.

Table 10.1.3C.1.1-1: SS-RSRP Intra-frequency absolute accuracy in FR2-NTN

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 7 | 10 | ≥-6 | Same value as SSB_RP derived from annex B.2.1.7, according to UE VSAT type |  | N/A | -70 |
| 9 | 12 |  | N/A |  | -70 | -50 |
| NOTE 1: Values based on EIS as defined in clause 10.3 of TS 38.101-5 [42]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 10, 15, 20, 25, 35, 50, 70, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | ≤ -122 |
| n248, n247, n509, n508 | Mobile VSAT | 4, 5 | ≤ -126 |
| n248, n247, n509, n508 | Mobile VSAT | 6 | ≤ -122 |

Therefore, a simple approach is to extend the current FR2-NTN measurement requirement to cover FR1-NTN Ku band VSAT as below.

Table 10.1.3C.1.1-1: SS-RSRP Intra-frequency absolute accuracy per NTN VAST in FR1-NTN or FR2-NTN

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| 7 | 10 | ≥-6 | Same value as SSB_RP derived from annex B.2.1.7, according to UE VSAT type |  | N/A | -70 |
| 9 | 12 |  | N/A |  | -70 | -50 |
| NOTE 1: Values based on EIS as defined in clause 10.3 of TS 38.101-5 [42]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Observation 1: For NTN VSAT OTA, minimum Io/SSB_RP is derived from vendor‑declared EIS rather than set per band group.

Proposal 1: Extend the current FR2-NTN measurement requirement to cover FR1-NTN Ku band VSAT.

#### Issue 1-2-4: Test cases for Ku bands in FR1-NTN

WayForward

- Further discuss the following options

- Option 1: reuse exiting test cases defined for SAN FR1

  - Add a table clarifying test applicability for the corresponding test cases for VSAT UEs.

  - Add OTA related parameters

- Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified

  - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Regarding the FR1-NTN test configuration, adopting option 2 appears simpler than adding new OTA parameters.

Proposal 2: For Test cases for Ku bands in FR1-NTN, reuse existing test cases defined for FR2-NTN unless otherwise specified and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology (Option 2).

# Summary

Our observations and proposals in this paper are summarized as below:

Observation 1: For NTN VSAT OTA, minimum Io/SSB_RP is derived from vendor‑declared EIS rather than set per band group.

Proposal 1: Extend the current FR2-NTN measurement requirement to cover FR1-NTN Ku band VSAT.

Proposal 2: For Test cases for Ku bands in FR1-NTN, reuse existing test cases defined for FR2-NTN unless otherwise specified and introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology (Option 2).

# References

- R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.), RAN4#116bis, Oct. 2025.