3GPP TSG-RAN WG4 Meeting #117		   				 		   R4-2520040

Dallas, USA, 17th – 21th Nov., 2025

Agenda Item:	8.6

Source:	MediaTek Inc.

Title:	Views on 6G RRM

Document for:	Discussion

# Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G RRM requirements with the focus on the following aspects:

- Measurement gap (MG) and interruption

  - MG framework

  - Fast RF retuning/interruption-free

- RRM framework: Measurement capability

- Mobility and HO interruption reduction

- Spectrum aggregation and SCell activation

- Other PHY signal/channel/procedure related RRM

  - UE Tx timing

  - MRTD

# Measurement gap (MG) and interruption

In the previous RAN4#116-bis meeting, RAN4 reached the following agreement [2]:

Agreement: - Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion. [Way forward]: - FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity))

In this document, due to the limited TU for the 6G study item, the discussion in this technical paper is focused on measurement gaps, gapless, UE assisted MG configuration, concurrent gaps and RF-retuning/interruption are discussed. In our views, other gaps-related topics can be revisited at later stage during this study item, if time allows.

## Measurement gaps and gapless framework

This section provides discussion on how measurement gaps are handled in NR (New Radio), especially on how the RRM requirements have evolved from Rel-15 to Rel-19 and how does that pave the way to 6G systems.

The measurement process usually begins when the serving cell’s RSRP measurement drops below a certain threshold—meaning the UE is no longer in the cell centre. At this point, the network configures the UE with measurement objects (MOs) so it can measure neighbouring cells. These MOs refer to reference signals in target bands, which may belong to intra-frequency, inter-frequency, or inter-RAT cells relative to the serving cell. Generally, a UE requires a measurement gap (MG) to measure reference signals (indicated by the MOs) on non-serving carriers, e.g., on a target cell operating in a band outside its serving cell’s band combination. If the UE informs the network in advance about which bands it can measure with or without a gap, based on this information the network can configure the UE with the MG configuration to enable these measurements. Throughout 5G NR development, multiple measurement gap related features where introduced, however, not all these features were successfully deployed in real field. In Rel-15/16 NR, two types of measurement gaps (Type 1 MG) were defined: per-UE and per-FR measurement gaps. In Rel-17, three enhancements were introduced: (i) concurrent MG (Type 2 MG); (ii) Pre-configured MG patterns (Pre-MG); and network-controlled small gap (NCSG). Rel-18 defined RRM requirements for the NeedForGap (NFG) feature, and Rel-19 introduced further work on measurement gap cancellation in the XR work item. A high-level summary of these features is given below:

Table 1 Summary of Gap features

| MG features | Details/pros/cons | Whether to consider this in 6G SI |
| --- | --- | --- |
| Type 1 MG | Pros: - gap location is known to both NW and UE - easy implementation from NW and UE Cons: - MGL is too long - Impacts all UL/DL data carriers | MG might be necessary for some scenarios, yet, the conventional MG impact the UL/DL scheduling and hence further study is needed on how to enhance this in 6G systems. Suggestion: include Type 1 MG in 6G SI. |
| Type 2 MG (concurrent-MG) | Pros: - gap location is known to both NW and UE - flexible configurations for NW Cons: - MGL is too long - Impacts all UL/DL data carriers - Complex MG collision handling | The con-current gap is important to improve NW configuration for MGs flexibility. Hence, RAN4 to further study whether this is needed from initial 6G releases. Suggestion: include Type 2 MG in 6G SI with simplifications, e.g., on the collision handling. |
| Pre-MG | Pros: - gap location is known to both NW and UE - un-needed MG occasions are cancelled Cons: - MGL is too long - Impacts all UL/DL data carriers - Limited benefits to intra-freq cells meas. - Does not work as long as one MO requires MG | This feature is associated with BWP switching and given it is not clear whether we have the same concept of BWP in 6G then there is no need to include this in 6G SI. Suggestion: not to discuss Pre-MG in 6G SI. |
| NCSG | Pros: - gap location is known to both NW and UE - UE can be scheduled during ML Cons: - Interruption (VIL) is still too long - Working assumption with an idle RF chain | This feature is beneficial compared to using gaps in all scenarios. Yet, further study on how to reduce the impact of interruption is needed. Consider this feature for 6G SI. Suggestion: include Gapless with/without interruption in 6G SI. |
| NFG | Pros: - UE might avoid interrupting scheduled slots - UE can be scheduled during meas. Cons: - Interruption is too long - Interruption location is un-known to NW - Working assumption with an idle RF chain | This feature is beneficial compared to using gaps in all scenarios. Yet, further study on how to reduce the impact of interruption is needed. Consider this feature for 6G SI. Suggestion: include Gapless with/without interruption in 6G SI. |
| MG cancellation | Pros: - Allowing more slots for data scheduling - Dynamic MG cancellation based on data traffic Cons: - Measurement delay becomes too long - Increase UE complexity | This feature is dependent on data traffic. Besides, the necessity of such feature is dependent also on how an MG occasion impact the data scheduling and given that the MG occasion structure is still to be discussed. A foundation of MG framework without cancellation is a prerequisite for this figure. Then, this feature can be left for future 6G releases. Suggestion: not to discuss MG cancellation in 6G SI. |

Based on the analysis provided in the table above, this document focuses on discussing: (i) Conventional MG (Type 1); (ii) Gapless with/without interruption; and (iii) general issues.

Conventional measurement gaps: In 5G NR spec, measurement gaps are defined in terms of measurement gap length (MGL) and measurement gap repetition period (MGRP), as in Table 9.1.2-1 [38.133]. According to the working assumptions in RRM group, the MGL consists of measurement length (ML) plus two parts of RF retuning time (RRT), as depicted in the figure below. Typical measurement length in FR1 can be either 5 ms or 2 ms, while the RF retuning is 0.5 ms at the sides of ML, resulting in 6 ms and 3 ms MGL. The mandatory gap patterns are configurations 0, and 1, which are MGL = 6ms, MGRP = 40ms/80ms, resulting in up to 15% interruption of data transmission and reception. Furthermore, the use of a measurement gap affects all uplink (UL) and downlink (DL) carriers, temporarily blocking data transmission and reception at the UE. As a result, the UE can only perform measurements on the specific target band or carrier defined by the measurement object. Therefore, the 15% of data interruption can impact system performance (e.g. lower throughput), also, a UE might not be able to achieve certain service requirements, such as latency.

Observation 1: Measurement gaps are typically responsible for a 15% interruption of data transmission and reception (e.g. MGL = 6ms, MGRP = 40ms), which impact user experience in terms of throughput and latency.

Proposal 1: Investigate methods to reduce the use of measurement gaps in 6G systems.

ML

Time

MGRP

MGL

RRT

MG

ML

RRT

Tx/Rx data only

Tx/Rx data only

No data 
Tx/Rx

Figure 1: representation of measurement gaps

In previous generations of mobile systems, such as LTE and NR, a User Equipment (UE) can indicate whether a MG is required to perform measurements on a target frequency band. However, the specifications do not explicitly state why a measurement gap is needed; as the spec only allows the UE to request one. It is generally understood within the industry that the UE requests an MG when it lacks sufficient RF chains to simultaneously measure frequencies outside its current operating carriers. In such cases, RF retuning is necessary to measure carriers on different frequency bands.

Additionally, the UE may require a measurement gap when the measurement object refers to reference signals in a target band that lies outside the operating band combination specified in 3GPP TS 38.101. In these scenarios, the need for a measurement gap arises due to desense (i.e., self-interference) between the serving cell bands and the target cell band, which contains the reference signals necessary for measurement. The desense, mentioned earlier, refers to interference caused by the UE's own transmissions or receptions, which can degrade measurement accuracy.

Observation 2: In LTE/NR, a UE might report a need for a measurement gap when RF retuning is necessary to measure frequency carriers outside its operating carriers.

Observation 3: In LTE/NR, a UE might report a need for a measurement gap due to a desense (MSD) issue between the target band, containing the SSB to be measured, and the serving bands.

Proposal 2: Identify and address the reasons behind measurement gaps in previous generations of mobile systems and how to handle these issues in 6G systems.

Gapless solutions: Gapless measurement solutions with/without interruption have been specified, allowing the UE to perform measurements without requiring conventional measurement gaps. These gapless solutions were first considered in 5G NR starting from Rel-16. On one hand, the NeedForGap solution was specified in terms of signalling since Rel-16, but the specifications did not fully address the RRM requirements until Rel-18. This created ambiguity regarding expected UE behaviour, particularly in terms of how the UE should avoid the impact of RF retuning during measurements. On the other hand, the NCSG solution was introduced in both signalling and RRM requirements in Rel-17. Although the specifications for NCSG were comprehensive, the late introduction of this feature made it challenging for companies to implement and deploy the solution in their products.

Based on the discussion during the RRM 6G study item, the above is a common understanding among all companies. Also, due to the foreseen benefits of gapless, RAN4 reached the agreement below, which considers gapless as part of the 6G SI. Nevertheless, to enable gapless in 6G systems, RAN4 needs to address some critical issues and address the challenges associated with it.

Agreement: - Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

General issues:

Issue 1: Available RF chain: As mentioned earlier in this document that the use of a measurement gap affects all UL and DL carriers, temporarily blocking data transmission and reception at the UE. As a result, during this measurement gap, the UE is only required to perform measurements on the target band or carrier specified by the measurement object. Nevertheless, from an implementation perspective, a single RF chain is typically required for the UE to perform measurements, with this RF chain operating as a downlink receiver for a certain carrier. This working assumption of un-used RF chain was the baseline consideration for 5G NR gapless solutions.

Additionally, it should be noted that UEs that support carrier aggregation are equipped with multiple RF chains, enabling simultaneous transmission and reception on multiple carriers. Therefore, it is necessary to study whether it is truly required to interrupt all UL and DL carriers during the MGL. In other words, further investigation is needed to determine if the measurement gap can be limited to specific number of serving carriers, rather than affecting all of them.

It should be noted that there is no impact on unused RF chains (i.e. gapless). Hence the RRM discussion shall differentiate UE behaviour based on the availability of RF chains.

Observation 4: In 5G NR, during measurement gaps length, a UE interrupts uplink and downlink data transmission and reception across all serving cell carriers to allow measurement of a target SSB of a target cell.

Proposal 3: Study the possibility of limiting the interruption or measurement gap to specific carriers (per-CC gap) without affecting the remaining serving cell carriers.

Observation 5: From a UE implementation perspective, whether none or specific number of carriers should be interrupted depends on the availability of an idle RF chain to perform measurements on the target cell.

Proposal 4: When defining measurement gap/interruption requirements and reporting signalling, different UE behaviours should be specified based on the availability of an idle RF chain.

Issue 2: UE assisted measurement gaps: There is no doubt that restricting measurement gaps during the measurement gap occasion to a specific number of serving carriers can improve system performance, particularly in terms of latency and throughput. However, during an offline discussion with experts from other companies at the previous meeting, a key concern was raised regarding the design of signalling between the network and the UE. The main challenge stems from the fact that the network does not know which target cell the UE is measuring during each gap occasion. As a result, implementing UE-assisted measurement gaps could be advantageous for both the network and the UE, as it would enable the use of limited gaps to specific carriers. Therefore, RAN4 to consider UE assisted measurement gap as part of the initial release of 6GR.

Observation 6: UE-assisted measurement gaps allow measurement gaps to be applied only to specific carriers, improving efficiency and reducing unnecessary interruptions.

Proposal 5: RAN4 to study and consider UE assisted measurement gap as part of the initial release of 6GR.

Issue 3: RF retuning time: In 5G NR, the working assumption values for RF retuning were 0.5 ms for FR1 and 0.25 ms for FR2. However, this number was based on initial NR implementation and have never been optimised throughout NR enhancements. Besides, from UE implementation perspective, it is expected that the RF retuning is much faster than that working assumption values. In NR, some Wis already introduced way faster RF retuning time for Rx or Tx switching (e.g., 35 us or 140 us). Therefore, further study shall be conducted to find new working assumption that are closer to real values based on latest UE implementation advancements.

Based on the discussion during the RRM 6G study item, the above is a common understanding among all companies. Also, due to the foreseen benefits of enhancing the RF retuning and interruption time, RAN4 reached the agreement below, which considers interruption enhancements as part of gapless in the 6G SI.

Agreement: - Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

Now, RAN4 needs to highlight the direction for enhancing the interruption requirements including interruption period and its side conditions. For a start, the legacy interruption requirements were based on slot-level granularity, and hence RAN4 shall thrive to define requirements based on symbol-level granularity.

Observation 7: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 8: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 6: In defining interruption, including RF retuning, RAN4 shall consider symbol-level granularity rather than slot-level granularity.

Issue 4: Visible/invisible interruption:

It is commonly understood that interrupting DCI would cause more serious issue in network scheduling. Compared to interrupting PDSCH. PDCCH requires better protections because it may carry more important indications, such as UL grant, BWP switch, TCI-state switch. In addition, for some data traffic, the UE is not fully scheduled all the time and hence a UE might be able to place interruption in symbols without impacting control and data packets. Therefore, RAN4 selects best possible location for the interruption to minimize PDCCH, HARQ ACK and data loss.

Proposal 7: RAN4 selects best possible location to minimize PDCCH, HARQ ACK and data loss.

In 5G NR gapless solutions, interruption requirements can be classified as either visible (known to both the network and UE) or invisible (known only to the UE). Visible interruptions are defined by their duration, with the assumption that the network will not schedule the UE for transmission or reception within the interruption period. In contrast, invisible interruptions are characterized by their duration, percentage, and the occurrence of missing ACK/NACK messages from the UE to the network, since the network is unaware of the exact interruption timing. For 6GR systems, RAN4 will discuss the advantages and disadvantages of each interruption method.

Proposal 8: For 6GR gapless solutions, RAN4 should further study whether interruptions should be specified as visible or invisible to the network.

Issue 5: UE self-interference caused by IMD/harmonics: In current specifications for cellular systems with carrier aggregation (CA), maximum sensitivity degradation (MSD) is defined for different CA band combinations to account for various sources of UE self-interference, such as intermodulation (IMD) and harmonics. Similarly, when a UE is configured to measure the reference signal of a target cell operating in a band outside its current CA band combination, the UE faces a challenge: it must determine whether IMD or harmonics will interfere with the reference signal of the target cell. As a result, the UE may request measurement gaps for the SSB in a target band that is not part of its currently configured CA bands.

For example, consider a UE operating in NR band n3 with uplink transmission, while also being configured to measure reference signals in target bands n77 and n46. Using the uplink harmonics formula, the figure below illustrates how harmonics generated by band n3 may impact the SSBs to be measured in bands n77 and n46.

Table 2: UL harmonics formula.

| UE UL carrier with band limit fx_low to fx_high | Harmonics formula for a band with centre freq fx |  |
| --- | --- | --- |
| Xth harmonics frequency limits | X* fx_low | X* fx_high |

Figure 2: UL harmonics of NR band n3 falling on SSB for NR bands n77 and n46.

Two main issues arise in such configurations:

- Issue#1: Direct Overlap, harmonics may directly overlap with the SSB reference signals. For instance, uplink harmonics from band n3 can overlap with an SSB in band n77.

- Issue#2: Measurement Gap Reporting, even if the uplink harmonics from band n3 do not directly overlap with the SSB in band n46, the UE’s measurement gap reporting is based on per-band reporting.

Therefore, in existing systems, the UE might request a measurement gap to measure SSBs, regardless of the actual availability of RF chains and regardless of whether there is direct overlap in terms of UE self-interference.

In our views, RAN4 should study the above two issues and address them in terms of defining requirements. For issue #1-direct overlap, RAN4 to further study the impact due to UE self-interference caused by IMD/harmonics and to find solution to limit the gap reporting for such scenarios. For example, scheduling restriction or limited MG on UL carriers can be considered. While for issue #2-measurement gap reporting, finer granularity reporting can be considered, thus a UE doesn’t need to report need for a gap unless there is a direct overlap between the UE self-harmonics and the SSB in frequency.

Observation 9: Direct Overlap, harmonics may directly overlap with the SSB reference signals. For instance, uplink harmonics from band n3 can overlap with an SSB in band n77.

Proposal 9: RAN4 to further study the impact due to UE self-interference caused by IMD/harmonics and to find solution to limit the gap reporting for such scenarios.

Observation 10: In 5G NR, gap reporting capability is based on frequency bands and is independent of the SSB location within the band, also, the SSBs within a band may be affected by IMD and harmonic products present in that band, however, not all SSBs within the band are necessarily impacted.

Proposal 10: Introduce finer gap reporting granularity, i.e. smaller than per target band, to reduce unnecessary measurement gaps.

Issue 6: Non-colliding multiple GAPs: In real field scenarios, different target cells are expected to require distinct measurement configurations, and each measurement is therefore associated with a specific MO. Associating different MO to different MG configurations has the potential of reducing unnecessary measurement gap occasions and increase network configuration flexibility. However, although the UE can be configured with different measurement settings via multiple MOs, these MOs are still constrained by the number of configured MG or NCSG in 5G NR. To address the issue, in 5G NR concurrent gap solutions have been introduced to address this limitation, such as concurrent gaps with NCSG. A key issue with concurrent gaps is that, while signalling allows for multiple gap configurations, the RRM requirements restrict the UE to a maximum of two configured gaps. This restriction is primarily due to the potential for collisions between the configured gaps, which limits the system to only two MG configurations. Furthermore, the concept of associating specific MOs with particular types of gap or gapless mechanisms has not been fully utilized, again mainly because of the two-gap limitation. Therefore, for 6G systems, it is necessary for RRM to further investigate how to associate different MOs with different gap or gapless mechanism. This would enable multiple gap configurations, allowing MOs for different carriers to be treated independently in terms of measurement requirements.

Observation 11: From a system operation perspective, applying different measurement gap configurations for different carriers can reduce unnecessary measurement gap occasions and increase network configuration flexibility.

Observation 12: In 5G NR, concurrent measurement gaps were introduced to improve network configuration flexibility and support a wider range of use cases. However, current RRM requirements limit the number of concurrent gaps to a maximum of two due to the collision among the configured gaps.

Proposal 11: Investigate the introduction of non-colliding multiple concurrent measurement gaps in 6G systems from the initial release.

Issue 7: Measurement gaps and gapless spec structure: In 5G NR RRM requirements, L3 measurements requirements are defined based on intra-f, inter-f and inter-RAT. Then, within each clause of these measurement types, there are further sub-clauses covering whether this measurement is done with gaps, interruption, NCSG, gapless. For example, the requirements are given as:

- NR intra-frequency measurements:

  - Intra-frequency measurements without measurement gaps.

  - Intra-frequency measurements with measurement gaps.

  - Intra-frequency measurements with NCSG.

- NR inter-frequency measurements:

  - Inter-frequency measurement with measurement gaps.

  - Inter-frequency measurements without measurement gaps

  - Inter-frequency measurement with NCSG

- Inter-RAT measurements:

  - NR – E-UTRAN measurements with autonomous gaps.

  - NR – E-UTRAN measurements without measurement gaps

However, the common shared part among these clauses is that the measurement with gaps/NCSG and without gaps. To simplify the implementation of the RRM specs, it should be studied on how to simplify these requirements writing to be categorised differently. For start, RAN4 to consider the following categorization, which is based on the mechanism used for measurements:

- Clause x-1: Measurements within gaps (including CSSF and delay)

- Clause x-2: Measurements outside gaps (including CSSF and delay)

  - Clause x-2a: Measurements with interruption/NCSG

  - Clause x-2b: Measurements without interruptions

Therefore, all L3 measurements that need gaps can be mentioned in the Clause x-1, while other L3 measurements can be written within the measurement without gaps (i.e. Clause x-2).

Observation 13: In 5G NR, RRM spec implementation for measurement is rather complex and has too much repetition between different clauses, such as inter-f and intra-f.

Proposal 12: RAN4 to study the consideration of spec writing that measurement categorization is written based on measurement mechanism, i.e. gaps/NCSG/interruption/gapless, such as:

- Clause x-1: Measurements within gaps (including CSSF and delay)

- Clause x-2: Measurements outside gaps (including CSSF and delay)

  - Clause x-2a: Measurements with interruption/NCSG

  - Clause x-2b: Measurements without interruptions

## Fast RF retuning/interruption-free

The interruption defined in RRM requirements is associated with various procedures, such as Bandwidth Part (BWP) switching, and consists of both RF retuning and baseband (BB) processing time. This results in an overall interruption requirement of approximately 1 ms. Based on current working assumptions, RF retuning is expected to take 0.5 ms for FR1 and 0.25 ms for FR2, while BB processing time is assumed to be 0.5 ms. However, it is important to note that BB processing time may not impact all carriers in the same way as RF retuning does. Furthermore, recent advancements in UE implementation suggest that RF retuning can be performed much faster than the original assumption of 0.5 ms from NR Rel-15. For a start, the legacy interruption requirements were based on slot-level granularity, and hence RAN4 shall thrive to define requirements based on symbol-level granularity.

In addition, it is commonly understood that interrupting DCI would cause more serious issue in network scheduling. Compared to interrupting PDSCH. PDCCH requires better protections because it may carry more important indications, such as UL grant, BWP switch, TCI-state switch. In addition, for some data traffic, the UE is not fully scheduled all the time and hence a UE might be able to place interruption in symbols without impacting control and data packets. Therefore, RAN4 selects best possible location for the interruption to minimize PDCCH, HARQ ACK and data loss.

Observation 14: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 15: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 13: In defining interruption, including RF retuning, RAN4 shall consider symbol-level granularity rather than slot-level granularity.

Proposal 14: RAN4 selects best possible location to minimize PDCCH, HARQ ACK and data loss.

# RRM framework

## Measurement capability

In 5G, 3GPP TS 38.133 (9.2.3, 9.3.3) specifies the UE measurement capability for number of cells. The requirement were introduced to support cell reselection and handover preparation.

- intra-frequency layer: 8 identified cells in FR1, 6 identified cells in FR2.

- inter-frequency layer: 4 identified cells in FR1 and FR2.

However, the maximum number of detectable cells (i.e. 8 cells) at the cell edge area is rarely observed from the 5G NR field data, especially under the -6 dB side condition. Table 3 provides an example of field log showing the number of detected cells in FR1. The data indicate that, with 90% probability, the number of detected cells in FR1 is <=4, and with 97% probability it is <=5. However, these values depend on the deployment scenario (e.g., cell size and operating frequency). This suggests that current requirements may overestimate practical needs, leading to unnecessary hardware complexity.

As 6G potentially utilizes more robust SS design and higher number of UE antennas. This increases the computational and memory burden on UE if the measurement requirements remain unchanged or are scaled up, RAN4 should reassess UE measurement requirements for 6G (i.e. cells and beams).

Table 3 Number of detected intra-frequency neighbor cells in FR1

| # of Detected neighbor cells (SINR>-6dB) | Probability |
| --- | --- |
| 0 | 2.7% |
| 1 | 15.1% |
| 2 | 27.8% |
| 3 | 28.4% |
| 4 | 16.4% |
| 5 | 7.1% |
| 6 | 2.0% |
| >=7 | 0.4% |

Observation 16: The UE capability of number of cells is underutilized in 5G.

Proposal 15: Study the appropriate measurement capability for number of cells in 6G. The number of detected cells in FR1 is <=4 with 90% probability and <=5 with 97% probability.

## Measurement delay overhead

Number of searchers depends on the UE architecture and other core features such as CA design etc, which is currently discussed on other WGs. In 5G NR, the number of searchers has been studied thoroughly and optimised in recent work item. Therefore, there is no need to occupy the 6G SI with discussion on the number of searchers.

Observation 17: The number of searchers has been thoroughly studied and optimised in latest release in 5G NR, also, it is correlated to the UE architecture, which is discussed in other WGs.

Besides, reducing the Rx beam sweeping factor in FR2 impacts the UE performance directly. Hence, making 6GR UE in FR2 worse compared to that of 5G NR in FR2. Also, in 5G NR, the number of searchers has been studied thoroughly and concluded with current 5G Rx beam sweeping factor. Therefore, for RX beam sweeping factor in FR2 can be reused for 6GR. Thus, there is no need to study Rx beam sweeping factor in FR2 for 6GR.

Observation 18: Reducing the Rx beam sweeping factor in FR2 can directly impact the UE performance.

Proposal 16: Deprioritize the study of Rx beam sweeping factor in FR2 and the number of searchers for 6GR SI.

## Unified measurements

In 5G, various measurement types, requirements, and side-conditions have been defined, each serving different purposes and use cases. The existing RRM measurement framework is a sophisticated design that provides the network with flexibility to enable specific measurements as needed. For example, in LTM, the network configures L1-RSRP to accelerate the handover procedure. This measurement has different requirements and side-conditions compared to L3-RSRP, which is used for general neighbouring cell measurements.

While the concept of unified measurement requirements (e.g., delay, accuracy, side conditions, …) for different procedures may initially seem appealing, it can have significant negative impacts on both UE performance and network configuration flexibility. Therefore, RAN4 should prioritize identifying the actual issues within the current measurement framework before proposing new enhancement solutions.

Observation 19: The RRM measurement framework in 5G NR is highly sophisticated, offering tailored requirements and operational conditions for a variety of use cases.

Proposal 17: It is recommended that RAN4 deprioritize the study of a unified measurement framework at this stage and instead focus on thoroughly identifying and understanding the actual challenges and limitations within the current 5G NR system.

# Mobility and HO interruption reduction

Usually, RAN2 leads the discussion on mobility. RAN4 starts the discussion on mobility after RAN1/2 have sufficient progress. It may be not easy to carry out the discussion on enhancements with RAN1/2 dependency. But RAN4 can start from the solutions with less RAN1/2-dependency.

Proposal 18: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

In traditional HO solution, UE needs to do DL synchronization on target cell, which contribute to a large part of HO delay and interruption. The longer SSB period is, the longer time it takes for DL synchronization. So do the HO interruption. As far as we know, there are many proposals in RAN1 to extend 6G SSB periodicity, which will potentially lead to longer HO interruption. For LTM, the delay and interruption can be reduced compared to traditional HO through early DL synchronization. But the gain can only be obtained under some conditions, e.g., measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms. Further check the conditions, it can be observed that the conditions are even more difficult to meet with longer SSB periodicity. So in 6G SI, we need to study the solutions that can mitigate the problem of longer SSB periodicity.

Proposal 19: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Before discussing 6G HO solutions, we would like to recall the enhanced handover features in NR, i.e., CHO, DAPS, LTM and C-LTM. We summarized the pros and cons of DAPS, LTM and CHO as in Table 4.

Table 4 Enhanced handover features in 5G

|  | Pros | Cons |
| --- | --- | --- |
| DAPS | Interruption is the shortest. - Source cell will keep the link until UE sets up the connection with target cell | limited applicable scenarios or high UE complexity |
| LTM | Delay and interruption can be reduced through early RRC decoding, early DL/UL synchronization. | - Complex conditions to get the gain of early DL synchronization - measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms - May be difficult to ensure measurement period within 160ms with even longer SSB periodicity in 6G. - Extra complexity to support early T/F tracking - Extra memory and conditions for early ASN.1 decoding - Additional interruption due to early RACH |
| CHO | Avoid the latency introduced by interaction between UE and NW (UE reports measurement results and then NW sends HO command) - HO initiated by UE under the guidance of NW | - Early preparation needed at Network - Potential resource waste due to network is not aware of when interruption starts. |
| Soft satellite switch | Interruption can be reduced | - Only supported in NTN |

It can be observed that the interruption length of DAPS is the shortest at the cost of higher UE complexity, i.e., extra hardware or spare CA resource. Due to this, DAPS is applicable for limited scenarios. As mentioned above, the gain of early DL synchronization can only be obtained with some conditions in LTM. In 6G Day-1, the designed HO solution should be applicable to all the scenarios and work well without conditions. Therefore, we don’t think it is enough to reuse one of the enhanced HO features in 6G.

Proposal 20: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

To achieve the above goal as in Proposal 20, the 6G HO solution should get the advantages of individual enhanced HO features while resolving their problem as much as possible. Usually, NW stops the link “source cell ↔ UE” after sending HO/cell switch command, e.g., in legacy L3 HO and LTM. So HO interruption is almost the same as HO delay. And most of the time, reducing HO delay is similar as reducing HO interruption. There may be some other ways to counteract the impact of long delay, e.g., trigger HO earlier. But there is no way to counteract the impact of long interruption. So it is more important to reduce HO interruption.

Proposal 21: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Even in NR DAPS, the KPIs of HO delay and HO interruption have been decoupled. The main concept of DAPS is that source cell will keep the link until UE sets up the connection with target cell. In this way, the interruption length can be much less than the delay to achieve nearly “0” interruption. But to keep dual active links, i.e., source cell ↔ UE and target cell ↔ UE, it requires UE to be able to receive/transmit with both source and target cell simultaneously, and leads to high UE complexity.

Observation 20: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 21: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Checking the current HO procedure, upon receiving the HO command, UE will right away decode the RRC (TRRC) and apply all the new configurations of the target cell (Tprocessing). After Tprocessing, UE cannot be scheduled by the source cell. As a result, the interruption starts, as shown in Figure 3.

However, in our view, UE can choose when to apply the new configurations. The later UE applies the new configurations, the longer UE can maintain the link with the source cell. For example, UE can perform RRC decoding, DL synchronization as well as the time waiting for the RACH occasion, without interruption on serving cell(s) as shown in Figure 3. But after Tprocessing, UE usually breaks the link with serving cell except in DAPS. Inspired by DAPS and LTM, we propose a HO procedure as shown in Figure 4. In the new HO procedure, Tprocessing is performed just before RACH occasion. In this way, even for blind HO, HO interruption can be reduced to as short as “ Tprocessing”. To get the gain, it requires source cell keeps the link until UE starts to break the link with source cell, i.e., before Tprocessing. In Figure 3, we use unknown HO which includes the most procedures during HO as an example. For other scenarios, the difference is that some components are not needed, e.g., Tsearch is no needed for known scenarios. No matter which scenario it is, the proposed solution is also applicable.

Figure 3 Timeline of legacy handover procedure

Figure 4 Proposed HO procedure

Actually, similar solution has been adopted for NTN as captured in 38.133 clause 6.1C.3.2.3. As shown below, in soft satellite switch, UE is supposed to perform Tprocessing after finishing T/F tracking and t-Service. Source satellite can keep serving UE before Tprocessing. In 6G, it is preferred to have unified solution for both TN and NTN if possible.

Figure 5 Timeline of Soft satellite switch

Proposal 22: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 23: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

Proposal 24: Better to have unified handover solution for both TN and NTN if possible.

Based on offline discussions, it seems there are concerns on that the location of interruption may be unknown to NW. For brainstorming, we list several potential methods to solve this issue:

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

A potential issue of this option is whether UE is allowed to handover to target cell faster than the minimum requirements. If interruption time starts earlier and NW keeps scheduling during the interruption time, retransmission after UE camps on target cell is needed and some resources are wasted. So UE can be allowed to handover faster but at the cost of wasting some NW radio resources.

- Option 2: Introduce new signalings to have common understanding between network and UE on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

It is equivalent to define a lower bound of HO delay requirement.

We believe there can be other methods to solve the ambiguity issue. We are open for more discussions.

Proposal 25: Discuss whether and how to let NW know when interruption starts. The following options can be used as a starting point.

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

- Option 2: Introduce new signalings to have common understanding on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

- Option 4: Others.

To further reduce HO delay, early preparation before HO/cell switch command, e.g., early RRC decoding, early DL/UL synchronization, is helpful. As mentioned in Table 4, supporting early T/F tracking requires some additional UE complexity. So the number of active TCI states that UE can support is limited. So for early RRC decoding. In 5G, early T/F tracking and early RRC decoding is totally under NW control. But the fact is that UE is always the first to know which cell is getting better. There will be always some latency introduced by reporting the measurement results. Inspired by CHO, we propose to support UE initiated early RRC decoding and early T/F tracking in 6G for better use of limited UE capability.

Proposal 26: Support UE initiated early RRC decoding and early T/F tracking in 6G.

# Spectrum aggregation and SCell activation

Cell with multiple carrier

In 5G, the carrier-aggregation (CA) framework is critical for spectrum aggregation, but it limits physical layer and protocol efficiencies because by artificially enforcing a one-to-one coupling of each physical carrier component (CC) and a logical cell, resulting in inefficiency as noted in proposed observation 9.1b with regards to spectrum utilization and aggregation at RAN1#122bis.

- Slow and complex activation of additional carrier

- Features (such as HARQ) defined per carrier leads to sub-optimal performance

- Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint

- limited applicable scenario of SSB adaptation for Scell

- ….

Specifically, within a contiguous spectrum, the current CA framework artificially makes a multi-carrier operation appears more cumbersome and less efficient than a single wider bandwidth from a system perspective.

For 6G, we recommend decoupling this one-to-one mapping and aggregating multiple carriers with similar propagation channel properties to a single multi-carrier cell. As illustrated in Figure 6, intra-band and nearby inter-band carriers can be aggregated into a single cell when they meet specific conditions, e.g., time and frequency alignment, frequency span, and receive power difference, thereby enabling fast carrier activation and reduce the control overhead. Based on 5G experiences, for example, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronisation reference and AGC can be used at a UE. In addition, it is a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where these conditions are typically met.

To increase the bandwidth of a multi-carrier cell, the interruption time associated with adding an additional carriers was discussed during an offline discussion. In our view, with the simplified 6G BWP currently under discussion in RAN1, the delay can be further reduced compared with the current BWP switch; moreover, because 5G already supports a 3 ms delay for SSB-less intra-band contiguous SCell activation, the interruption time for adding a carrier to a multi-carrier cell can be similar or lower when the corresponding conditions satisfied, since 6G allows delay and interruption to be decoupled, as proposed in our mobility contribution.

Observation 22: The interruption time to add a carrier in a multi-carrier cell can be kept below the current BWP switch delay and further reduced by decoupling delay from interruption.

Note that carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

While RAN1 will study multi-carrier cell support and procedures, RAN4 RF will need to study RF-switch time (to switch on a new carrier within a cell) requirements, acceptable Transmit timing alignment (TAE), BS frequency error, and total power differences at UE between carriers; RAN4 RRM can then assess aggregation delay, accounting for AGC resettling and fine time tracking.

Figure 6 Illustration of multi-carrier cell

Observation 23: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 24: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 27: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Due to the limited number of Tx chains at the UE, the number of bands that the UE can be configured for is constrained. For instance, a UE with two Tx chains can either be configured with one band using UL-MIMO or with two bands without UL-MIMO (one Tx chain per band). UL Tx switching allows the UE to dynamically switch the Tx chain(s) among bands, enabling configuration with more bands and supporting UL-MIMO transmissions without requiring additional Tx chains. UL Tx switching was introduced incrementally from R16 to R19. Each release targets a different scenarios, which adds unnecessary limitation to 5G networks. For example, if more than 2 UL bands are needed, it can only be supported from Release 18. For 3Tx UE, it can only be supported in Rel-19, but it is limited in 2 UL bands. Enabling flexible carrier switching in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity by ensuring the uplink resources for the UE can be allocated where resources are available. Therefore, improvements for UL Tx switching should be investigated in 6G to simplify and streamline its operation.

Beyond UL, enabling more flexible DL carrier switching without requiring carrier aggregation could deliver benefits comparable to those achieved with UL carrier switching; in 5G it was considered only in Rel-19 with limited scope, while carrier switching is especially valuable under hardware constraints (e.g., few RF chains or synthesizers) and in the presence of DL/UL interference and intermodulation distortion, so the corresponding switching-delay will need to be studied in RAN4 RF and the interruption requirements can be further studied in RAN4 RRM.

For 6G, we expect a more flexible mechanism for DL/UL switching with forward compatibility on aspects on number of bands, number of Tx/Rx, … etc. In this 6G SI, it would be good to take this as one design target and study its corresponding impact in 3GPP specs.

Observation 25: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 28: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

Since Rel-15, 5G NR has continuously refined Scell activation to address a variety of conditions (with or without measurement reports, different synchronization states, power differences), diverse RS configurations (SSB-less, A-TRS, P/SP/AP CSI-RS), and different types of UE RF architecture (intra-/inter-band, co-/non-collocated), yet only limited scenario has seen widespread adoption in commercial 5G.

For 6G, the carrier aggregation activation should ensure timely CSI so the network can schedule sufficient radio resources for busty traffic, and the Scell activation should be specified only for critical, high-likely deployment cases to simply the requirement design.

Based on 5G experience, the SCell/carrier activation at least involves RF switch, AGC settling, cell search, fine sync tracking, and CSI report. RAN4 RF will need to study RF switch-time requirements, and RAN4 RRM will assess SCell activation interruption and the overall delay, taking other factors into account.

Observation 26: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 29: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

# Other PHY signal/channel/procedure related RRM

## UE Tx timing

Initial Transmission Timing

UL performance depends on the UE’s TX timing accuracy, with requirements tied to the DL reference signal SCS (for DL timing detection) and the UL data SCS to avoid the interference between UEs beyond the CP length. High SCS can make compliance difficult, e.g. 60 kHz in NTN, especially in the present of additional impairment such as GNSS/UE positioning inaccuracies and satellite position errors in NTN scenarios. However, those errors can be corrected after initial access.

Currently RACH shares the same timing requirement as UL channels such as PUCCH/PUSCH, but PRACH’s CP can be much longer than theirs, so its tolerance to timing error can also be much higher. Unnecessarily tight PRACH timing requirements will limit the use of high SCS, because after initial access the UE’s initial timing error can be corrected via the Timing Advance command and subsequent PUCCH/PUSCH with higher SCS will have chance to reduce the timing error within CP. Given PRACH’s distinct CP length and role in initial access, defining PRACH-specific timing requirements could better enable higher SCS.

Observation 27: Relaxing PRACH timing requirements would accommodate initial timing errors within CP, allow timing correction after initial access, and enable higher SCS. Especially in NTN scenarios, where impairments such as GNSS/UE positioning inaccuracies and satellite position errors can be corrected after initial access.

Proposal 30: Study PRACH (if introduced) specific timing accuracy requirement.

Timing Adjustment

In 5G, especially for advanced use cases such as Non-Terrestrial Networks (NTN) and high-speed trains (HST), the distance between the user equipment (UE) and the base station (BS) can change rapidly. This affects signal propagation time, and the UE transmit (Tx) timing needs to be adjusted so that uplink transmissions from different UEs arrive at the BS without collision and so that the BS can track each UE’s uplink (UL) timing. UE timing may change for the following scenarios:

  - UE movement. As shown in Figure 7 (a). The received timing changes due to UE motion or a frequency offset between the BS and UE, and the UE must maintain the timing difference between the receive and transmit timing with respect to the system timing.

  - Path change. As shown in Figure 7 (b). The dominant receive path may be attenuated or disappear due to channel variations, and the UE needs to re-synchronize its transmit timing to another receive path.

  - NTN Satellite movement. With the introduction of Non-Terrestrial Networks (NTN) in 3GPP Release 17, UEs implement timing/frequency pre-compensation, based on the provided Ephemeris and UE location, to project and adjust UE’s transmission timing.

  - Residual oscillator drift. Even without movement, small mismatches in clock frequencies between UE and BS or between UE TX and UE RX can accumulate, requiring periodic timing corrections.

  - UE-based TA for LTM cell switch. UE-based TA for LTM cell switch. When the UE switches cells, the distance to the new cell’s base station may be different, so the timing advance (TA) must be updated to maintain uplink synchronization.

(a) UE movement					(b) path change

Figure 7 Scenarios for UE timing adjustment.

In 5G, two opposite-direction methods are used to manage UE timing adjustment, as illustrated in Figure 8:

Method 1: UE Gradual timing adjustment

- This legacy mechanism has present since 3G and was retained through 4G and 5G (Rel-15), where the UE monitors DL timing changes, which reflect the propagation delay changes, and shifts the UL transmit timing in the same direction of DL timing change; thus, if the DL timing is delayed, the UE further delays its UL, and the BS observes roughly double timing changes; to realign, Timing Advance Commands (TAC) must advance the UL timing (i.e., in the opposite direction of the DL change), which adds control overhead, and the adjustment rate is limited to 0.18–0.9 μs/s per TS 38.133 to prevent UL reception failures at the BS, making the mechanism potentially insufficient in fast timing-change scenarios such as NTN or HST.

Method 2: UE pre-compensation

- This mechanism whereby the UE proactively adjusts its UL transmit timing in the opposite direction of DL timing change. For example, if the UE detects the DL timing is delayed, it advances its UL timing by the same amount, preventing misalignment before the BS detects it.

- UE pre-compensation has been adopted in R17 NTN, R18 FR2 HST/POS and is typically applied when timing changes are large (e.g., > CP/4) or when TCI changes occur; in NTN, the adjustment is based on satellite/BS position to project the correct timing at satellite, though the UE can also derive it from the detected DL timing changes for TN network.

- Observation 28: In 5G, two opposite direction methods are used to adjust UE UL timing: gradual adjustment (Method 1), which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

(a) UE movement				(b) path change

Figure 8 Methods for UE timing adjustment.

In 5G systems, gradual timing adjustment is the baseline, with mandatory test cases in place since Release 15. Specifically, as defined in TS 38.133 A.6.4.1 (shown below), step 3 imposes a DL timing change of Δtp, as shown in Figure 8, and step 4 expects the UE to adjust its UL toward (2Δtp±Te), but as shown in Figure 8, if the UE performs timing pre-compensation using Method 2, the arrived timing at the NB will remains transparent to the NB yet differ from what test system expects, causing the UE to fail.

Observation 29: With propagation delay changes, UE pre-compensation (Method 2) leaves NB observed UL timing unchanged, yet it differs from what test system expects in the 5G R15 mandatory test defined in TS 38.133 A.6.4.1, so the UE fails the test.

TS 38.133. A.6.4.1.1.2	Test requirements

..

3)	The test system shall adjust the timing of the DL path by values given in table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna.  Skip this step for test 2 with DRX configured.

In 6G, we propose studying the feasibility of adopting UE pre-compensation as the baseline approach, specifically replacing gradual timing adjustment for handling UE mobility and path changes, while retaining the Timing Advance command to correct the UE’s initial UL timing.

Proposal 31: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

## MRTD

In legacy systems, the maximum receive timing difference (MRTD) is defined by 3GPP as the maximum relative timing difference between serving carriers that a UE can handle. According to 3GPP, MRTD consists of two components: time alignment error (TAE) and RF propagation delay difference (∆Tprop), such that MRTD = TAE + ∆Tprop. The requirements for MRTD and TAE vary depending on the configuration. For intra-band operation, MRTD is set to 3 µs and TAE to 65 ns; for inter-band operation, MRTD is 33 µs and TAE is 260 ns. It should be noted that higher MRTD values can lead to increased complexity in UE implementation.

Given the higher performance expectations for 6GR systems, it is important to optimize TAE so that UE complexity does not increase compared to 5G NR.

Observation 30: Higher MRTD values can lead to increased complexity in UE implementation.

Proposal 32: RAN4 to seek optimisation of TAE for MRTD to a level maintaining the UE complexity lower than that for 5G systems.

# Summary

Our observations and proposals on the 6G RRM requirements are summarized as below:

// MG framework

Conventional measurement gaps:

Observation 1: Measurement gaps are typically responsible for a 15% interruption of data transmission and reception (e.g. MGL = 6ms, MGRP = 40ms), which impact user experience in terms of throughput and latency.

Proposal 1: Investigate methods to reduce the use of measurement gaps in 6G systems.

Observation 2: In LTE/NR, a UE might report a need for a measurement gap when RF retuning is necessary to measure frequency carriers outside its operating carriers.

Observation 3: In LTE/NR, a UE might report a need for a measurement gap due to a desense (MSD) issue between the target band, containing the SSB to be measured, and the serving bands.

Proposal 2: Identify and address the reasons behind measurement gaps in previous generations of mobile systems and how to handle these issues in 6G systems.

Gapless solutions

Issue 1: Available RF chain:

Observation 4: In 5G NR, during measurement gaps length, a UE interrupts uplink and downlink data transmission and reception across all serving cell carriers to allow measurement of a target SSB of a target cell.

Proposal 3: Study the possibility of limiting the interruption or measurement gap to specific carriers (per-CC gap) without affecting the remaining serving cell carriers.

Observation 5: From a UE implementation perspective, whether none or specific number of carriers should be interrupted depends on the availability of an idle RF chain to perform measurements on the target cell.

Proposal 4: When defining measurement gap/interruption requirements and reporting signalling, different UE behaviours should be specified based on the availability of an idle RF chain.

Issue 2: UE assisted measurement gap

Observation 6: UE-assisted measurement gaps allow measurement gaps to be applied only to specific carriers, improving efficiency and reducing unnecessary interruptions.

Proposal 5: RAN4 to study and consider UE assisted measurement gap as part of the initial release of 6GR.

Issue 3: RF retuning time:

Observation 7: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 8: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 6: In defining interruption, including RF retuning, RAN4 shall consider symbol-level granularity rather than slot-level granularity.

Issue 4: Visible/invisible interruption

Proposal 7: RAN4 selects best possible location to minimize PDCCH, HARQ ACK and data loss.

Proposal 8: For 6GR gapless solutions, RAN4 should further study whether interruptions should be specified as visible or invisible to the network.

Issue 5: UE self-interference caused by IMD/harmonics

Observation 9: Direct Overlap, harmonics may directly overlap with the SSB reference signals. For instance, uplink harmonics from band n3 can overlap with an SSB in band n77.

Proposal 9: RAN4 to further study the impact due to UE self-interference caused by IMD/harmonics and to find solution to limit the gap reporting for such scenarios.

Observation 10: In 5G NR, gap reporting capability is based on frequency bands and is independent of the SSB location within the band, also, the SSBs within a band may be affected by IMD and harmonic products present in that band, however, not all SSBs within the band are necessarily impacted.

Proposal 10: Introduce finer gap reporting granularity, i.e. smaller than per target band, to reduce unnecessary measurement gaps.

Issue 6: Non-colliding multiple GAPs

Observation 11: From a system operation perspective, applying different measurement gap configurations for different carriers can reduce unnecessary measurement gap occasions and increase network configuration flexibility.

Observation 12: In 5G NR, concurrent measurement gaps were introduced to improve network configuration flexibility and support a wider range of use cases. However, current RRM requirements limit the number of concurrent gaps to a maximum of two due to the collision among the configured gaps.

Proposal 11: Investigate the introduction of non-colliding multiple concurrent measurement gaps in 6G systems from the initial release.

Issue 7: Measurement gaps and gapless spec structure

Observation 13: In 5G NR, RRM spec implementation for measurement is rather complex and has too much repetition between different clauses, such as inter-f and intra-f.

Proposal 12: RAN4 to study the consideration of spec writing that measurement categorization is written based on measurement mechanism, i.e. gaps/NCSG/interruption/gapless, such as:

- Clause x-1: Measurements within gaps (including CSSF and delay)

- Clause x-2: Measurements outside gaps (including CSSF and delay)

  - Clause x-2a: Measurements with interruption/NCSG

  - Clause x-2b: Measurements without interruptions

Fast RF retuning/interruption-free

Observation 14: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 15: In 5G NR, interruptions due to RF retuning were defined with slot-level granularity and without differentiating between control and data channels.

Proposal 13: In defining interruption, including RF retuning, RAN4 shall consider symbol-level granularity rather than slot-level granularity.

Proposal 14: RAN4 selects best possible location to minimize PDCCH, HARQ ACK and data loss.

// RRM framework

Measurement capability

Observation 16: The UE capability of number of cells is underutilized in 5G.

Proposal 15: Study the appropriate measurement capability for number of cells in 6G. The number of detected cells in FR1 is <=4 with 90% probability and <=5 with 97% probability.

Measurement delay overhead

Observation 17: The number of searchers has been thoroughly studied and optimised in latest release in 5G NR, also, it is correlated to the UE architecture, which is discussed in other WGs.

Observation 18: Reducing the Rx beam sweeping factor in FR2 can directly impact the UE performance.

Proposal 16: Deprioritize the study of Rx beam sweeping factor in FR2 and the number of searchers for 6GR SI.

Unified measurements

Observation 19: The RRM measurement framework in 5G NR is highly sophisticated, offering tailored requirements and operational conditions for a variety of use cases.

Proposal 17: It is recommended that RAN4 deprioritize the study of a unified measurement framework at this stage and instead focus on thoroughly identifying and understanding the actual challenges and limitations within the current 5G NR system.

// Mobility and HO interruption reduction

Proposal 18: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

Proposal 19: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Proposal 20: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

Proposal 21: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Observation 20: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 21: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Proposal 22: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 23: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

Proposal 24: Better to have unified handover solution for both TN and NTN if possible.

Proposal 25: Discuss whether and how to let NW know when interruption starts. The following options can be used as a starting point.

- Option 1: Determine the location of interruption based on the defined minimum requirements in RAN4.

- Option 2: Introduce new signalings to have common understanding on when interruption starts.

- Option 3: Define the earliest location of interruption in RAN4.

- Option 4: Others.

Proposal 26: Support UE initiated early RRC decoding and early T/F tracking in 6G.

// Scell activation, and spectrum aggregation

Cell with multiple carrier

Observation 22: The interruption time to add a carrier in a multi-carrier cell can be kept below the current BWP switch delay and further reduced by decoupling delay from interruption.

Observation 23: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 24: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 27: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Observation 25: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 28: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Observation 26: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 29: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

// Other PHY signal/channel/procedure related RRM

UE Tx timing

Observation 27: Relaxing PRACH timing requirements would accommodate initial timing errors within CP, allow timing correction after initial access, and enable higher SCS. Especially in NTN scenarios, where impairments such as GNSS/UE positioning inaccuracies and satellite position errors can be corrected after initial access.

Proposal 30: Study PRACH (if introduced) specific timing accuracy requirement.

Observation 28: In 5G, two opposite direction methods are used to adjust UE UL timing: gradual adjustment (Method 1), which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

Observation 29: With propagation delay changes, UE pre-compensation (Method 2) leaves NB observed UL timing unchanged, yet it differs from what test system expects in the 5G R15 mandatory test defined in TS 38.133 A.6.4.1, so the UE fails the test.

Proposal 31: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

MRTD

Observation 30: Higher MRTD values can lead to increased complexity in UE implementation.

Proposal 32: RAN4 to seek optimisation of TAE for MRTD to a level maintaining the UE complexity lower than that for 5G systems.

# References

- RP-251881, “Study on 6G Radio”, NTT DOCOMO (Moderator)

- R4-2514644, “WF on 6G RRM”, Apple, Prague, Czech Republic, October, 2025.