3GPP TSG-RAN WG4 #117	R4-2520041

Dallas, US, November 17-21, 2025

Agenda item:	8.6		RRM

Source:	Qualcomm Incorporated

Title:	Further comments on radio resource management aspects of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on the 6GR core and performance requirements. In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In RAN4 #116bis the work on 6G RRM was started. Meeting minutes of the discussion and the way forward are provided in [4], [5].

In this contribution, we discuss our views on the goals of the RAN4-driven radio resource management aspects of the 6G study item including enhancements for the requirement framework.

# Views on the 6G Study Item for RRM

In RAN4 #116bis the work on radio resource management aspects for the 6G study item started and a way forward was agreed in [5]. Several topics were discussed in RAN4 #116bis how to structure the discussions on RRM. So far, the two high level categories measurement gap (MG) and interruption and RRM framework were agreed, and several other categories were proposed. According to the way forward in [5] in RAN4 #117 further discussion on the categories and on the scope of each category will take place.

In the subsequent sections we provide our view on the scope and the content of each category.

## Measurement Gap (MG) and Interruption

In RAN4 #116bis regarding the scope of the category measurement gap and interruption has been achieved.

Agreement in main session (Tue):

Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

- Interruption enhancement due to gap-less measurement will be part of the MG discussion.

It has been discussed but not formally been captured in the agreement that interruption enhancements are not only related to measurement gaps but the discussions on interruptions should also aim to enhance interruptions for scenarios not related to measurement gaps. Hence, we propose to split the category measurement gap and interruption in the two subsections measurement gap and interruptions.

Observation 1: It has been discussed but not formally been captured in the agreement that interruption enhancements are not only related to measurement gaps.

Proposal 1: The discussions on measurement gaps and interruptions should also aim to enhance interruptions for scenarios not related to measurement gaps. We propose to split the category measurement gap and interruption in the two subsections measurement gap and interruptions.

### Measurement Gap

During the last RAN4 meeting, several companies proposed study directions for measurement gaps. These proposals can be broadly categorized into two foundational goals:

- Measurement Gap Configuration Optimization

- Measurement Gap Interruption Mitigation

We recommend that RAN4 adopt these two high-level goals as the baseline for the 6GR study item phase.

Proposal 2: RAN4 to follow the following high-level goals for the 6GR study item phase:

- Goal 1: Study mechanisms to minimize and optimize measurement gap configurations

- Goal 2: Study mechanisms to reduce or eliminate measurement gap interruptions

Measurement gap configuration optimization

As we know, NR has introduced a variety of gap types across releases—such as per-UE/per-FR gaps, pre-configured MGs, NCSG, autonomous gaps, and positioning gaps—many of which can operate concurrently. This proliferation increases complexity on both the UE and network sides, often resulting in low adoption of certain features (e.g., NCSG), thereby undermining the value of standardization efforts.

Observation 2: The introduction of multiple gap types and their concurrent operation increase system complexity, contributing to low adoption and underutilization of MG-related features.

Thus, we believe the upcoming 6GR should undertake a comprehensive study of the measurement gap (MG) framework to identify inefficiencies—both technical and operational—and explore solutions that strive to minimize such fragmentation of MGs, by possibly having a unified MG framework.

Proposal 3: RAN4 to study how to specify a unified and flexible approach to MG framework that limits fragmentation and simplifies implementation across vendors and deployments.

Additionally, several companies noted that only a small subset of MG patterns (e.g., MGL, MGRP) are actively used in networks. This suggests a need to streamline configuration options, including measurement gap patterns to better align with actual measurement needs and likelihood of deployment.

Proposal 4: RAN4 should evaluate how to reduce the MG patterns (e.g., MGL, MGRP), ensuring they reflect real-world deployment requirements and usage patterns.

Measurement gap Interruption mitigation

On the measurement gap interruption aspects, we observed that current NR implementations exhibit inefficiencies due to long, static gap configurations that are not always aligned with UE-specific needs. For example, measurement gap lengths (MGLs) are designed to accommodate various SSB durations, considering the number of SSBs in bursts and their SCS, but measurement objects (MOs) may be dynamically updated (added/removed). Networks often lack the resources to reconfigure MGs for each UE in response to these changes. As a result, in many deployments, MG configurations remain static and do not reflect the actual measurement requirements of the UE, leading to wasted resources and reduced scheduling flexibility.

We think RAN4 should prioritize the investigation of mechanisms such as NCSG-like approaches that support interruption-free measurements. Early infrastructure support for such mechanisms is crucial to ensure timely deployment and avoid the risk of these features being sidelined.

Observation 3: Existing gap configurations are often long and predefined, resulting in underutilization when measurements are not actively needed.

Observation 4: UEs currently have limited ability to influence gap configurations, resulting in suboptimal measurement scheduling and resource usage.

Proposal 5: RAN4 should study the overall measurement gap framework in 6GR and identify all inefficiencies—technical and operational—that hinder optimal gap usage.

Proposal 6: RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR.

In addition, we propose that RAN4 studies mechanisms to support UE-initiated gap configurations. As networks often do not dynamically update MG configurations in response to changes in measurement objects, this leads to inefficient use of gaps. UEs, however, have the most accurate view of their mobility conditions and can optimize their measurement scheduling to minimize the number and duration of gaps required. Therefore, it is logical to allow UEs to indicate preferred MG configurations that balance mobility needs with throughput considerations.

Proposal 7: RAN4 should study how to enable UEs to indicate preferred MG configurations, allowing for more adaptive and efficient measurement scheduling

### Interruption

5G NR allows interruptions for EN-DC, SA, NE-DC and NR-DC for various scenarios like PSCell/SCell addition/release, SCell activation/deactivation, during measurements on SCC and many other cases. In many cases, interruptions on some or all active serving cells are allowed in case certain actions happen on other serving cells. For example, in case that a deactivated SCell is activated, an interruption of the SMTC duration plus an additional slot in any active serving cell is allowed that is in the same FR1 band as the SCell to be activated.

It is obvious that this can lead to inefficiencies for the network if the network simply follows in its scheduling decisions the RAN4 interruption requirements or in case that the network would be able to honor the true interruption duration in the device, but the signaling is not supported. For several of the interruption scenarios, the true interruptions may be shorter than the existing requirements or in some cases, no interruptions may be needed at all. Also, over time the UE implementation may have improved, and interruption durations became shorter or even fully disappeared.

Observation 5: 5G NR allows for many scenarios interruptions on the active serving cells that impact the efficiency of the network significantly.

Observation 6: In specific scenarios, the true interruption of state-of-the-art UEs may be shorter than the minimal requirement in RAN4. In some scenarios, even no interruption may be needed.

Hence, it is important that RAN4 re-investigates all scenarios with interruptions whether the requirement can be relaxed or whether interruption is still needed.

Proposal 8: RAN4 should study all existing interruption scenarios in 5G NR, whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR.

The minimal requirement of the duration of the interruption depends on the scenario. For example, for intra-band carrier aggregation for SCell addition/release, SCell activation/deactivation and SCell measurements the interruption length depends on the duration TSMTC duration of the SMTC window. In all these different scenarios very different call flows apply in the UE. Each of those scenarios should be analyzed separately. As a starting point, RAN4 could analyze interruptions for activation and deactivation of SCells and interruptions for measurements on deactivated SCells.

Proposal 9: As a starting point, RAN4 could analyze interruptions for activation and deactivation of SCells and interruptions for measurements on deactivated SCells.

#### RF Tune Time

In the context of interruption also the RF tune time for measurement gaps should be mentioned. In the previous section, the necessity of enhancing the gap framework was discussed. Figure 2-1 shows the general definition of a gap that includes the measurement duration and a switching time at the beginning and the end of a gap that is free of measurements. The allowed switching time in TS 38.133 is 0.5 ms for FR1 and 0.25 ms for FR2. Depending on the subcarrier spacing, this duration is half a slot for 15 kHz, one slot for 30 kHz and two slots for 120 kHz.

Figure 2-1: Gap Duration and Switching Time

Like interruptions, the switching time causes overhead and reduces network efficiency. For 30 kHz and 120 kHz, the existing minimal requirement can consume one or two full slots. In this case, overhead can only be reduced if the true switching time is lower than the minimal requirement. However, for 15 kHz, the minimal requirement for the switching time is just half a slot. In this case it may be possible to use the second half of the slot for other purposes, e.g., data transmission on the carrier the RF is tuned from. The efficient use of the 0.5 ms half-slot should be examined.

Observation 5: For 30 kHz and 120 kHz, the minimal requirement for the RF tune time of a measurement gap consumes one or two full slots.

Observation 6: For 15 kHz, the minimal requirement for the RF tune time of a measurement gap consumes half a slot. The latter half of the slot is unused for RF switching or data transmission, resulting in wasted capacity.

Proposal 10: RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission.

Similar analysis applies for 30 kHz subcarrier spacing in case that the RF tune time is less than a slot.

## RRM Framework: Measurement Capability, Delay, Overhead, Accuracy, Unified Measurement

### RRC and MAC Processing Timelines

In our view the work on the RRM framework should also include investigating the timelines for RRC and MAC processing. Typical assumption for MAC CE in 5G NR is that the MAC CE command applied 3Nslotsubframe,μ slots after the slot when the transmits the HARQ-ACK information for the PDSCH carrying the MAC C command. That means independent on the content of the MAC CE command, the processing timeline of the MAC CE is always 3 ms in 5G NR. In LTE, however, the MAC CE processing timeline is dependent on the content of the MAC CE, e.g., 6 subframes for timing advance and 8 subframes for SCell activation.

Observation 7: Independent on the content of the MAC CE command, the processing timeline of the MAC CE is always 3 ms in 5G NR. In LTE, however, the MAC CE processing timeline is dependent on the content of the MAC CE

However, depending on whether a MAC CE command involves baseband only entities or also RF, whether it only applies for a single carrier or multiple carriers, etc., the activation time of a MAC CE command may be different. Generally, the activation time depends on the content. To define more realistic timelines, RAN4 should study the dependence of the MAC CE processing timeline on the content of the MAC CE command, like what was done in LTE.

Observation 8: Depending on whether a MAC CE command involves baseband only entities or also RF, whether it only applies for a single carrier or multiple carriers, etc., the activation time of a MAC CE command may be different.

Proposal 11: RAN4 should study the dependence of the MAC CE processing timeline on the content of the MAC CE command.

A similar study is also necessary for RRM requirements that involve the RRC processing timeline. In the existing RRM requirements (such as RRC-based BWP switch and RRC-based direct SCell activation), it is assumed that lower-layer (physical-layer) operations begin upon transmission of the corresponding RRCReconfigurationComplete message, whose timeline is defined in the RRC specification. The problem arises because this timeline is constant, regardless of whether the RRC message reconfigures only those parameters directly related to the requirement or activates multiple SCells concurrently. In real implementations, these factors can affect the timeline.

Observation 9: Current RRM requirements involving the RRC processing timeline do not account for the fact that a single RRC message can reconfigure parameters beyond those considered in the respective RRM requirements.

Given the emerging demand for a unified signaling framework supporting measurement, mobility, and other procedures, the effective RRC processing delay is expected to heavily depend on the specific content of the message. This may result in significant deviations from the 5G timeline. RAN4 should study how to accommodate this variability when defining RRC processing and/or application timelines

Proposal 12: RAN4 should study how to accommodate real UE implementation timelines based on the content of the RRC message when defining RRM requirements. For example, RRC-based BWP switch and RRC-based direct SCell activation.

### RX Beam Sweeping Factor

RAN4 has assumed the RX beam sweep factor of eight in many FR2 measurement periods of Rel-15. RAN4 assumed that UE needs to sweep eight RX beams in FR2 to cover angle of arrivals in relevant azimuth and elevation domain and detect or measure an unknown reference signal. This scaling factor of 8 significantly increases many evaluation periods in FR2. This long evaluation period slows down the mobility procedures significantly.

Observation 10: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced beam sweeping factors for serving and neighbor cells. RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period requirements of 6G.

Observation 11: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells.

Proposal 12: RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period related requirements in 6G.

### Carrier Specific Scaling Factor

5G Rel-15 requirements were defined assuming that UE can measure measurement objects (MO) of up to two frequency layers simultaneously without measurement gap in CA/DC scenarios. Rel-19 introduced a new UE capability that allows UE to measure MOs of three frequency layers simultaneous without measurement gap in CA/DC scenarios. This new carrier specific scaling factor (CSSF) related Rel-19 capability can reduce measurement delay considerably for 5G UEs. Ideally, basic 6G UEs should be assumed to support this capability. However, UE’s ability to simultaneously measure MOs of multiple frequency layers depends on the structure of SSB. RAN4 can study simultaneous measurement of MOs across multiple layers only after RAN1 makes progress regarding the SSB structure of 6G.

Observation 12: 5G Rel-15 requirements were defined assuming that UE can measure measurement objects (MO) of up to two frequency layers simultaneously without measurement gap in CA/DC scenarios.

Observation 13: Rel-19 introduced a new UE capability that allows UE to measure MOs of three frequency layers simultaneously without measurement gap in CA/DC scenarios.

Observation 14: UE’s ability to simultaneously measure MOs of multiple frequency layers depends on the structure of SSB.

Proposal 13: RAN4 should study simultaneous measurement of MOs across multiple frequency layers without measurement gap in 6G only after RAN1 makes progress regarding the 6G SSB structure.

### Identification, Measurement and Reporting Delay

Setting aside the FR2 RX beam sweeping factor and carrier specific scaling factor, some other assumptions of evaluation periods in 5G might be more relaxed than necessary. Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins. One contributing could be that 5G spec might have given UE more samples for identification and measurement than what UE needs in some scenarios. Hence, 6G should study if and how to reduce these identification, measurement and reporting periods.

Observation 15: Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins.

Proposal 14: RAN4 should evaluate identification/measurement/reporting delay related requirements, in general, and study if and how to reduce them in 6G.

## Mobility related RRM

Unified measurement and mobility

There have been multiple evolutions to RRC connected UE mobility in 5G as shown below:

- RRC based baseline handover in Rel-15

- Conditional handover in Rel-16

- Dual Active Protocol Stack (DAPS) in Rel-16

- Inter-Cell Beam Management (ICBM) in Rel-17

- Lower-layer Triggered Mobility (LTM) in Rel_18

- Enhanced LTM in Rel-19

To the best of our understanding at the time of writing, Conditional Handover and DAPS have not yet been widely deployed in commercial networks, each for different reasons. In contrast, we’ve observed significant traction from infrastructure vendors around LTM. Moreover, based on perspectives shared by companies in 3GPP and other workshops, it is evident that LTM is being positioned as the baseline framework for unified measurement and mobility in 6G.

That said, we should acknowledge that the 5G LTM RRM requirement definition has made UE implementation highly challenging, potentially limiting feasibility of the feature to specific scenarios in practice. Below are the identified UE implementation challenges and practical use case limitations:

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Observation 16: Although LTM is being positioned as the baseline framework for unified measurement and mobility in 6G, as reflected in 3GPP discussions and industry workshops, its practicality remains uncertain due to the following anticipated implementation complexity and constraints related to deployment and applicability.

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Proposal 15: If a unified measurement and mobility framework for 6G is defined based on the 5G LTM feature, RAN4 should re-evaluate the benefits of the feature, thoroughly analyze the implementation challenges arising from its complexity and associated RRM requirements, and investigate how to address the identified issues to the extent that the feature can be implemented with reasonable complexity.

Excessive latency in handover requirements with limited relevance to field deployments

There are several instances where existing mobility requirements lack demonstrable effectiveness and field relevance. One notable example involves RRM-spec-defined “unknown” cells, where the resulting mobility requirements often lead to excessively long interruption durations and delays. These requirements are not representative of realistic deployment scenarios and pose questions about the purpose of defining such relaxed requirements.

Observation 17: The 5G RRM specification contains several instances where the defined mobility requirements result in excessively long interruption durations and delays, for example, the “unknown” cell handover scenario. These requirements do not reflect realistic deployment conditions and raise questions about the rationale behind specifying such relaxed requirements.

Additionally, field observations indicate that “inter-frequency” handovers predominantly occur in specific configurations, most commonly between FDD and TDD bands. Despite this, current inter-frequency mobility requirements are defined in a scenario-agnostic manner, without anchoring to concrete use cases. This generalized approach complicates optimization efforts and may result in requirements that are either overly conservative or misaligned with practical needs.

Observation 18: Current inter-frequency mobility requirements are defined in a scenario-agnostic manner, despite field evidence showing that such handovers predominantly occur in specific configurations (e.g., FDD↔TDD). This generalized approach complicates optimization and may lead to overly conservative or misaligned requirements.

Defining mobility requirements that must apply uniformly across all possible scenarios can dilute their technical precision and operational applicability. Moreover, it may adversely affect the readability of the RAN4 specification by introducing excessive conditional logic, such as numerous ‘if’, ‘else if’, and ‘else’ statements. Unless RAN4 explicitly agrees to differentiate requirements based on scenario types (which should be avoided unless strongly justified), it is more practical and effective to focus on scenarios with clear field relevance and deployment evidence.

Observation 19: Defining mobility requirements uniformly across all scenarios can dilute technical precision and operational relevance. It also risks reducing the readability of the RAN4 specification due to excessive conditional logic. A more practical approach would be to focus on scenarios with clear field relevance and deployment evidence, unless strong justification exists for broader applicability.

Proposal 16: 6G mobility requirements should be constrained to scenarios where the resulting handover latencies or interruption durations are technically meaningful and operationally relevant. Requirements that lead to excessively large values should be avoided.

Target Handover Latency and Interruption Requirements

Field data analysis from 5G networks indicates that user data forwarding from the source cell to the target cell is not always as fast as the mobile UE’s handover latency. Without enhancements on the network side, further optimization of UE handover latency may have limited practical value, as the gating factor from the end-user perspective is the overall data path delay — not just the UE’s mobility performance.

In 6G, it is important to study and account for network-side data forwarding latency when defining achievable handover latency targets. This approach would allow RAN4 to avoid unnecessarily aggressive latency and interruption requirements, thereby reducing UE implementation burden and avoiding optimizations that may not yield meaningful field benefits.

Proposal 17: RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness.

Our views on Other Topics

R4-2514644, Way forward from RAN4#116bis - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

There are multiple proposals categorized under ‘Mobility-related RRM,’ as shown in the table above. Among them, the following proposals appear to be either heavily dependent on design details from other working groups or more suitable for the work item phase.

- Solutions for longer SSB periodicity in mobility

- Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility

Proposal 18: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- Solutions for longer SSB periodicity in mobility

- Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility

## RRM related Energy Efficiency

Several companies submitted proposals during the last RAN4 meeting on UE and BS energy efficiency and power-saving mechanisms. While these are important topics for 6GR, we recommend that RAN4 defer detailed study of these items until there is clearer direction and progress from RAN1 and/or RAN2.

Proposal 19: Suggest postponing all discussions on energy efficiency related RRM discussions till RAN1and RAN2 have made some progress.

## Spectrum Aggregation related RRM

Evolution of SCell/SCG activation

Several enhancements related to SCell and SCG activation have been introduced through RAN1, RAN2, and RAN4 efforts, as outlined below:

- Rel-15:

  - MAC-based single SCell activation

- Rel-16:

  - MAC-based multiple SCell activation

  - RRC based SCell activation (TCI state activation upon the SCell activation was introduced in Rel-17)

- Rel-17:

  - PUCCH SCell activation

  - Aperiodic temporary TRS based SCell activation latency enhancement (limited to known SCell)

  - SCG activation

  - Conditional PSCell addition

- Rel-18:

  - Event-like L3 report based unknown SCell activation latency enhancement

  - SSB-less inter-band NES SCell activation

- Rel-19:

  - Idle/Inactive mode L3 measurement based SCell activation latency enhancement

  - OD-SSB based NES SCell activation

  - LB-LB CA

Since Rel-15, each release has introduced multiple SCell/SCG-related enhancement features, as listed above. As a result, the current SCell activation requirements have become increasingly complex. However, almost all enhancement features — except for MAC-based single and multiple SCell activation — have not been adopted in commercial networks. In other words, only the features that UEs are mandated to support have seen successful field deployment. Despite this near-zero adoption rate, there were attempts to further optimize SCell activation latency for Rel-20. This disconnect between specification and field relevance should be seriously considered as a lesson learned when defining SCell-related features for 6G.

Observation 20: Despite multiple SCell/SCG enhancements introduced since Rel-15, only mandatory MAC-based activation features have seen commercial adoption. This highlights a gap between specification complexity and field relevance.

Proposal 20: For 6G carrier aggregation features, RAN4 should investigate which SCell activation features have been commercially deployed or are planned for deployment and prioritize those based on proven deployment evidence to avoid unnecessary complexity and ensure practical impact.

RRM requirements

RRM requirements associated with SCell activation are defined for the following aspects:

- Interruption: verified through missing HARQ feedback on active serving cells:

  - Upon deactivated SCell measurements

  - During SCell activation and deactivation

- Latency: verified through the first non-zero CQI report for the to-be-activated SCell

  - Latency varies significantly depending on multiple factors, including known vs. unknown SCell, band relationships between serving cells, measurement cycle of the SCell, etc.

In 5G, interruption requirements during deactivated SCell measurement and SCell activation are overly conservative. Given that UEs typically operate under high traffic demand when SCell activation is triggered, excessively long interruption durations across all serving cells may undermine the intended performance benefits.

Observation 21: 5G interruption requirements during SCell activation result in blanking scheduling opportunities across all serving cells for extended periods, which may not reflect practical traffic conditions.

Proposal 21: In 6G, interruption requirements during deactivated SCell measurement and SCell activation should be defined to align with state-of-the-art UE implementations. For example, the interruption duration for intra-band CA in 5G is excessively long and may not reflect practical deployment needs.

SCell activation latency requirements in 5G are overly complex and heavily dependent on the SCell’s state — particularly whether it is ‘known’ or ‘unknown’ — along with several other factors such as measurement and reporting periods, intra-/inter-band configurations, SSB-less and SMTC-less setups, and co-location conditions like RTD/RPD-based same-Tx-Beam.

However, based on real product analysis, these latency requirements do not consistently vary across all defined factors. In many cases, the total activation latency specified in RAN4 is significantly longer than what is observed in real-world implementations, offering limited practical effectiveness. As a result, SCell deactivation decisions may be overly conservative due to concerns about long reactivation delays, leading to increased UE power consumption.

One example is that the RAN4 requirement assumes the UE uses the SSB of the to-be-activated SCell, which aligns in the time domain with all other SSBs of serving cells in the same band — a condition not necessarily required in actual implementations. In other words, although AGC operation may cause interruptions to other serving cells, AGC does not necessarily need to occur only when the SSBs from all serving cells in the same band are time-aligned.

Another example is the assumption that the UE requires one SSB each for coarse AGC, fine AGC, coarse time synchronization, and fine time/frequency synchronization. Additionally, RAN4 assumes five SSB samples are needed for pathloss measurement during PUCCH-based SCell activation.

These examples illustrate how RAN4 requirements diverge from real-world implementations and contribute to unnecessarily large SCell activation latency on paper.

On the other hand, the RAN4 requirement has simplified SCell addition in a way that activation latency does not scale proportionally with the number of SCells added or activated via RRC. This can make UE implementation challenging, especially in scenarios where multiple SCells are added simultaneously. For instance, when several SCells are added and activated, physical layer activation may not begin until the UE brings up the baseband and RF chains for all cells, introducing delays.

Observation 22: RAN4’s SCell activation latency requirements in 5G are overly complex and misaligned with real-world implementations. Assumptions such as strict SSB alignment and multi-step synchronization inflate latency on paper, leading to conservative deactivation behavior and increased UE power consumption.

Proposal 22: For 6G carrier aggregation, RAN4 should re-evaluate the assumptions made in the 5G SCell activation requirement definition, based on state-of-the-art UE implementations, and investigate whether and to what extent SCell activation latency modeling can be simplified to better align requirements with practical UE behavior and implementation realities.

Another issue overlooked in the SCell activation requirement definition was RRC-based SCell activation. The existing RRC-based SCell activation requirements inherited those for MAC-CE-based activation. The key difference is that lower-layer (physical-layer) operations for RRC-based activation begin only after the corresponding RRCReconfigurationComplete message transmission, whose timeline is defined in the RRC specification.

The problem arises because this timeline is fixed, regardless of how many SCells are activated concurrently upon the RRC message or whether the RRC message reconfigures the current serving cells’ configuration. In real implementations, those can affect the timeline due to factors such as RF script building and programming.

Observation 23: The current RRM requirement for RRC-based SCell activation does not account for the number of SCells to be added/activated and serving cells’ RRC reconfiguration upon the RRC message in the RRC processing timeline.

Proposal 23: RAN4 should study how to accommodate real UE implementation constraints in the RRC processing timeline. For example:

- Single SCell addition/activation vs. multiple SCells addition/activation

- SCell addition/activation only vs. SCell addition/activation combined with serving cell configuration updates triggered by the RRC message.

Our views on Other Topics

R4-2514644, Way forward from RAN4#116bis - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics if there are sufficient conclusions from other WGs and RAN4 RF session - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics once there are sufficient conclusions from other WGs and RAN4 RF session

There are multiple proposals categorized under ‘2.5	Spectrum Aggregation related RRM,’ as shown in the table above. Among them, the following proposals appear to be either heavily dependent on design details from other working groups or more suitable for the work item phase.

- RRM conditions and requirements for Single Cell Multi-Carriers

- RRM impacts of DL and UL decoupling

- Carrier switch enhancements for UL and DL

- RRM impacts of realistic SCS for spectrum

- relaxation of the requirement on timing alignment between carriers

- RRM for MRSS

Proposal 24: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- RRM conditions and requirements for Single Cell Multi-Carriers

- RRM impacts of DL and UL decoupling

- Carrier switch enhancements for UL and DL

- RRM impacts of realistic SCS for spectrum

- relaxation of the requirement on timing alignment between carriers

- RRM for MRSS

## MIMO and mTRP Operation related RRM

In our view the topic of MIMO and mTRP operation related RRM depends heavily on the sections 11.8 MIMO operation and 11.9 physical layer control, data scheduling and HARQ in the RAN1 agenda. According to RAN1 time schedule all these sections start in the February meeting. In addition, the RAN4 agenda foresees only 7 TUs including the February meeting. Hence, we propose to delay all discussions on this topic till RAN1 has started the work and made some progress.

Observation 24: The RAN1 work on the relevant sections in the RAN1 agenda for MIMO and mTRP operation related RRM will not start before February.

Proposal 25: We propose to delay all discussions on PHY signal/channel/procedure related RRM till RAN1 has started the work and made some progress.

## NTN related RRM

We understand that NTN should be included in Day 1 of 6G use cases to better harmonize TN and NTN specification design, and we have already observed significant momentum driven by the satellite community. We also note that a list of detailed items is being compiled by interested companies to thoroughly consider more seamless 6G services, including TN-NTN mobility, multi-orbit-based services, and more. However, since the overall framework should first be established by other working groups, we believe RAN4 should wait for their progress in this area.

Proposal 26: We propose to delay all discussions on NTN related RRM till other working groups have started the work and made some progress.

## Initial Access related RRM

A couple of companies proposed to study initial access related RRM requirements during the last meeting, e.g., whether to define cell selection delay requirement during initial access. In general, initial access related RRM procedures depend heavily on RAN1 design. For example, cell selection delay requirements depend on the designed SSB for 6G, i.e., the periodicity and structure of SSB, if SSB would be periodic or on-demand., etc. RAN4 should wait to discuss these initial access related requirements until RAN1 makes more progress regarding this topic.

Proposal 27: RAN4 should wait to study initial access related RRM requirements until RAN1 makes more progress regarding the signalling and procedure of initial access.

## Other PHY Signal/Channel/Procedure related RRM

Many topics being proposed under the category of other PHY signal/channel/procedure related RRM depend heavily on the sections 11.7 initial access, 11.8 MIMO operation and in particular 11.9 physical layer control, data scheduling and HARQ in the RAN1 agenda. According to RAN1 time schedule all these sections start in the February meeting. We share our view on the individual topics below.

### TCI Switching Delay Reduction

Figure 4-1: Timeline of known TCI state switch in 5G

5G RAN4 spec allows UE to receive one SSB to update its loop setting in both “known” and “unknown” TCI state switch procedure before UE is expected to communicate with the new TCI state. SSB in PCell has primarily been restricted to cell defined periodic SSB in 5G. Hence, RAN4 performance tests allow UE 20 ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process. This increases the timeline of “known” and “unknown” TCI state switch. Figure 4-1 shows the timeline of known TCI state switch in 5G.

Besides, 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is CSI-RS (SSB). This significantly increases the timeline of “unknown” TCI state switch procedure.

The extended timeline of unknown TCI state switch may become prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Hence, reducing the timelines of TCI state switch procedure would be very beneficial in 6G. RAN4 should study to reduce the TCI state switch timeline.

However, the reduction of TCI state switch procedure heavily depends on RAN1’s design of SSB. Hence, we propose to delay this discussion until RAN1 has started the work on SSB design for 6G and made some progress.

Observation 25: RAN4 performance tests allow UE 20ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process.

Observation 26: 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is QCled to CSI-RS (SSB).

Observation 27: The extended timeline of unknown TCI state switch is prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Proposal 28: RAN4 should discuss TCI state switch reduction after RAN1 has made more progress regarding SSB design.

### Selection of Channel for RRM Test Cases

Some settings and assumptions of evaluation periods in 5G might be more relaxed than necessary. Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins. One contributing factor is that 5G core requirements got defined for the worst-case scenario of TDL and AWGN channels, but performance tests were defined for AWGN in most cases. In 6G, RAN4 should discuss whether to use TDL, instead of AWGN, in more performance tests of 6G RRM. Given the limited number of TUs for RRM during 6G study item, RAN4 can have this discussion during the work item.

Observation 28: Core requirements of many 5G RAN4 RRM features were derived based on the worst-case performance among TDL-A/B/C and AWGN channels, but performance tests were defined for AWGN in most cases.

Proposal 29: During 6G work item discussion, RAN4 should discuss whether to use TDL, instead of AWGN, in more performance tests.

### RRM-specific category

We have observed the following proposals in the table excerpted from WF of Operation Efficiency (R4-2514648) that pursue the same goal as our proposal on RRM-specific categories. Essentially, all these proposals share the underlying idea of making the network aware that certain UEs may exceed the minimum requirements defined in the RRM specification.

WF on Operation Efficiency (R4-2514648) - Proposal 7: - Proposal 7d: Discuss whether to define two threads requirements, one focus on the baseline and another focus on the strict performance with real field request. - Proposal 7e: Study how to define RRM requirements that allow UE implementation based on minimum requirements but also allow UEs that can outperform the minimum requirements the benefits from such better performance - Proposal 11: - Introduce a more intuitive and simpler way to define RRM requirements, and new forms of representation if necessary.

WF on 6G RRM (R4-2514644) - RRM-specific Categories: - Proposal 1(QC): - Study how to establish a more intuitive and practical framework for differentiating UE RRM performance beyond baseline RRM requirements and individual capabilities: - The framework aims to reflect real UE implementation-based RRM performance, while RRM requirements continue to serve as the baseline for minimum performance. - The framework aims to cover a limited set of scenarios and cases with field/deployment relevance, within those defined in the RRM requirement specification. - The framework aims to focus on the most important RRM KPIs, such as measurement latency, mobility latency, carrier aggregation latency, interruption, etc. - The framework aims to accommodate different UE implementations across vendors without requiring disclosure of implementation details, unless such details must be known to the network. - The framework aims to minimize impacts on existing 3GPP specifications. - The framework aims to provide relationships to UE capabilities, where applicable. - The framework aims to be maintainable and verifiable with minimal effort.

To differentiate advanced UE RRM performance from minimum RRM performance, two possible approaches can be considered:

- Approach #1: Define two RRM requirement sets within the RAN4 RRM framework based on reference UE designs

- Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

- Advanced UE RRM requirements – based on an advanced reference UE design for all or a subset of scenarios.

- Approach #2: Define a separate set of UE RRM performance categories for specific KPI sets (e.g., mobility, measurement, CA)

- Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

- UE RRM-specific categories – without any reference UE design, applicable only to field-relevant scenarios.

To our understanding, Proposals 7d and 7e align with Approach #1, while Proposal 11 and Proposal 1 align with Approach #2. Although both approaches can differentiate advanced UE RRM performance from minimum RRM performance, we believe Approach #2 is more effective and practical due to the following drawbacks of Approach #1:

- Reaching consensus on an advanced reference UE design that accommodates different advanced UE implementations would be difficult.

- Even with an agreed advanced reference UE design, multiple optional UE capabilities would likely be associated with the design for different RRM requirements.

- Adding another type of UE reference design to RRM requirement specification would increase complexity, making intuitive assessment of UE RRM performance more difficult than today.

On the other hand, Approach #2 raised the following technical questions during offline discussions at RAN4#116bis:

- How should KPIs be selected and scenarios orchestrated for each KPI?

- Who determines the field-relevant scenarios and configurations for each KPI, and what is the decision process?

- How can UE RRM-specific performance categories be linked to other optional features introduced by other working groups?

These questions are valid and need to be addressed for Approach #2.

Proposal 30: Study how to differentiate real UE RRM performance beyond baseline RRM requirements and individual capabilities, in order to make the best use of advanced UE RRM performance in the field. Consider the following two approaches:

- Approach #1: Define two RRM requirement sets within the RAN4 RRM framework based on reference UE designs

  - Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

  - Advanced UE RRM requirements – based on an advanced reference UE design for all or a subset of scenarios.

- Approach #2: Define a separate set of UE RRM performance categories for specific KPI sets (e.g., mobility, measurement, CA)

  - Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

  - UE RRM-specific categories – without any reference UE design, applicable only to field-relevant scenarios.

### Our views on Other Topics

R4-2514644, Way forward from RAN4#116bis - FFS followings in the RAN4#117 meeting (other sub-topics are not precluded) - UE Tx timing: - Proposal 1(MTK): - Study PRACH (if introduced) specific timing accuracy requirement. - Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command. - Proposal 2(Nokia): - In 6G, the UE transmit timing requirements would have to address practical aspects from deployments and define UE requirements for these. - Inter-RAT sync: - Proposal 1(MTK): - Synchronized Inter-RAT to be the baseline assumption for 6G to enable inter-RAT measurement and mobility. - Testability: - Proposal 1(QC): - RAN4 should study whether to use TDL, instead of AWGN, in more performance tests of 6G RRM. - RRM-specific Categories: - Proposal 1(QC): - Study how to establish a more intuitive and practical framework for differentiating UE RRM performance beyond baseline RRM requirements and individual capabilities: - The framework aims to reflect real UE implementation-based RRM performance, while RRM requirements continue to serve as the baseline for minimum performance. - The framework aims to cover a limited set of scenarios and cases with field/deployment relevance, within those defined in the RRM requirement specification. - The framework aims to focus on the most important RRM KPIs, such as measurement latency, mobility latency, carrier aggregation latency, interruption, etc. - The framework aims to accommodate different UE implementations across vendors without requiring disclosure of implementation details, unless such details must be known to the network. - The framework aims to minimize impacts on existing 3GPP specifications. - The framework aims to provide relationships to UE capabilities, where applicable. - The framework aims to be maintainable and verifiable with minimal effort. - TCI switching reduction: - Proposal 1(QC): - RAN4 should study the mechanisms to reduce TCI state switch timeline in 6G. - Proposal 2(Nokia): - RAN4 to study the steps involved in TCI switching and identify potential reductions in TCI switching delay for 6G. - Duplexing: - Proposal 1(Samsung): - In 6GR, RAN4 RRM shall collaborate with RAN1 to discuss on how to support multiple types of duplexing including SBFD. - RRM relaxation and simplification for 6G massive IoT: - Proposal 1(Sony): - RAN4 should study the RRM relaxation and simplification for 6G massive IoT, comparing it with legacy IoT devices, to reduce device complexity and improve network/device energy efficiency. - Sensor based RRM: - Proposal 1(CATT): - RAN4 to consider enriching RRM measurement content and introducing sensing information based mobility management. - In addition to link quality measurement such as RSRP/RSRQ/SINR, the measurement content can also include speed, distance, angle, positioning, imaging, or activity detection, etc. - RAN4 to consider RRM impact for multi-functional RAN, where communication and sensing functionalities are jointly supported. - user-centric based RRM: - Proposal 1(CATT): - RAN4 to study the RRM impact for user-centric operation. - Unified UE capability: - Proposal 1(Xiaomi): - Unified modular based UE RRM Capability Definitions – Study common, reusable capability blocks for fundamental UE abilities (e.g., beam-sweeping factor, message-processing footprint) to improve consistency and scalability across RRM procedures. - BWP switch: - Proposal 1(vivo): - For BWP switch, reduction on BWP switch time may need study once BWP design (if there is any) in 6G is clear. Particularly, the reduction on the duration for UE parsing time may be studied if the BWP framework is further simplified. At the same time, RAN4 could study whether the RF retuning time can be further improved or not. - MRTD: - Proposal 1(Ericsson): - When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system. - When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between ARP. - TDD Cell Phase Synchronization: - Proposal 1(Ericsson): - Keep TDD Cell Phase Synchronization requirement the same as in NR NR. - CGI reading: - Proposal 1(Ericsson): - RAN4 should define the CGI reading requirement in 6G first release. - Purpose-based measurement requirements: - Proposal 1(Nokia): - For Connected mode, Idle mode, and Inactive mode, RAN4 to study defining measurement requirements depending on purpose of the configured measurement: mobility or data (CA). - FFS: - Due to the limited TU of the 6G SI for RRM, the topics in “Issue 13: Other PHY signal/channel/procedure related RRM” will not be studied in 6G SI timeline, unless it can be well justified with following criteria in RAN4#117 meeting: - Topics that can be initiated directly in RAN4 - Topics with clear commercial demand for RRM - Topics for fundamental feature in RRM (not incremental enhancement from 5G) - Topics with the strongest support from companies - Topics whose study can address the most critical pain points in 5G RRM - Topics requested from other WGs

There are multiple proposals categorized under ‘Other PHY signal/channel/procedure related RRM,’ as shown in the table above. Among them, the following proposals appear to be either heavily dependent on design details from other working groups or more suitable for the work item phase.

- UE Tx timing

- Inter-RAT sync

- Duplexing

- RRM relaxation and simplification for 6G massive IoT

- Sensor based RRM

- Unified UE capability

- BWP switch

- MRTD

- TDD Cell Phase Synchronization

- CGI reading

Proposal 31: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- UE Tx timing

- Inter-RAT sync

- Duplexing

- RRM relaxation and simplification for 6G massive IoT

- Sensor based RRM

- Unified UE capability

- BWP switch

- MRTD

- TDD Cell Phase Synchronization

- CGI reading

# Conclusions

In this paper we provided our views for RRM in the 6GR study item. Our observations and proposals are summarized as follows:

Measurement Gap (MG) and Interruption

Observation 1: It has been discussed but not formally been captured in the agreement that interruption enhancements are not only related to measurement gaps.

Proposal 1: The discussions on measurement gaps and interruptions should also aim to enhance interruptions for scenarios not related to measurement gaps. We propose to split the category measurement gap and interruption in the two subsections measurement gap and interruptions.

Proposal 2: RAN4 to follow the following high-level goals for the 6GR study item phase:

- Goal 1: Study mechanisms to minimize and optimize measurement gap configurations

- Goal 2: Study mechanisms to reduce or eliminate measurement gap interruptions

Observation 2: The introduction of multiple gap types and their concurrent operation increase system complexity, contributing to low adoption and underutilization of MG-related features.

Proposal 3: RAN4 to study how to specify a unified and flexible approach to MG framework that limits fragmentation and simplifies implementation across vendors and deployments.

Proposal 4: RAN4 should evaluate how to reduce the MG patterns (e.g., MGL, MGRP), ensuring they reflect real-world deployment requirements and usage patterns.

Observation 3: Existing gap configurations are often long and predefined, resulting in underutilization when measurements are not actively needed.

Observation 4: UEs currently have limited ability to influence gap configurations, resulting in suboptimal measurement scheduling and resource usage.

Proposal 5: RAN4 should study the overall measurement gap framework in 6GR and identify all inefficiencies—technical and operational—that hinder optimal gap usage.

Proposal 6: RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR.

Proposal 7: RAN4 should study how to enable UEs to indicate preferred MG configurations, allowing for more adaptive and efficient measurement scheduling

Observation 5: 5G NR allows for many scenarios interruptions on the active serving cells that impact the efficiency of the network significantly.

Observation 6: In specific scenarios, the true interruption of state-of-the-art UEs may be shorter than the minimal requirement in RAN4. In some scenarios, even no interruption may be needed.

Proposal 8: RAN4 should study all existing interruption scenarios in 5G NR, whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR.

Proposal 9: As a starting point, RAN4 could analyze interruptions for activation and deactivation of SCells and interruptions for measurements on deactivated SCells.

Observation 5: For 30 kHz and 120 kHz, the minimal requirement for the RF tune time of a measurement gap consumes one or two full slots.

Observation 6: For 15 kHz, the minimal requirement for the RF tune time of a measurement gap consumes half a slot. The latter half of the slot is unused for RF switching or data transmission, resulting in wasted capacity.

Proposal 10: RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission.

RRM Framework: Measurement Capability, Delay, Overhead, Accuracy, Unified Measurement

Observation 7: Independent on the content of the MAC CE command, the processing timeline of the MAC CE is always 3 ms in 5G NR. In LTE, however, the MAC CE processing timeline is dependent on the content of the MAC CE

Observation 8: Depending on whether a MAC CE command involves baseband only entities or also RF, whether it only applies for a single carrier or multiple carriers, etc., the activation time of a MAC CE command may be different.

Proposal 11: RAN4 should study the dependence of the MAC CE processing timeline on the content of the MAC CE command.

Observation 9: Current RRM requirements involving the RRC processing timeline do not account for the fact that a single RRC message can reconfigure parameters beyond those considered in the respective RRM requirements.

Proposal 12: RAN4 should study how to accommodate real UE implementation timelines based on the content of the RRC message when defining RRM requirements. For example, RRC-based BWP switch and RRC-based direct SCell activation.

Observation 10: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

Observation 11: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells.

Proposal 12: RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period related requirements in 6G.

Observation 12: 5G Rel-15 requirements were defined assuming that UE can measure measurement objects (MO) of up to two frequency layers simultaneously without measurement gap in CA/DC scenarios.

Observation 13: Rel-19 introduced a new UE capability that allows UE to measure MOs of three frequency layers simultaneously without measurement gap in CA/DC scenarios.

Observation 14: UE’s ability to simultaneously measure MOs of multiple frequency layers depends on the structure of SSB.

Proposal 13: RAN4 should study simultaneous measurement of MOs across multiple frequency layers without measurement gap in 6G only after RAN1 makes progress regarding the 6G SSB structure.

Observation 15: Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins.

Proposal 14: RAN4 should evaluate identification/measurement/reporting delay related requirements, in general, and study if and how to reduce them in 6G.

Mobility related RRM

Observation 16: Although LTM is being positioned as the baseline framework for unified measurement and mobility in 6G, as reflected in 3GPP discussions and industry workshops, its practicality remains uncertain due to the following anticipated implementation complexity and constraints related to deployment and applicability.

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Proposal 15: If a unified measurement and mobility framework for 6G is defined based on the 5G LTM feature, RAN4 should re-evaluate the benefits of the feature, thoroughly analyze the implementation challenges arising from its complexity and associated RRM requirements, and investigate how to address the identified issues to the extent that the feature can be implemented with reasonable complexity.

Observation 17: The 5G RRM specification contains several instances where the defined mobility requirements result in excessively long interruption durations and delays, for example, the “unknown” cell handover scenario. These requirements do not reflect realistic deployment conditions and raise questions about the rationale behind specifying such relaxed requirements.

Observation 18: Current inter-frequency mobility requirements are defined in a scenario-agnostic manner, despite field evidence showing that such handovers predominantly occur in specific configurations (e.g., FDD↔TDD). This generalized approach complicates optimization and may lead to overly conservative or misaligned requirements.

Observation 19: Defining mobility requirements uniformly across all scenarios can dilute technical precision and operational relevance. It also risks reducing the readability of the RAN4 specification due to excessive conditional logic. A more practical approach would be to focus on scenarios with clear field relevance and deployment evidence, unless strong justification exists for broader applicability.

Proposal 16: 6G mobility requirements should be constrained to scenarios where the resulting handover latencies or interruption durations are technically meaningful and operationally relevant. Requirements that lead to excessively large values should be avoided.

Proposal 17: RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness.

Proposal 18: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- Solutions for longer SSB periodicity in mobility

- Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility

RRM related Energy Efficiency

Proposal 19: Suggest postponing all discussions on energy efficiency related RRM discussions till RAN1and RAN2 have made some progress.

Spectrum Aggregation related RRM

Observation 20: Despite multiple SCell/SCG enhancements introduced since Rel-15, only mandatory MAC-based activation features have seen commercial adoption. This highlights a gap between specification complexity and field relevance.

Proposal 20: For 6G carrier aggregation features, RAN4 should investigate which SCell activation features have been commercially deployed or are planned for deployment and prioritize those based on proven deployment evidence to avoid unnecessary complexity and ensure practical impact.

Observation 21: 5G interruption requirements during SCell activation result in blanking scheduling opportunities across all serving cells for extended periods, which may not reflect practical traffic conditions.

Proposal 21: In 6G, interruption requirements during deactivated SCell measurement and SCell activation should be defined to align with state-of-the-art UE implementations. For example, the interruption duration for intra-band CA in 5G is excessively long and may not reflect practical deployment needs.

Observation 22: RAN4’s SCell activation latency requirements in 5G are overly complex and misaligned with real-world implementations. Assumptions such as strict SSB alignment and multi-step synchronization inflate latency on paper, leading to conservative deactivation behavior and increased UE power consumption.

Proposal 22: For 6G carrier aggregation, RAN4 should re-evaluate the assumptions made in the 5G SCell activation requirement definition, based on state-of-the-art UE implementations, and investigate whether and to what extent SCell activation latency modeling can be simplified to better align requirements with practical UE behavior and implementation realities.

Observation 23: The current RRM requirement for RRC-based SCell activation does not account for the number of SCells to be added/activated and serving cells’ RRC reconfiguration upon the RRC message in the RRC processing timeline.

Proposal 23: RAN4 should study how to accommodate real UE implementation constraints in the RRC processing timeline. For example:

- Single SCell addition/activation vs. multiple SCells addition/activation

- SCell addition/activation only vs. SCell addition/activation combined with serving cell configuration updates triggered by the RRC message.

Proposal 24: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- RRM conditions and requirements for Single Cell Multi-Carriers

- RRM impacts of DL and UL decoupling

- Carrier switch enhancements for UL and DL

- RRM impacts of realistic SCS for spectrum

- relaxation of the requirement on timing alignment between carriers

- RRM for MRSS

MIMO and mTRP Operation related RRM

Observation 24: The RAN1 work on the relevant sections in the RAN1 agenda for MIMO and mTRP operation related RRM will not start before February.

Proposal 25: We propose to delay all discussions on PHY signal/channel/procedure related RRM till RAN1 has started the work and made some progress.

NTN related RRM

Proposal 26: We propose to delay all discussions on NTN related RRM till other working groups have started the work and made some progress.

Initial Access related RRM

Proposal 27: RAN4 should wait to study initial access related RRM requirements until RAN1 makes more progress regarding the signalling and procedure of initial access.

Other PHY Signal/Channel/Procedure related RRM

Observation 25: RAN4 performance tests allow UE 20ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process.

Observation 26: 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is QCled to CSI-RS (SSB).

Observation 27: The extended timeline of unknown TCI state switch is prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Proposal 28: RAN4 should discuss TCI state switch reduction after RAN1 has made more progress regarding SSB design.

Observation 28: Core requirements of many 5G RAN4 RRM features were derived based on the worst-case performance among TDL-A/B/C and AWGN channels, but performance tests were defined for AWGN in most cases.

Proposal 29: During 6G work item discussion, RAN4 should discuss whether to use TDL, instead of AWGN, in more performance tests.

Proposal 30: Study how to differentiate real UE RRM performance beyond baseline RRM requirements and individual capabilities, in order to make the best use of advanced UE RRM performance in the field. Consider the following two approaches:

- Approach #1: Define two RRM requirement sets within the RAN4 RRM framework based on reference UE designs

  - Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

  - Advanced UE RRM requirements – based on an advanced reference UE design for all or a subset of scenarios.

- Approach #2: Define a separate set of UE RRM performance categories for specific KPI sets (e.g., mobility, measurement, CA)

  - Minimum UE RRM requirements – based on a basic reference UE design for all scenarios.

  - UE RRM-specific categories – without any reference UE design, applicable only to field-relevant scenarios.

Proposal 31: Deprioritize the following items during the study phase, as they are either heavily dependent on design details from other working groups or more suitable for the work item phase:

- UE Tx timing

- Inter-RAT sync

- Duplexing

- RRM relaxation and simplification for 6G massive IoT

- Sensor based RRM

- Unified UE capability

- BWP switch

- MRTD

- TDD Cell Phase Synchronization

- CGI reading

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- R4-2514625, AdHoc meeting minutes on 6G RRM, RAN4 #116bis, October 2025

- R4-2514644, WF on [116bis][105] 6G RRM, RAN4 #116bis, October 2025