3GPP TSG-RAN WG4 Meeting #117                        R4-2520053                                                                                                       Dallas, Texas, USA, 17 – 21 November 2025

Source:	China Telecom

Title:	Discussion on UE requirements for 6Rx UE with interference

Agenda Item:	7.5.4

Document for:	Discussion

Introduction

In RAN #109 meeting, the new WID on NR demodulation performance: Phase6 was approved [1], with the following objectives for the UE requirements for 6Rx UE with interference:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

The RAN4#116bis is the first meeting companies discussed the test scopes, parameters and simulation assumptions for 6Rx requirements and the latest status are captured in [2].

In this contribution, our views on UE requirements for 6Rx UE with interference are provided.

Discussion

General

Coverage of UE types

- Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices

It is clearly stated in the WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. And ‘whether to define one set or two sets of requirements for handheld and FWA UEs’ is related to each individual test parameter issues.

According to the latest status in the WI ‘NR_ENDC_RF_Ph4-Perf’, it is unlikely to define UE declaration on UE type or to define 2 sets of requirements for handheld and FWA UE in Rel-19 [3]. Thus, we do not observe the necessity to define 2 sets of requirements for handheld and FWA 6Rx UEs particularly under interference scenario.

We propose to define only one set of requirements for both handheld and FWA 6Rx UE, i.e., for each test, the same test parameters will be applied for both UE device types.

Observation 1: For basic 6Rx requirements in Rel-19, it is unlikely to define UE declaration on UE type or to define 2 sets of requirements for handheld and FWA UE.

Proposal 1: Define only one set of requirements for both handheld and FWA 6Rx UE, i.e., for each test, the same test parameters will be applied for both UE device types.

Antenna correlation

- Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

The latest status for the basic 6Rx requirements is as below [3]:

Parameter; Value. Propagation channel; 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = [0.07179] 6 Layers: TDLA30-10 ULA Low.

For rank 4 test, there will be single antenna correlation applies for both handheld and FWA UEs. Although there are 2 antenna correlation values defined for FWA UE, we do not think the rank 6 configuration needs to be considered since we have decided to limit the total layer within 4 for both ICI and MU-MIMO testing.

We propose to reuse the same antenna correlation defined for rank 4 SU-MIMO requirement for 6Rx, i.e., ULA correlation channel with α = 0 and β = [0.07179] (where [ ] depends on final check in WI ‘NR_ENDC_RF_Ph4-Perf’).

Proposal 2: Reuse the same antenna correlation defined for rank 4 SU-MIMO requirement for 6Rx, i.e., ULA correlation channel with α = 0 and β = [0.07179] (where [ ] depends on final check in WI ‘NR_ENDC_RF_Ph4-Perf’).

PDCSH requirements with inter cell interference

Propagation condition

- Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

As discussed above, we do not support to define different requirements/test parameters for different device types.

As we have decided in the last meeting to cover both HomNet and HetNet scenarios, we propose to use TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) to align with existing 2/4Rx requirements.

Proposal 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

MCS

- Option 1: MCS 17 for HomNet and MCS 13 for HetNet - Option 2: MCS 17 - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

We have agreed in the last meeting to cover both rank 1/2/4 for initial simulation, thus we propose to use MCS 17 for rank 1/2 and MCS 13 for rank 4.

Proposal 4: For initial simulation, use MCS 17 for rank 1/2 and MCS 13 for rank 4.

PDCSH requirements with intra cell inter user interference

Propagation condition

- Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

As discussed above, we do not support to define different requirements/test parameters for different device types.

Since we have agreed to cover both rank 1+1 and rank 2+2 for initial simulation, we propose to consider TDLC300-100 for rank 1+1 and TDLA30-10 for rank 2+2 for initial simulation purpose.

Proposal 5: For initial simulation, consider TDLC300-100 for rank 1+1 and TDLA30-10 for rank 2+2.

Precoding method for the co-scheduled UE

- Option 1: Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests - Option 2: Orthogonal precoding for rank 2+2 tests

Since we have agreed to cover both rank 1+1 and rank 2+2 for initial simulation, we propose to align with the 2/4Rx simulation assumption, i.e., Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Proposal 6: For initial simulation, consider random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

MCS for the target UE

- Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case - Option 2: MCS 17 (Table 1) for 2+2 case - Option 3: Cover MCS 13 and MCS 17 for evaluation

Proposal 7: For initial simulation, consider MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case.

Wideband CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

It has been decided in Rel-19 basic 6Rx that there will be CQI requirements for 6Rx UE [3]:

- Agreements: - Define CQI requirements with rank 4 under static channel - Propagation channel: AWGN - Antenna: 4x6 with static channel specified in Annex B.1 - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

Therefore, it is now clear according to the WID [1] that RAN4 will introduce CQI reporting requirements with inter-cell interference with MMSE-IRC receiver.

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - xxx - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - xxx

Interference modelling and test parameters for CQI reporting requirements with inter-cell interference

We propose the following simulation assumptions as a star point:

| Parameter | Value |
| --- | --- |
| SCS/CBW | 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| SINR | TBD based on simulation results |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA correlation channel with α = 0 and β = [0.07179] |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Measurement channel | As specified in Table A.4-2, TBS.2-2 (Rank 2 for the serving cell) |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Test metric for CQI reporting requirements with inter-cell interference

As for the test metric, we propose to reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx:

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 8 RAN4 will introduce CQI reporting requirements with inter-cell interference with MMSE-IRC receiver according to the WID.

Proposal 9 Use the following simulation assumptions as a star point.

| Parameter | Value |
| --- | --- |
| SCS/CBW | 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| SINR | TBD based on simulation results |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA correlation channel with α = 0 and β = [0.07179] |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Measurement channel | As specified in Table A.4-2, TBS.2-2 (Rank 2 for the serving cell) |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Proposal 10 Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Test applicability and release independent

Test applicability for 6Rx requirements with interference

- Option 1: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule - Option 2: Define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not

We think both option 1 and option 2 are aligned with the existing applicability rules for 8Rx requirements with interference.

Considering there could be UEs supporting both 6Rx and 8Rx, RAN4 has agreed the following additional applicability rule in Rel-19 [3], which could be used for new requirements with interference.

Issue 1-4: Test applicability rule for support different number Rx antenna ports - Implement the following proposal into the CR, and further check the CR can be agreeable or not. - Corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass. - xxx

Proposal 11 Define the following applicability rules for 6Rx requirements:

- For UEs supporting 6Rx and other antenna ports (2/4/8Rx), if the UE passes tests with interference with more UE antennas, it can skip requirements with interference with less UE antennas, no matter the same test configurations are defined or not.

- No additional test applicability is needed.

Release independent for 6Rx requirements with interference

- Option 1: Follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ - Option 2: All new 6Rx interference requirements introduced shall be release independent from Rel-17

No matter what decisions are made within Rel-19 WI ‘NR_ENDC_RF_Ph4’, we believe the same release independent rule should be applied for the new 6Rx requirements with interference.

Proposal 12 Follow the same conclusion for release independent as in the Rel-19 WI ‘NR_ENDC_RF_Ph4’.

Conclusion

Observation 1: For basic 6Rx requirements in Rel-19, it is unlikely to define UE declaration on UE type or to define 2 sets of requirements for handheld and FWA UE.

Proposal 1: Define only one set of requirements for both handheld and FWA 6Rx UE, i.e., for each test, the same test parameters will be applied for both UE device types.

Proposal 2: Reuse the same antenna correlation defined for rank 4 SU-MIMO requirement for 6Rx, i.e., ULA correlation channel with α = 0 and β = [0.07179] (where [ ] depends on final check in WI ‘NR_ENDC_RF_Ph4-Perf’).

Proposal 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

Proposal 4: For initial simulation, use MCS 17 for rank 1/2 and MCS 13 for rank 4.

Proposal 5: For initial simulation, consider TDLC300-100 for rank 1+1 and TDLA30-10 for rank 2+2.

Proposal 6: For initial simulation, consider random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Proposal 7: For initial simulation, consider MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case.

Proposal 8 RAN4 will introduce CQI reporting requirements with inter-cell interference with MMSE-IRC receiver according to the WID.

Proposal 9 Use the following simulation assumptions as a star point.

| Parameter | Value |
| --- | --- |
| SCS/CBW | 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| SINR | TBD based on simulation results |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA correlation channel with α = 0 and β = [0.07179] |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Measurement channel | As specified in Table A.4-2, TBS.2-2 (Rank 2 for the serving cell) |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Proposal 10 Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 11 Define the following applicability rules for 6Rx requirements:

- For UEs supporting 6Rx and other antenna ports (2/4/8Rx), if the UE passes tests with interference with more UE antennas, it can skip requirements with interference with less UE antennas, no matter the same test configurations are defined or not.

- No additional test applicability is needed.

Proposal 12 Follow the same conclusion for release independent as in the Rel-19 WI ‘NR_ENDC_RF_Ph4’.

Reference

RP-252957, WID: NR demodulation performance: Phase 6, Apple, RAN#109, Sep 2025

R4-2514823, WF on NR_demod_Ph6_Ph5, China Telecom, RAN4#116bis, Oct 2025

R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116bis, Oct 2025