3GPP TSG-RAN WG4 Meeting #117                                                                                                      R4-2520065

Dallas , USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Further discussion on framework simplification for co-location/co-existence requirements

Agenda item:	12.1

Document for:	Approval

- Introduction

In RAN4#116bis meeting, A Way forward for RAN Task on simplification of co-existence and co-location tables in BS specifications [1] was agreed and remaining issues need further discussion.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

Co-location requirement in TS 37.145-1

Referring to WF[1], the simplified co-location requirement table for TS 38.104  is copied as below:

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with (Note 4) | Basic limits (dBm/100kHz) (Note 1) |  |  |
| --- | --- | --- | --- | --- |
| co-located BS (MHz) (Note 5) |  | WA BS | MR BS | LA BS |
| 824 - 849 | GSM850 or CDMA850 | -98 | -91 | -70 |
| 876 - 915 | GSM900 | -98 | -91 | -70 |
| 1710 - 1785 | DCS1800 | -98 | -91 | -80 |
| 1850 - 1910 | PCS1900 | -98 | -91 | -80 |
| 49, 51/n51, n91, n93 | E-UTRA or NR | N/A | N/A | -88 |
| 46/n46, 53/n53 | E-UTRA or NR | N/A | -91 | -88 |
| n100, n101 | NR | -96 | N/A | N/A |
| n96, n102 | NR | N/A | -90 | -87 |
| n104 | NR | -95 | -90 | -87 |
| Other operating band | UTRA, E-UTRA or NR | -96 | -91 | -88 |

For TS 37.145-1, the co-location requirement with other Base Stations in clause 6.6.6.5.2.6 of TS 37.145-1 is defined in Table 6.6.6.5.2.6-1 copied as below:

Table 6.6.6.5.2.6-1: Spurious emissions basic limits for MSR, E-UTRA or UTRA (FDD) or NR BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limit (WA BS) | Basic limit (MR BS) | Basic limit (LA BS) | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876 - 915 MHz | -98 dBm | -91 dBm | MSR -88 dBm, UTRA, E-UTRA -70 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -98 dBm | -91 dBm (UTRA -96 dBm) | MSR -88 dBm, UTRA, E-UTRA -80 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -98 dBm | -91 dBm (UTRA -96 dBm) | MSR -88 dBm UTRA, E-UTRA -80 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | -91 dBm | MSR -88 dBm UTRA, E-UTRA -70 dBm | 100 kHz |  |
| … | … | … | … | ... | … |  |

The co-location requirement in TS 37.145-1 is different from that in TS 38.104, The co-location requirement in TS 37.145-1 is defined for MSR, E-UTRA or UTRA (FDD) or NR BS. As above table, the Basic limit varies with MSR, E-UTRA, UTRA, or NR BS, for example, for LA BS, Basic limit for MSR is -88dBm, which is different from -70dB for UTRA  when type of co-located BS is GSM 900. The Basic limit(yellow shadow) are not captured in endorsed draft CR R4-2515118. RAN4 need to discuss whether to add them as shown in the table below.

Table 6.6.6.5.2.6-1: Spurious emissions basic limits for MSR, E-UTRA or UTRA (FDD) or NR BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with (Note 5) | Basic limits (dBm/100kHz) |  |  |
| --- | --- | --- | --- | --- |
| co-located BS (MHz) (Note 6) |  | WA BS | MR BS | LA BS |
| 824 - 849 | GSM850 or CDMA850 | -98 | -91 | -70 (MSR -88 dBm) |
| 876 - 915 | GSM900 | -98 | -91 | -70 (MSR -88 dBm) |
| 1710 - 1785 | DCS1800 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| 1850 - 1910 | PCS1900 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| n51/51, n91, n93 | NR or E-UTRA | N/A | N/A | -88 |
| n46/46, n53/53 | NR or E-UTRA | N/A | -91 | -88 |
| n100, n101 | NR | -96 | N/A | N/A |
| n96, n102 | NR | N/A | -90 | -87 |
| n104 | NR | -95 | -90 | -87 |
| Other operating band | UTRA, E-UTRA or NR | -96 | -91 | -88 |

So, we propose,

Proposal 1: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 37.145-1 as below:

Table 6.6.6.5.2.6-1: Spurious emissions basic limits for MSR, E-UTRA or UTRA (FDD) or NR BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with (Note 5) | Basic limits (dBm/100kHz) |  |  |
| --- | --- | --- | --- | --- |
| co-located BS (MHz) (Note 6) |  | WA BS | MR BS | LA BS |
| 824 - 849 | GSM850 or CDMA850 | -98 | -91 | -70 (MSR -88 dBm) |
| 876 - 915 | GSM900 | -98 | -91 | -70 (MSR -88 dBm) |
| 1710 - 1785 | DCS1800 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| 1850 - 1910 | PCS1900 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| n51/51, n91, n93 | NR or E-UTRA | N/A | N/A | -88 |
| n46/46, n53/53 | NR or E-UTRA | N/A | -91 | -88 |
| n100, n101 | NR | -96 | N/A | N/A |
| n96, n102 | NR | N/A | -90 | -87 |
| n104 | NR | -95 | -90 | -87 |
| Other operating band | UTRA, E-UTRA or NR | -96 | -91 | -88 |

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Proposal 1: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 37.145-1 as below:

Table 6.6.6.5.2.6-1: Spurious emissions basic limits for MSR, E-UTRA or UTRA (FDD) or NR BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with (Note 5) | Basic limits (dBm/100kHz) |  |  |
| --- | --- | --- | --- | --- |
| co-located BS (MHz) (Note 6) |  | WA BS | MR BS | LA BS |
| 824 - 849 | GSM850 or CDMA850 | -98 | -91 | -70 (MSR -88 dBm) |
| 876 - 915 | GSM900 | -98 | -91 | -70 (MSR -88 dBm) |
| 1710 - 1785 | DCS1800 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| 1850 - 1910 | PCS1900 | -98 | -91 (UTRA -96 dBm) | -80 (MSR -88 dBm) |
| n51/51, n91, n93 | NR or E-UTRA | N/A | N/A | -88 |
| n46/46, n53/53 | NR or E-UTRA | N/A | -91 | -88 |
| n100, n101 | NR | -96 | N/A | N/A |
| n96, n102 | NR | N/A | -90 | -87 |
| n104 | NR | -95 | -90 | -87 |
| Other operating band | UTRA, E-UTRA or NR | -96 | -91 | -88 |

- References

- R4-2515069, Way forward for RAN Task on simplification of co-existence and co-location tables in BS specifications, Ericsson, RAN4#116bis

- R4-2515118, (TEI19)CR for 37.145-1, on framework simplification for co-location/co-existence requirement[BDaT_simp_improvement], CATT, RAN4#116bis