3GPP TSG-RAN WG4 Meeting #117	R4-2520075

# Dallas, Texas, United States, Nov 17th – 21st , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.768 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 38.768 to modify the formulas for the switching gap calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the approved TP R4-2514606 in the RAN4#116bis, there are some aspects at least to be further improved for the switching gap calculation. This is a resubmission of approved TP R4-2514606. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To improve the switching gap calculation in the clause 5.2 of TR 38.768. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some aspects at least to be further improved for the switching gap calculation in TR 38.768. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>> Start of Changes <<

5.2	Switching period location

5.2.1	General

For what follows we consider switching between an FDD and SDL band for the CA-n5A-29A configuration in Table 4-1 but without loss of generality: one UL is configured in a paired FDD band with the DL SCell configured in an SDL band and both bands belong to Sub-1GHz bands.

The switching period, defined in units of 𝜇s according to UE capability, is located within a switching gap located within the switched-from carrier. Switching gaps, expressed in units of 15KHz SCS symbol, are accommodated by the network according to the physical layer design of the feature.

The timing reference for the switching pattern and switching gap at the UE is the DL timing of the Pcell (in the FDD band). The uplink radio frame transmission on the Pcell takes place TTA = (NTA + TAoffset)Tc before the reception of the first detected path (as defined in TS 38.133) in time on the corresponding DL radio frame. This means that the timing advance must be considered at switching events.

The duration of the switching gap is RRC configured. The network ensures that the switching gap is large enough to cover the switching period (35𝜇s, 70𝜇s, and 140𝜇s according to the UE capability), transient periods and the TA, if needed, in both switch directions.

5.2.2	FDD to SDL switch

For the FDD to SDL switch the gNB configures a gap long enough to accommodate timing advance such that no collisions should occur. Figure 5.2.2-1 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods, switching period (35𝜇s, 70𝜇s, and 140𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier.

The UL slot will always end before the DL slot, neglecting any timing errors in case TTA = 0 µs. The worst case in terms of switching is that in which TTA = 0 µs, the DL and UL timing are the same (again neglecting the errors that are of the order of 1 µs) and the SDL starts RTD µs before the Pcell DL slot timing. Switching period is always after UL transient period.  The switching gap with respect to the end of the last FDD DL symbol must therefore be at least

when FDD UL trailing end transient period - TTA * 106≤0:

actual switching gap length ≥ ceil[(Tswitch + RTDP2S + SDL leading transient period)/(1/14*103)]

when FDD UL trailing end transient period - TTA * 106 > 0:

actual switching gap length ≥ ceil[ (Tswitch + RTDP2S + FDD UL trailing end transient period- TTA * 106 + SDL leading transient period)/(1/14*103)]

where the RTDP2S is the receiving timing difference among the closest DL slot timing boundaries between PCell and SCell, with the range of negative MRTD to positive MRTD. And the MRTD = 3us assuming that the FDD and SDL carriers are colocated. UL transient period is assumed as 10us.

5.2.3	SDL to FDD switch

For the SDL to FDD switch, the switching gap ends at end of slot on “switch from” carrier (SDL). The timing advance must also be accounted for; Figure 5.2.3-1 illustrates this for all applicable switching periods Tswitch (35μs, 70μs, and  140𝜇s according to UE capability),  by placing the switching period X. The applicability of ON and OFF power requirements is also shown.

Figure 5.2.3-1: SDL to FDD switch with the switch occurring before the start of the UL slot.

The time mask could be devised such that the switching period occurs before the first symbol of the UL slot of the switched-to FDD carrier, this to avoid dropping symbols on the UL that starts TTA before the first symbol of the FDD DL slot. Notwithstanding, the switching gap on the SDL carrier must be at least

Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ RTDS2P + FDD UL leading transient period ) / (1/14*103) ]

Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. UL transient period is assumed as 10us.

In the live network operation, timing advance for a UE in a cell is variable. It is the network task to adjust the length of the switching gap by gapDurationSCelltoPCell considering maximum cell coverage of PCell without the knowledge of accurate NTA for each UE, so that the UL transient period is not overlapping with switching period X.

>> End of Changes <<