3GPP TSG-RAN WG4 Meeting #117	R4-2520076

# Dallas, Texas, United States, Nov 17th – 21st , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3047 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1 to introduce DL RMC for LB-LB CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, MediaTek Inc., Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce the DL RMC for LB-LB CA via switching based on the endorsed CR R4-2514604. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the Reference sensitivity power level for Inter-band CA via switching in clause 7.3A.2.5. - To introduce the DL RMC for inter-band CA via switching in clause A.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification can’t support the LB-LB via switching without RF requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.5 (New), A.8 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>> Start of Changes <<

7.3A.2.5	Reference sensitivity power level for Inter-band CA via switching

The reference sensitivity power level REFSENS for Inter-band CA via switching in a band pair is the minimum mean power applied to each band respectively at each one of the UE antenna ports for all UE categories, at which the throughput in the DL scheduling before and/or after the Switching Gap shall meet or exceed the requirements for the specified reference measurement channel. It’s noted that the DL scheduling in the RMC for Inter-band CA via switching as specified in Annexes A.8 is closest to the Switching Gap (gapDurationPCelltoSCell-r19 and gapDurationSCelltoPCell-r19) configured by network.

For a UE indicating the capability [switchingPeriodForFDD-SDL] for the band pair of NR inter-band CA combinations defined in Table 5.2A.3-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2/A8.1 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with the REFSENS specified in Table 7.3.2-1a for PCell FDD band, the REFSENS specified in Table 7.3.2-1b for SCell SDL band and uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 for PCell FDD band.

>> Next of Changes <<

A.8	Inter-band CA via switching reference measurement channels

A.8.1	Inter-band CA via switching DL reference measurement channels

Table A.8.1-1 DL RMC for inter-band CA via switching

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Switching Pattern for Low-band CA | LowBandCA-Switching-r19 -> switchingPattern-r19 | A bitmap of slots | 00011 00110 01100 11001 11100 11001 10011 00110 |
|  | Period of Switching Pattern | Slots | 40 |
|  | Slots on the PCell |  | A bit value of ‘0’ in LBCA-SwitchingPattern |
|  | Slots on the SCell |  | A bit value of ‘1’ in LBCA-SwitchingPattern |
| Switch-from DL slot on the PCell/SCell | Slot Location |  | Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18 |
|  | The number of Switch-from Slot |  | 18 slots for each period of 40 slots. |
|  | CORESET frequency domain allocation |  | Full BW |
|  | CORESET time domain allocation |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  | Type A |
|  | PDSCH start symbol index (S) |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  | 12 - gapDurationPCelltoSCell-r19 for the slot indicated as ‘0’ in the LBCA-SwitchingPattern Or 12 - gapDurationSCelltoPCell-r19 for the slot indicated as ‘1’ in the LBCA-SwitchingPattern |
|  | PDSCH PRB bundling | PRBs | 2 |
|  | Dynamic PRB bundling |  | false |
|  | Overhead value for TBS determination |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  | 2 |
|  | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | FDM between DMRS and PDSCH |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) | Slots | 2 |
| Switch-to DL slot on the PCell/SCell | Slot Location |  | Mod(i, 20) = 3, 5, 7, 9, 11, 13, 15, 17, 19 |
|  | The number of Switch-from Slot |  | 18 slots for every period of 40 slots. |
|  | CORESET frequency domain allocation |  | Full BW |
|  | CORESET time domain allocation |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  | Type A |
|  | PDSCH start symbol index (S) |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  | 12 |
|  | PDSCH PRB bundling | PRBs | 2 |
|  | Dynamic PRB bundling |  | false |
|  | Overhead value for TBS determination |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  | 2 |
|  | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | FDM between DMRS and PDSCH |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) | Slots | 2 |
| HARQ process number |  |  | 4 |
| Subcarrier spacing |  | kHz | 15 |
| Subcarrier spacing configuration µ |  |  | 0 |
| Subcarriers per resource block |  |  | 12 |
| Allocated slots per period of 40 slots on the PCell |  | Slots | 18 |
| Allocated slots per period of 40 slots on the SCell |  | Slots | 18 |
| MCS Index |  |  | 4 |
| MCS Table for TBS determination |  |  | 64QAM / table 5.1.3.1-1: MCS index table 1 for PDSCH in TS 38.214 |
| Modulation |  |  | QPSK |
| Target Coding Rate R x [1024] |  |  | 308 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Channel bandwidth |  |  | These parameters can refer to Table A.8.1-2 for differernt channel bandwidth. As PCell and SCell can be configured with different channel bandwidth, TBS per slot for PCell and SCell could be different. |
| Allocated resource blocks |  |  |  |
| Information Bit Payload (TBS) per Slot |  |  |  |
| Number of Code Blocks per Slot |  |  |  |
| Number of Code Blocks per Slot |  |  |  |
| Max. Throughput averaged over 4 frame |  | Mbps | (18*TBSPCell + 18*TBSSCell) / 0.04 / 10^6 |
| RTDP2S (NOTE2) |  | µs | 0 |
| RTDS2P (NOTE2) |  | µs | 0 |
| NTA |  | Tc | 0 |
| NTA,offset |  | Tc | 0 |
| gapDurationPCelltoSCell-r19 |  | Symbol | ‘1’ for 35us of [switchingPeriodForFDD-SDL] ‘2’ for 70us of [switchingPeriodForFDD-SDL] ‘3’ for 140us of [switchingPeriodForFDD-SDL] |
| gapDurationSCelltoPCell-r19 |  | Symbol | ‘1’ for 35us of [switchingPeriodForFDD-SDL] ‘2’ for 70us of [switchingPeriodForFDD-SDL] ‘3’ for 140us of [switchingPeriodForFDD-SDL] |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | For PCell FDD band, CSI-RS offset are: 0 for CSI-RS resource 1 and 2 1 for CSI-RS resource 3 and 4 For SCell SDL band, CSI-RS offset are: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |
| NOTE 1: Slot i is slot index per four frames. NOTE 2: The definition is referred to the clause 5.2.2 and 5.2.3 of TR 38.768 |  |  |  |

Table A.8.1-2 DL TBS for different channel bandwidth under the inter-band CA via switching

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 5 | 10 | 15 | 20 | 25 |
| Allocated resource blocks |  |  | 25 | 52 | 79 | 106 | 133 |
| For Slots i in which PDSCH is not allocated, If Mod(i, 20) = 0 or 1 | Information Bit Payload per Slot | Bits | NA | NA | NA | NA | NA |
|  | Number of Code Blocks per Slot | CBs | NA | NA | NA | NA | NA |
|  | Binary Channel Bits per Slot | Bits | NA | NA | NA | NA | NA |
| For Switch-from Slots i in which PDSCH is allocated, if Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18 and ‘1’ is indicated by gapDurationPCelltoSCell-r19 or gapDurationSCelltoPCell-r19 | Allocated data symbols of PDSCH | Symbols | 8 |  |  |  |  |
|  | Information Bit Payload per Slot | Bits | 1480 | 2976 | 4480 | 6144 | 7680 |
|  | Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
|  | LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot / CBs | CBs | 1 | 1 | 1 | 1 | 1 |
|  | Binary Channel Bits per Slot / Bits | Bits | 4800 | 9984 | 15168 | 20352 | 25536 |
| For Switch-from Slots i in which PDSCH is allocated, if Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18 and ‘2’ is indicated by gapDurationPCelltoSCell-r19 or gapDurationSCelltoPCell-r19 | Allocated data symbols of PDSCH | Symbols | 7 |  |  |  |  |
|  | Information Bit Payload per Slot | Bits | 1256 | 2664 | 3968 | 5376 | 6656 |
|  | Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
|  | LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot / CBs | CBs | 1 | 1 | 1 | 1 | 1 |
|  | Binary Channel Bits per Slot / Bits | Bits | 4200 | 8736 | 13272 | 17808 | 22344 |
| For Switch-from Slots i in which PDSCH is allocated, if Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18 and ‘3’ is indicated by gapDurationPCelltoSCell-r19 or gapDurationSCelltoPCell-r19 | Allocated data symbols of PDSCH | Symbols | 6 |  |  |  |  |
|  | Information Bit Payload per Slot | Bits | 1128 | 2280 | 3496 | 4608 | 5760 |
|  | Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 |
|  | LDPC base graph |  | 2 | 2 | 2 | 1 | 1 |
|  | Number of Code Blocks per Slot / CBs | CBs | 1 | 1 | 1 | 1 | 1 |
|  | Binary Channel Bits per Slot / Bits | Bits | 3600 | 7488 | 11376 | 15264 | 19152 |
| For Switch-to Slots i in which PDSCH is allocated, If Mod(i, 20) = 3, 5, 7, 9, 11, 13, 15, 17, 19 | Allocated data symbols of PDSCH | Symbols | 9 |  |  |  |  |
|  | Information Bit Payload per Slot | Bits | 1672 | 3368 | 5120 | 6912 | 8712 |
|  | Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
|  | LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot | CBs | 1 | 1 | 1 | 1 | 2 |
|  | Binary Channel Bits per Slot | Bits | 5400 | 11232 | 17064 | 22896 | 28728 |
| NOTE 1: Additional parameters are specified in Table A.8.1-1. NOTE 2: If TB size is larger than 3824, the CRC sequence of L = 24 Bits is attached to each Code Block. NOTE 3: SS/PBCH block is transmitted in slot 0, 1 and 20, 21 of each period of 40 slots NOTE 4: Slot i is slot index per four frames. |  |  |  |  |  |  |  |

>> End of Changes <<