3GPP TSG RAN WG4 Meeting #117		R4-2520089

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on R20 HPUE coexistence study

Agenda Item:	7.1.3

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN4#116bis meeting, a WF [1] for R20 HPUE coexistence study was approved. In this paper, we’d like to provide our views on the UL power control, performance metric and ISD assumption.

2 Discussion

- UL power control:

Power control model specified in Section 9.1 TR 36.942 as follows is adopted. where, - Pmax = 32dBm - CLx-ile = [88 + 10*log10(200/X) + 11 – Y + Pmax – 23, where X is UL transmission BW (MHz) and Y is the BS noise figure] - γ = 1

In the last meeting, RAN4 reached the WF on the UL power control. Generally, RAN4 assume the open loop UL power control with complete compensation of path loss. The targeted SNR is 15dB.

Thus, CLx-ile=Pmax – Targeted SNR – BS thermal Noise = 32 – 15 – (-174+NF+10log10(BW)+60). Based on the approved WF [1], BS Noise figure is 5dB for below 6 GHz, and 6dB for 7GHz. 100MHz BW is assumed for the coexistence study.

CLx-ile=Pmax – Targeted SNR – BS thermal Noise = 32 – 15 – (-174+NF+10log10(BW)+60)

CLx-ile = 116dB for below 6 GHz

CLx-ile = 115dB for 7 GHz

Proposal 1: According to the formula (CLx-ile=Pmax – Targeted SNR – BS thermal Noise), the following CLx-ile can be assumed for 32dBm Pmax for different BS NF.

CLx-ile = 116dB for below 6 GHz

CLx-ile = 115dB for 7 GHz

- performance metric

In the last meeting, RAN4 reached the following WFs on the performance metric. There is no need to make decision right now. RAN4 can collect the results in next February with these two options. In addition, for option 2, the baseline is 23dBm for both victim and aggressor systems. For these ultra-high TDD bands, generally UE MOP is 26dBm.

8 Performance metric - Option 1 Average throughput loss and 5%-ile throughput loss should be less than 5%. - Option 2: The following procedure shall be performed to obtain the performance metric: 1) Run the UL to UL coexistence study, assuming UE max TX power of both systems are 23 dBm. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. The victim system performance degradation results in this scenario are used as the baseline. 2) Run the UL to UL coexistence study, assuming 32 dBm power class UE is deployed in interfering system only and obtain the victim system performance degradation results. 3) Compare the victim system performance degradation obtaining in steps 1) and 2), choose the 32 dBm UE ACLR value so that the victim system performance degradation due to 32 dBm UE in 2) is similar to 1).

Proposal 2: RAN4 can collect the coexistence results in next February by keeping two options of performance metrics.

P2-1: for option 2, RAN4 can discuss whether the baseline UE MOP for both victim and aggressor systems are assumed as 26dBm instead of 23dBm.

- Pmax other than 32dBm

For 7GHz, RAN4 only assumed 23dBm UE maximum output power to do the coexistence studies captured into the TR 38.921. In this WI, it’s encouraged for companies to provide the coexistence results for 7GHz assuming 26dBm and 29dBm together.

Proposal 3: it’s encouraged for companies to provide the coexistence results for 7GHz assuming 26dBm and 29dBm together.

- New TR

According to the WID [1], there is no new TR for recording the coexistence studies. Thus, it’s suggested to add one new TR for recording the coexistence studies. Apart from that, there are two objectives for study.

Proposal 4: it’s suggested to add one new TR for recording the coexistence studies and some technical outcomes for SI objectives.

- ISD

In the last meeting, RAN4 reached the following WF for ISD.

| Inter-site distance | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies) |
| --- | --- |

In the TR 37.829, the following ISDs are assumed for different frequency ranges.

| Environment | ISD (km), 2.6Ghz | ISD (km), 3.5GHz | ISD (km), 1.9GHz | ISD (km), 1.75GHz |
| --- | --- | --- | --- | --- |
| Urban | .75 | .5 | 2 | 2 |
| Suburban | 2.8 | 1 | 5.1 | 5.1 |
| Rural | 6 | 2 | 9.3 | 9.3 |
| Rural | 8 | 5 |  |  |

In the TR 38.921, the following ISDs are assumed for different frequency ranges.

| Inter-site distance (7GHz and 10GHz) | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| --- | --- | --- |

It’s observed that there is no ISD 600m assumption for 7GHz in TR 38.921.

Proposal 5: RAN4 need to further clarify the accurate ISD assumptions for different frequency ranges (2.6GHz, 3.5GHz, 4.9GHz and 7GHz) in this meeting.

3 Summary

Proposal 1: According to the formula (CLx-ile=Pmax – Targeted SNR – BS thermal Noise), the following CLx-ile can be assumed for 32dBm Pmax for different BS NF.

CLx-ile = 116dB for below 6 GHz

CLx-ile = 115dB for 7 GHz

Proposal 2: RAN4 can collect the coexistence results in next February by keeping two options of performance metrics.

P2-1: for option 2, RAN4 can discuss whether the baseline UE MOP for both victim and aggressor systems are assumed as 26dBm instead of 23dBm.

Proposal 3: it’s encouraged for companies to provide the coexistence results for 7GHz assuming 26dBm and 29dBm together in this WI.

Proposal 4: it’s suggested to add one new TR for recording the coexistence studies and some technical outcomes for SI objectives.

Proposal 5: RAN4 need to further clarify the accurate ISD assumptions for different frequency ranges (2.6GHz, 3.5GHz, 4.9GHz and 7GHz) in this meeting.

Reference

[1] R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence”, Huawei, HiSilicon

[2] RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (Vivo)

[2] TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”