3GPP TSG RAN WG4 Meeting #117		R4-2520092

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on R20 low bands CA via switching

Agenda Item:	7.8.3

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN4#116bis meeting, a WF [2] was approved for this topic. A lot of issues and discussion points are identified.

Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states- UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information Issue 3: Open issues related to the UE reference architecture - Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3 - Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3 - Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3 - Whether confirmation by RAN1 is needed on any assumptions made by RAN4 - How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests Issue 7: Possible co-existence issues - The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis) - Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation - It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item Scenario; Type of interference; CA_n12-n29; CA_n28-n67. UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation); Co-channel; UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)). Blocking; Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out. BS to UE / UE to BS; Given that co-location is assumed, all those wouldn’t have an issue.. BS to BS; Co-channel interference and blocking; Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out.. Issue 8: Impact on RX requirements - Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

In this paper, we’d like to share our views on these aspects for this WI.

2 Discussion

2.1 RF architecture

1) CA_n12A-n29A

For CA_n12A-n29A, the reference RF architecture is illustrated below according to the WF [2]. That means one DL band filter (717MHz ~ 746MHz) will cover both DL bands n12 and n29.

One open issue is whether only one LO/mixer is shared for both DL bands n12 and n29 or two LOs/Mixers are used for both DL bands n12 and n29 respectively. This issue is similar to intra-band non-collocated CA or DL CA_n20A-n28A with the DL shared filter for both DL band n28A and n20.

Figure 1 the reference RF architecture for CA_n12-n29

If RAN4 allow Type 1 UE for UE implementation, a new UE capability [Inter-band_NRCA_SharedRxChain] which is similar to the intra-band non-collocated CA is needed. Because RAN4 assume separated Rx Chains for DL concurrent inter-band NR CA.

For CA_n28A-n67A, two possible implementations are proposed. In current industry, the band n67 specific DL filter is not widely implemented. Generally, band n67 shared the same DL filters with DL band n12 and n14, i.e. one filter cover 729MHz ~ 768MHz.

Figure 2 the reference RF architecture for CA_n28-n67 with n28 DL carrier located in 768MHz~788MHz

Figure 3 the reference RF architecture for CA_n28-n67 with n28 DL carrier located in 758MHz~768MHz

Observation 1: generally, RAN4 assume separated Rx Chains for DL concurrent inter-band NR CA.

Proposal 1: for both CA_n12-n29 and CA_n28-n67, if RAN4 allow Type 1 UE (two DL concurrent inter-band carriers fully shared one Rx chain) for UE implementation, a new UE capability [Inter-band_NRCA_SharedRxChain] which is similar to the intra-band non-collocated CA is needed.

Proposal 2: for CA_n28-n67, the reference RF architectures in figure 2 and figure 3 can be considered for different DL n28 frequency location.

2.2 Switching time mask

Figure 4 The whole picture on switching time mask for RTDS2P ≤ 0

Figure 5 The whole picture on switching time mask for RTDS2P > 0

The whole pictures on the switching time mask between UL/DL FDD band and Dual-DL FDD+SDL bands are shown in the figure 4 and 5.

It’s RAN4’s common understanding that the switching gaps in both FDD DL band and SDL band are needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. That means UE can’t maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band.

Proposal 3: it’s RAN4’s common understanding that the switching gaps in both FDD DL band and SDL band are needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. That means UE can’t maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band.

Based on the proposal 3, we can derive the calculation of switching gap for the following scenarios.

- From UL/DL FDD band to Dual-DL FDD+SDL bands:

FFS

- From Dual-DL FDD+SDL bands to UL/DL FDD band:

Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ l * RTDS2P + FDD UL leading transient period ) / (1/14*103) ]

UL transient period is assumed as 10us. Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. If RTDS2P ≥ 0, then l = 1; if RTDS2P < 0, then l = 0.

Proposal 4-1: RAN4 consider to introduce the following calculation of switching gaps for the switching scenario from Dual-DL FDD+SDL bands to UL/DL FDD band.

Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ l * RTDS2P + FDD UL leading transient period ) / (1/14*103) ] UL transient period is assumed as 10us. Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. If RTDS2P ≥ 0, then l = 1; if RTDS2P < 0, then l = 0.

Proposal 4-2: FFS on the switching gaps calculation of switching scenario from UL/DL FDD band to Dual-DL FDD+SDL bands.

2.3 REFSENS requirements and DL RMC

As a new switching pattern between the PCell and the SCell will be indicated by network IE LBCA-SwitchingPattern referring to clause 24 of TS 38.213, current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching. That means current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for this case.

The clause 7.3A.2.3 of TS 38.101-1: 7.3A.2.3 Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6. For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2. The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 CA intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

Observation 2: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching.

Apart from REFSENS requirements, it’s very important to verify the DL receiving performance for UE before and/or after the Switching gap. Alternatively, RAN4 can design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Observation 3: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

In order to verify the DL REFSENS requirements and DL receiving performance for UE before and/or after the Switching gap, it’s proposed to schedule the DL switch-from slots and switch-to slots for the DL RMC.

Considering the worst case and simplifying the specification, it’s proposed to configure two symbols PDSCH mapping Type B for DL RMC referring to the table 5.1.2.1-1 of TS 38.214. It’s better to configure the same length of symbols in all the DL switch-from slots and switch-to slots.

Table 5.1.2.1-1: Valid S and L combinations PDSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; {0,1,2,3} (Note 1); {3,…,14}; {3,…,14}; {0,1,2,3} (Note 1); {3,…,12}; {3,…,12}. Type B; {0,…,12}; {2,4,7}; {2,…,14}; {0,…,10}; {2,4,6}; {2,…,12}. Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3.

For the LBCA-SwitchingPattern, it’s proposed to design the DL RMC as ‘00011 00110 01100 11001 11100 11001 10011 00110’ in order to increase DL samples (18 DL Slots for PCell and 18 DL Slots for SCell). Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. In addition, one symbol PDCCH is scheduled at the first symbol in each DL slot except for SSB slots. The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown below.

Slot Index; Pattern; UL FDD band; DL FDD band; SDL band; Frame Index. 0; 0; PUCCH/PUSCH; SSB; NA; i. 1; 0; PUCCH/PUSCH; SSB; NA. 2; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 3; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 4; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 5; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 6; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 7; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 8; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 9; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 10; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA; i+1. 11; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 12; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 13; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 14; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 15; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 16; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 17; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 18; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 19; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 20; 1; NA; SSB; SSB; i+2. 21; 1; NA; SSB; SSB. 22; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 23; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 24; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 25; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 26; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 27; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 28; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 29; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 30; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH; i+3. 31; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 32; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 33; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 34; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 35; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 36; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 37; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 38; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 39; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA.

Figure 6 The DL RMC switching pattern for R20 LB CA via switching

Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

5-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

5-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

5-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

5-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 3.

2.4 Relationship and/or fallback behavior between R19 and R20 low band switching

In the last meeting, one open issue was identified.

Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

Based on our analysis, there are two alternatives at least.

Alternative 1: There is no fallback behaviour between R19 and R20 low band switching. Network can use the same IE (LowBandCA-Switching-r19 -> switchingPattern-r19 / gapDurationPCelltoSCell-r19 / gapDurationSCelltoPCell-r19) for both R19 and R20 low band switching. But UE need an additional UE capability to indicate whether it support R20 LB CA switching or not.

Alternative 2: There is a fallback behaviour between R19 and R20 low band switching. If UE indicate the support of R20 LB CA switching. That means UE also need to support R19 LB CA switching. Thus, additional IE is need for network to indicate whether the switching pattern is for R19 LB CA switching or R20 LB CA switching.

Proposal 6: RAN4 can further discuss the relationship between R19 and R20 low band switching based on the following two alternatives.

Alternative 1: There is no fallback behaviour between R19 and R20 low band switching. Network can use the same IE (LowBandCA-Switching-r19 -> switchingPattern-r19 / gapDurationPCelltoSCell-r19 / gapDurationSCelltoPCell-r19) for both R19 and R20 low band switching. But UE need an additional UE capability to indicate whether it support R20 LB CA switching or not.

Alternative 2: There is a fallback behaviour between R19 and R20 low band switching. If UE indicate the support of R20 LB CA switching. That means UE also need to support R19 LB CA switching. Thus, additional IE is need for network to indicate whether the switching pattern is for R19 LB CA switching or R20 LB CA switching.

3 Summary

- RF architecture

Observation 1: generally, RAN4 assume separated Rx Chains for DL concurrent inter-band NR CA.

Proposal 1: for both CA_n12-n29 and CA_n28-n67, if RAN4 allow Type 1 UE (two DL concurrent inter-band carriers fully shared one Rx chain) for UE implementation, a new UE capability [Inter-band_NRCA_SharedRxChain] which is similar to the intra-band non-collocated CA is needed.

Proposal 2: for CA_n28-n67, the reference RF architectures in figure 2 and figure 3 can be considered for different DL n28 frequency location.

Figure 2 the reference RF architecture for CA_n28-n67 with n28 DL carrier located in 768MHz~788MHz

Figure 3 the reference RF architecture for CA_n28-n67 with n28 DL carrier located in 758MHz~768MHz

- Switching time mask

Figure 4 The whole picture on switching time mask for RTDS2P ≤ 0

Figure 5 The whole picture on switching time mask for RTDS2P > 0

Proposal 3: it’s RAN4’s common understanding that the switching gaps in both FDD DL band and SDL band are needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. That means UE can’t maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band.

Proposal 4-1: RAN4 consider to introduce the following calculation of switching gaps for the switching scenario from Dual-DL FDD+SDL bands to UL/DL FDD band.

Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ l * RTDS2P + FDD UL leading transient period ) / (1/14*103) ] UL transient period is assumed as 10us. Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. If RTDS2P ≥ 0, then l = 1; if RTDS2P < 0, then l = 0.

Proposal 4-2: FFS on the switching gaps calculation of switching scenario from UL/DL FDD band to Dual-DL FDD+SDL bands.

- REFSENS requirements and DL RMC

Observation 2: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching.

Observation 3: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

5-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

5-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

5-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

5-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 3.

- Relationship and/or fallback behavior between R19 and R20 low band switching

Proposal 6: RAN4 can further discuss the relationship between R19 and R20 low band switching based on the following two alternatives.

Alternative 1: There is no fallback behaviour between R19 and R20 low band switching. Network can use the same IE (LowBandCA-Switching-r19 -> switchingPattern-r19 / gapDurationPCelltoSCell-r19 / gapDurationSCelltoPCell-r19) for both R19 and R20 low band switching. But UE need an additional UE capability to indicate whether it support R20 LB CA switching or not.

Alternative 2: There is a fallback behaviour between R19 and R20 low band switching. If UE indicate the support of R20 LB CA switching. That means UE also need to support R19 LB CA switching. Thus, additional IE is need for network to indicate whether the switching pattern is for R19 LB CA switching or R20 LB CA switching.

Reference

[1] RP-252953, “New WID on enhancement of low NR band carrier aggregation via switching”, Moderator (Huawei)

[2] R4-2515156, “Way Forward for [116bis][320] NR_low_band_CA_switching”, Moderator (Apple)

Annex

In the RAN#109 meeting, a new WID [1] was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.