3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520099

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	Further considerations on coexistence between A-IoT and NR in outdoor scenarios

Agenda item:	7.11.3

Document for:	Approval

- Introduction

In RAN4#116bis meeting, some agreements on co-existence assumptions were reached in [1], but there are still some issues need to be further discussed, in this contribution, we give our analysis on the open issues.

- Discussion

2.1 Output power for A-IoT reader for in-band operation

For in-band operation, it is very probably A-IoT will share hardware with NR, which means the power should also be shared between A-IoT and NR. If we assumed 10MHz BS with 46dBm total output power, the output power for 1RB is 29dBm. RB power boosting requirement should be discussed in WI stage and depend on the need, but we can still assume A-IoT has the same RB power boosting capability as NB-IoT and NR which is +6dB, then the maximum output power for 1RB can be reach to 35dBm, which is between the two values 38dBm and 33dBm of RAN1 link level assumption.

In last RAN1 meeting, some companies submit the link level evaluation and the observations are quite similar that R2D coverage is sufficient to meet the coverage requirement in WID, but D2R coverage is not enough. Therefore, we think output power is not a big issue in the evaluation, we can assume either 29dBm or 35dBm for 1RB. But for co-existence study, as we focus on the interference issue, so we propose to use 35dBm for 1RB in the simulation for interference statistics for in-band operation.

Proposal 1: It is proposed to use 35dBm for 1RB in co-existence study for interference statistics for in-band operation.

2.2 SNR from LLS

In [1], for performance metric used in coexistence simulation, there are two options listed as below.

Issue 3-1: performance metric Agreement: - For calibration purpose, consider coupling loss, SINR with ACI, and SINR without ACI - For formal coexistence simulation, - Option 1: Use SINR degradation of [1dB] - Option 2: Use SNR from LLS

For option 2, SNR from link level simulation needs to be considered in co-existence simulation. Here we provide our company LLS results as below which are also align with our RAN1 contribution [2].

The link performances simulation assumption is as in Table 1 and the results are shown in Figure 1.

Table 1 Link performance simulation assumptions

| Parameters |  | Assumptions |
| --- | --- | --- |
| R2D/D2R common parameters | Carrier frequency | 900MHz |
|  | Information bit length | 96bits |
|  | Sampling frequency | 2.88MHz |
|  | SFO | - Device 2b: 1000ppm - Device C: 10ppm |
|  | CFO | - Device 2b: 100ppm - Device C: 10ppm |
|  | Channel | - Channel model: TDL-C - Delay spread: 300ns - Velocity: 3km/h |
| R2D specific parameters | Modulation | OOK-4, M={2, 6, 12, 24} |
|  | Line code | Manchester |
|  | Antenna | 2T1R |
| D2R specific parameters | Modulation | OOK, BPSK, MSK |
|  | Bit duration | 16.67us |
|  | Channel coding | TBCC, code rate = 1/3 |
|  | Antenna | 1T2R |

Figure 1: Link performance of the R2D link (left) and D2R link (right)

As we can observed from above results, if we use BLER of 1% as criteria, required SNR value is not low. But how to capture this SNR value in co-existence study needs further consideration. Below are some analysis from our views:

First, SNR in LLS is mainly evaluate the coverage, if RAN1 observe that the coverage objective cannot be met in SI stage, they will enhanced it through technical methods in WI stage, it’s not a RAN4 issue. If the simulation results of intra-system SINR show a lot of outage devices, it should not worried in RAN4, we can send LS to RAN1 inform the observations.

Second, RAN4’s responsibility is to work on the interference caused by adjacent channel and define the proper ACIR requirements. From above LLS simulation results, we can observe that the BLER will not abruptly degrade if the SINR degrade eventually.

Proposal 2: It is proposed RAN4 only focus on option 1 for performance metric in co-existence study. If the simulation results of intra-system SINR show a lot of outage devices, RAN4 can send LS to RAN1 inform the observations.

2.3 Frequency reuse

As the frequency resource is very precious in outdoor scenario, only if the initial simulation results of intra-system SINR curve show unacceptable performance degradation in section 2.2, we can accept frequency reuse for AIoT. Below figure gives the frequency configuration of frequency reuse equal to 3.

RB1

RB2

RB3

RB1

RB2

RB3

RB1

RB2

RB3

RB1

RB2

RB3

Figure 2 Illustration of frequency reuse 3

Proposal 3: Only if the initial simulation results of intra-system SINR show unacceptable performance degradation, we can accept frequency reuse for A-IoT as illustrated in Figure 2.

2.4 ACIR assumption

ACIR assumption should be different for standalone and in-band operation. For in-band operation, it is not exactly adjacent channel, it should be adjacent RB with or without guard RBs. The below table gives the initial assumptions:

Table 2 ACIR assumptions

| Scenario | Parameters | Values for evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NR BS | A-IoT reader | NR UE | A-IoT device |
| Standalone | ACLR | 45dBc |  | 30dBc | 30dBc |
|  | ACS | 46dBc |  | 33dBc | 33dBc |
| In-band operation | ACLR (RB level) | 30dBc |  | 30dBc | [20dBc] |
|  | ACS (RB level) | 20dBc (ICS) |  | 33dBc | [20dBc] |

Table 3 ACIR model for standalone

Proposal 4: ACIR assumption should be different for standalone and in-band operation. Table1 gives the initial assumptions.

- Conclusion

This contribution provides proposals as below:

Proposal 1: It is proposed to use 35dBm for 1RB in co-existence study for interference statistics for in-band operation.

Proposal 2: It is proposed RAN4 only focus on option 1 for performance metric in co-existence study. If the simulation results of intra-system SINR curve shows a lot of outage devices, RAN4 can send LS to RAN1 inform the observations.

Proposal 3: Only if the initial simulation results of intra-system SINR curve show unacceptable performance degradation, we can accept frequency reuse for A-IoT as illustrated in figure 1.

Proposal 4: ACIR assumption should be different for standalone and in-band operation. Table1 gives the initial assumptions.

- References

- R4-2515152 Way Forward for [116bis][321] Study of A-IoT for outdoor

- R1-2508588, Evaluation methodology for A-IoT outdoor deployment scenarios, CATT