3GPP TSG-RAN WG4 Meeting #117	R4-2520115

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on performance requirements for other RRM requirements

Source: 	CATT

Agenda item:	6.19.3.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the Way Forward for [116bis][327] Rel-19 RRM performance_Part3 including NR_NTN_Ph3 performance requirement was approved [1]. In this contribution, we further discuss and provide our views on the test scope of SMTC enhancement for the UE capable of coverage enhancement.

- Discussion

- Test scope of SMTC enhancement for UE capable of coverage enhancement

In last meeting, RAN4 discussed the test scope of SMTC enhancement for the UE capable of coverage enhancement. For supporting two SMTC periodicities with different offsets in one frequency layer, RAN4 agreed to only introduce new test case for L3 event triggered reporting. The agreement is copied as follows [1]:

< Agreement >: - For supporting two SMTC periodicities with different offsets in one frequency layer, - RAN4 to only introduce new test case for L3 event triggered reporting. - For more than 4 SMTCs per frequency (i.e. 6) in RRC Idle/Inactive mode, - Wait for conclusion on core part discussion before discussing whether to define new test cases on cell reselection.

For more than 4 SMTCs per frequency in RRC Idle/Inactive mode, RAN4 is still discussing whether to capture the specific SMTC selection rule in the spec when the number of configuring SMTCs for intra-frequency carrier does exceed the UE capability.

In our view, even though the specific SMTC selection rule is agreed to captured in the spec eventually, it will be only about something like ‘SMTC selection based on UE position and associated location for each SMTC’ as RAN2 spec, it is still not clear for UE specific selection behaviour, and it will also up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode. RAN2 agreement is copied as follows [2]:

Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

According to the above, we prefer not to define the related test case for this SMTC enhancement in RRC Idle/Inactive mode.

Observation 1: Even though SMTC selection is based on UE position and associated location for each SMTC, it is still not clear for UE specific selection behaviour, and it will also up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

Proposal 1: RAN4 not to define the related test case for this SMTC enhancement in RRC Idle/Inactive mode.

- Conclusion

This document discussed the issues for performance requirements for SMTC enhancement for the UE capable of coverage enhancement and presented following observation and proposal:

Observation 1: Even though SMTC selection is based on UE position and associated location for each SMTC, it is still not clear for UE specific selection behaviour, and it will also up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

Proposal 1: RAN4 not to define the related test case for this SMTC enhancement in RRC Idle/Inactive mode.

- Reference

[1]	R4-2515056, Way Forward for [116bis][327] Rel-19 RRM performance_Part3, Moderator (CATT)

[2]      R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, RAN2