3GPP TSG-RAN WG4 Meeting # 117                                                              R4-2520131

Dallas, USA, Nov. 17-21, 2025

Title: 	draft TP for TS 38.195 to introduce A-IoT CW transmitter characteristics

Source: 	CATT

Agenda item:	6.14.2

Document for:	Approval

- Introduction

This contribution provides a draft TP to TS 38.195 for A-IoT CW transmitter characteristics. The agreements of in the WF [2] are included in the TP.

Reference

[1] TS 38.195, V0.0.1

[2] R4-2515058, “WF for [116bis][314] A-IoT_BSCW”, Huawei, RAN4#116bis

[3] TS 38.194

TP for TS 38.195

8	A-IoT CW transmitter characteristics

8.1	General

### 8.1.1	CW node

General test conditions for conducted transmitter tests of CW node are given in clause 8, including interpretation of measurement results and configurations for testing.

8.2	CW Output power

### 8.2.1	Definition and applicability

The conducted CW node output power requirements are specified at the antenna connector.

The rated output power of the CW node shall be less than or equal to +33 dBm.

### 8.2.2	Minimum requirement

The minimum requirement applies at the connector supporting transmission in the operating band.

The minimum requirement for CW node is defined in TS 38.194 [3], clause 8.2.2.

### 8.2.3	Test purpose

The test purpose is to verify the accuracy of the maximum output power across the frequency range and under normal and extreme conditions.

### 8.2.4	Method of test

#### 8.2.4.1	Initial conditions

Test environment:

-	Normal, see annex [B.2],

-	Extreme, see annexes [B.3 and B.5].

RF frequency point to be tested for single-tone signal: B, M and T; see clause [4.7].

Under extreme test environment, it is sufficient to test on one RF frequency point position, and with the test configuration defined in clauses [4.5]. Testing shall be performed under extreme power supply conditions, as defined in [Annex B.5].

NOTE:	Tests under extreme power supply conditions also test extreme temperatures.

#### 8.2.4.2	Procedure

- Connect the power measuring equipment to the connector under test as shown in [annex D.1.1] for CW node.

2)	For single tone, set the connector under test to transmit according to the applicable test configuration in clause [4.5] using the corresponding test models or set of test signal  in clause [4.7] at rated output power Prated for CW node.

3)	Measure the maximum output power (Pmax) at the connector under test.

### 8.2.5	Test requirement

For the connector under test, the power measured in clause 8.2.4.2 in step 3 shall remain within the values provided in table 8.2.5-1 for normal and extreme test environments, relative to the manufacturer's declared Prated for CW node:

Table 8.2.5-1: Test requirement for conducted CW node output power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| CW node | f ≤ 3.0 GHz: ± 2.7 dB | f ≤ 3.0 GHz: ± 3.2 dB |
| NOTE 1: Apply for CW node operates in licensed spectrum only. |  |  |

8.3	Frequency error

#### 8.3.1	Definition and applicability

Frequency error is the measure of the difference between the actual CW node transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

For CW node, this requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 8.3.2	Minimum Requirement

The minimum requirement is in TS 38.194 [3], clause 8.3.2.

#### 8.3.3	Test purpose

The test purpose is to verify that frequency error is within the limit specified by the minimum requirement.

#### 8.3.4	Method of test

8.3.4.1	Initial conditions

Test environment: Normal; see [annex B.2].

RF frequency point to be tested for single-tone signal: M; see clause [4.7].

##### 8.3.4.2	Procedure

1)	Connect the connector under test to measurement equipment as shown in [annex D.1.1] for CW node.

2)	For a connector declared to be capable of single-tone operation only ([D.16]), set the representative connector under test to transmit according to the applicable test configuration in clause [4.5] at rated output power (Prated, [D.21]).

3)	Measure the frequency error at the connector under test.

#### 8.3.5	Test Requirements

Table 8.3.5-1: Frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| CW node | ±(0.1 ppm + 12 Hz) |

8.4	Unwanted emission

### 8.4.1 Phase noise

#### 8.4.1.1	Definition and applicability

The phase noise is the unwanted emissions outside the centre frequency of carrier wave resulting from random fluctuations in the phase of signal in the transmitter but excluding spurious emissions.

#### 8.4.1.2	Minimum requirement

The minimum requirement applies the connector supporting transmission in the operating band.

The minimum requirement for CW node is defined in TS 38.194 [3], clause 8.5.2.2.

#### 8.4.1.3	Test purpose

The test purpose is to verify that phase noise is within the limit specified by the minimum requirement.

#### 8.4.1.4	Method of test

##### 8.4.1.4.1	Initial conditions

Test environment: Normal; see [annex B.2].

RF frequency point to be tested for single-tone signal: M; see clause [4.7].

##### 8.4.1.4.2	Procedure

1)	Connect the connector under test to measurement equipment as shown in [annex D.1.1] for CW node.

2)	For a connector declared to be capable of single-tone operation only ([D.16]), set the representative connector under test to transmit according to the applicable test configuration in clause [4.5] at rated output power (Prated, [D.21]).

3)	Measure the phase noise at the connector under test.

#### 8.4.1.5	Test requirements

The phase noise power of CW transmitter shall not exceed the levels specified in Table 8.4.1.5-1.

Table 8.4.1.5-1: CW phase noise emission limit

| Δf (kHz) | Phase noise emission limit (dBc/Hz) |
| --- | --- |
| ± 7.5 | -97 |
| ± 120 | -102 |

### 8.4.2 	Operating band unwanted emissions

#### 8.4.2.1	Definition and applicability

Unless otherwise stated, the operating band unwanted emission (OBUE) limits in FR1 are defined from 10 MHz below the lowest frequency of each supported uplink operating band up to 10 MHz above the highest frequency of each supported uplink operating band. Basic limits are specified in the tables below, where:

-	f is the separation between the assigned transmission frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

#### 8.4.2.2	Minimum requirement

The minimum requirement applies the connector supporting transmission in the operating band.

The minimum requirement for CW node is defined in TS 38.194 [3], clause 8.5.3.2.

#### 8.4.2.3	Test purpose

This test measures the emissions close to the assigned transmission frequency of the wanted signal, while the transmitter is in operation.

#### 8.4.2.4	Method of test

##### 8.4.2.4.1	Initial conditions

Test environment: Normal; see [annex B.2].

RF frequency point to be tested for single-tone signal: B, M and T; see clause [4.7].

##### 8.4.2.4.2	Procedure

1)	Connect the connector under test to measurement equipment as shown in [annex D.1.1] for CW node.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in [Table 4.1.2.2-1].

2)	For a connector declared to be capable of single-tone operation only, set the representative connector under test to transmit according to the applicable test configuration in clause [4.5] at output power Prated for CW node. Test signal set-up shall be according to [TBD].

3)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth.

4)	Repeat the test for the remaining test cases, with the test signal set-up according to [TBD].

#### 8.4.2.5	Test requirements

##### 8.4.2.5.1	General requirements

For CW node operating in Band n8, basic limits are specified in tables 8.4.2.5.1-1.

Table 8.4.2.5.1-1: CW node Operating band unwanted emissions

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~10000 | -23 | 1 MHz |

8.4.3	Transmitter spurious emissions

8.4.3.1	Definition and applicability

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of each supported uplink operating band, up to 10 MHz above the highest frequency of each supported uplink operating band..

Unless otherwise stated, all requirements are measured as mean power (RMS).

8.4.3.2	Minimum requirement

The minimum requirement applies connector supporting transmission in the operating band.

The minimum requirement for CW node is defined in TS 38.194 [3], clause 8.5.4.2.

8.4.3.3	Test purpose

This test measures conducted spurious emissions while the transmitter is in operation.

8.4.3.4	Method of test

8.4.3.4.1	Initial conditions

Test environment: Normal; see [annex B.2].

RF frequency point to be tested for single-tone signal:

-	B when testing the spurious emissions below FUL_low - 10 MHz,

-	T when testing the spurious emissions above FUL_high + 10 MHz; see clause [4.7].

8.4.3.4.2	Procedure

1)	Connect the connector under test to measurement equipment as shown in [annex D.1.1] for CW node.

2)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 8.4.3.5.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table [4.1.2.2-1].

3)	For a connector declared to be capable of single-tone operation only ([D.16]), set the representative connector under test to transmit according to the applicable test configuration in clause [4.5] at rated output power (Prated, D.21). Test signal set-up shall be according to [TBD].

4)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8.4.3.5	Test requirements

8.4.3.5.1	Basic limits

8.4.3.5.1.1 Tx spurious emissions

The limits of  table 8.4.3.5.1.1-1 (Category B limits) shall apply. The application of Category B limits shall be the same as for operating band unwanted emissions in clause 8.4.2, and as declared by the manufacturer ([D.4]).

Table 8.4.3.5.1.1-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |