3GPP RAN WG4 Meeting #117	R4-2520152

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on impacts of AIML measurement event prediction on RRM core requirements

Source: 	CATT

Agenda item:	7.12.4

Document for:	Discussion

Introduction

In RAN#109 meeting, revised WID of Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved with the following objectives [1]:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In RAN4#116bis meeting, RAN4 started to discuss the RRM impacts of AI/ML measurement event prediction use case and agreements are captured in the way forward [2]. In this contribution, we will continue to analyse the potential impacts and present our views.

Discussion

Scope and principle of measurement event prediction

For the deployment scenario, as we discussed in our companion contribution for RRM measurement prediction [3] submitted to RAN4#116bis, RAN4 need to consider the following scenarios for AI/ML measurement event prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2:

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only.

In addition, according to the WID [1], RAN4 need to discuss indirect measurement event prediction use case first and further check whether direct event prediction can be supported in RAN#112. Furthermore, RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction. In other words, RAN4 should try to reuse the requirements defined for RRM measurement prediction as much as possible, in which case requirements for RRM measurement prediction should be discussed first.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]:

- Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction

  - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction

- For direct event prediction, study feasibility of core requirement in Q2 2026.

- Specify LCM-related core requirements to support the LCM functionality management , if any

Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

Proposal 1: RAN4 consider the following scope for AI/ML measurement event prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2;

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only;

- Indirect measurement event prediction considered only.

Proposal 2: RAN4 try to reuse the requirements defined for RRM measurement prediction as much as possible.

Proposal 3: RAN4 to discuss the requirements for RRM measurement prediction first.

Impact on core requirements

Based on the discussions during SI phase, it is clear that measurement event prediction use case impacts the conditional handover delay requirements. For example, in RAN2#129 meeting [4], RAN2 provides potential objectives for some measurement event prediction mechanisms:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

According to the both objectives above, some components in current conditional handover delay requirements can be updated or removed, e.g., Tprocessing and/or Tmeasure. But before RAN4 start to discuss the detailed impacts on RRM core requirements, more RAN2 design is expected.

Observation 1: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure.

Proposal 4: RAN4 to discuss the detailed impacts of measurement event prediction on RRM core requirements after RAN2 make enough progress.

Conclusions

This contribution discussed the impacts of AI/ML measurement event prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: RAN4 consider the following scope for AI/ML measurement event prediction use case:

- Intra-frequency temporal domain prediction in both FR1 and FR2;

- Inter-frequency (frequency domain) for FR1 to FR1 with co-located deployment;

- L3 cell level prediction considered only;

- Indirect measurement event prediction considered only.

Proposal 2: RAN4 try to reuse the requirements defined for RRM measurement prediction as much as possible.

Proposal 3: RAN4 to discuss the requirements for RRM measurement prediction first.

Observation 1: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure.

Proposal 4: RAN4 to discuss the detailed impacts of measurement event prediction on RRM core requirements after RAN2 make enough progress.

References

[1] RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital, RAN#109.

[2] R4-2514827, WF on NR_AIML_Mob_RRM, OPPO, RAN4#116bis.

[3] R4-2513227, Discussion on impacts of AIML RRM measurement prediction on RRM core requirements, CATT, RAN4#116bis.

[4] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.

[5] R4-2513532, Topic summary for [116bis][226] NR_AIML_Mob_RRM, Moderator, RAN4#116bis.