3GPP TSG-RAN WG4 Meeting #117	                                                                                            R4-2520160

Dallas, US, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for IoT NTN support PC 1.5

Agenda item:	7.6.3

Document for:	Approval

- Introduction

In RAN4 #116-bis meeting, a WF [1] for NTN and IoT NTN phase 2 PC 1.5 IoT NTN HPUE has been approved. However, there remain some issues that require further discussion. This paper shares some views on RF requirements for IoT NTN support PC 1.5.

- Discussion

In RAN4 #116-bis meeting, the WF for IoT NTN support PC 1.5 has been approved, as shown below:

### 2.2.1 Sub-topic 2-1: General issues #### Issue 2-1-1: Scope clarification, feasibility Agreement: - The feasibility of PC 1.5 single Tx NB-IoT NTN UE in both handhelds and non-handhelds is confirmed. - Both category NB1 and NB2 are in the scope of this WI. - The eMTC-based IoT-NTN is out of scope of this release. #### Issue 2-1-2: Work plan on the TR Agreement: - Suggest to update the WID in the next RAN meeting to include a TR for NTN HPUE #### Issue 2-1-6: Capability signalling for PC1.5 NB-IoT NTN UE Agreement: - The signaling to indicate support for PC1.5 is needed to be added to LTE RRC signalling with preference on early implementation. #### Issue 2-1-7: Power boosting for PC1.5 NB-IoT NTN UE Agreement: - Power boosting is not considered for PC1.5 NB-IoT NTN UE. ### 2.2.2 Sub-topic 2-2: Tx requirements for PC1.5 NB-IoT NTN UE #### Issue 2-2-1: The tolerance of maximum output power Agreement: - Apply +2/-3 dB tolerance for the 29 dBm NB-IoT based NTN PC1.5 power class. #### Issue 2-2-2: SAR solution for PC1.5 NB-IoT NTN UE Agreement: - Re-use existing P-MPR method for PC1.5 NB-IoT NTN UE. #### Issue 2-2-3: UE RF front end assumptions and related accommodations WayFoward: - FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements) - The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118. #### Issue 2-2-4: ACLR requirement for PC1.5 NB-IoT NTN UE Agreement: - Apply UTRA ACLR value of 37 dBc for PC1.5 single Tx NB-IoT NTN UE. #### Issue 2-2-5: SEM requirement for PC1.5 NB-IoT NTN UE Agreement: - Reuse the existing SEM requirement in TS 36.102 clause 6.5B.3.2 - Apply the following for the additional SEM requirement for NS_02N (for band 255) - 4 dBm when ΔfOOB is ± 0.09–0.28 MHz - -6 dBm when ΔfOOB is ± 0.28–0.85 MHz - -13 dBm when ΔfOOB is larger than ± 0.85 MHz #### Issue 2-2-6: MPR requirements for PC1.5 NB-IoT NTN UE Agreement: - Take the following tables as starting point for the MPR for PC1.5 NB-IoT NTN UE (Based on MPR = maximum {0, PC1 MPR - 2dB}) Table 2-2-6-1 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone Modulation; π/2 BPSK, π/4 QPSK; π/2 BPSK, π/4 QPSK. SCS; 15 kHz; 3.75 kHz. Tone positions for 1 Tone allocation; 0-1; 2-9; 10-11; 0-5; 6-41; 42-47. MPR; ≤ [0] dB; [0] dB; ≤ [0] dB; ≤ [1.5] dB; [0] dB; ≤ [1.5] dB. Note 1: For single-tone allocation, the listed modulations and SCS are supported.. Table 2-2-6-2 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone Modulation; QPSK. SCS; 15 kHz. Tone positions for 3 Tones allocation; 0-2; 3-5 and 6-8; 9-11. MPR; ≤ [0] dB; ≤ [0] dB; ≤ [0] dB. Tone positions for 6 Tones allocation; 0-5 and 6-11. MPR; ≤ [0.5]dB; ≤ [0.5] dB. Tone positions for 12 Tones allocation; 0-11. MPR; ≤ [2] dB. Note 1: For multi-tone allocation, the listed modulation and SCS are supported.. - Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed - Option 1 - Calibration points for PC1.5 (based on NR PC2 PA): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss /or [5] dB - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc / or [31] dBc - CIM3: 60 dBc - NR ACLR: 31dBc - Option 2 - DFT-s-OFDM 1RE (Single Tone) - Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK] - [3] or [4] dB post PA loss - 0dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value #### Issue 2-2-7: A-MPR requirement for band 256 PC1.5 NB-IoT NTN UE Agreement: - Take the following as the starting point for the A-MPR for band 256 PC1.5 NB-IoT NTN UE with NS_24, depending on the final MPR value Table 2-2-7.1 Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5 Network Signalling value; Requirements (subclause); E-UTRA Band; A-MPR (dB). NS_24; 6.5B.4.4.3; 256; ≤ [8~8.5]. - Interested companies can provide further evaluation in the next RAN4 meeting based on the assumptions in issue 2-2-6, including whether the above A-MPR is applicable for all frequencies of band 256 or a subset of frequencies of band 256. - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value #### Issue 2-2-8: A-MPR requirement for band 255 PC1.5 NB-IoT NTN UE Agreement: - A-MPR is not needed for band 255 PC1.5 NB-IoT NTN UE with NS_02N - FFS on A-MPR evaluation for band 255 PC1.5 single Tx NB-IoT NTN UE with additional ETSI requirements associated with NS_10N and NS_14N ### 2.2.3 Sub-topic 2-3: Rx requirements for PC1.5 NB-IoT NTN UE #### Issue 2-3-1: RSD requirement Agreement: - No RSD requirement is needed for NB-IoT based NTN PC1.5 UE. #### Issue 2-3-2: Other Rx requirement Agreement: - Confirm there is no Rx requirement impact for PC1.5 NB-IoT based IoT-NTN.

During the previous meeting, the UE RF front end issues for PC 1.5 NTN IoT HPUE were discussed. For some specific UE implementation schemes, the UE may encounter some power handling challenges. Generally speaking, the protection and operational conditions for RF front end components may prove difficult to standardise from a specification perspective, as these aspects are more strongly tied to the deployment of the product. Meanwhile, if the UE seek to resolve implementation issues by adopting a network-based scheduling approach, this may not constitute a reliable option. This is because the scheduler of BS will typically perform relevant scheduling actions from the perspective of the entire network.

One possible approach is to find solutions from existing mechanisms that have proven reliable. For instance, certain existing signalling such as UAI could be leveraged to prevent UE overheating. Concurrently, exploring solutions from the RRM perspective represents another viable option: scheduling availability might be utilised to notify the network that the UE cannot be scheduled for UL transmission at specific times.

Observation 1: RAN4 may investigate whether existing mechanisms could be leveraged to resolve UE front end issues.

- Conclusion

This contribution provides our views on RF requirements for IoT-NTN PC 1.5 HPUE. The following proposals are concluded as follows:

Observation 1: RAN4 may investigate whether existing mechanisms could be leveraged to resolve UE front end issues.

- References

- R4-2515157, WF for [116bis][318] NR_IoT_NTN_Ph2, CHTTL, RAN4 #116-bis