3GPP TSG-RAN4 Meeting #117	R4-2520171

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | 0134 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.108, Correction on SAN OTA ACLR and OTA in-channel selectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct the SAN channel bandwidth of ACLR and characteristic of interfering signal of OTA ICS for SAN operating under Ku band |  |  |  |  |  |  |  |  |
| Summary of change: |  | To correct the SAN channel bandwidth of ACLR for SAN operating under Ku band To correct the characteristic of interfering signal of OTA ICS for SAN operating under Ku band |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SAN channel bandwidth of ACLR and characteristic of interfering signal of OTA ICS for SAN operating under Ku band will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.3, 10.9.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.181 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

9.7.3.2	Minimum requirement for SAN type 1-O operating below 10 GHz

The ACLR limit specified in tables 6.6.3.2-1 for SAN GEO class and 6.6.3.2-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 6.6.3.2 shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 6.6.3.2-1 and 6.6.3.2-2.

9.7.3.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

The ACLR limit specified in tables 9.7.3.2a-1 for SAN GEO class and 9.7.3.2a-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 9.7.3.2a shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 9.7.3.2a-1 and 9.7.3.2a-2.

Table 9.7.3.2a-1: SAN type 1-O ACLR limit for SAN GEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- | --- |
| n248, n247 | 10, 15, 20, 25, 35, 50, 70,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |
| NOTE 1: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |

Table 9.7.3.2a-2: SAN type 1-O ACLR limit for SAN LEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- | --- |
| n248, n247 | 10, 15, 20, 25, 35, 50, 70,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |
| NOTE 1: BWChannel and BWConfig are the SAN channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |

==============Next change==============

10.9.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA.

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For SAN type 1-O operating above 10GHz, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.9.2a-1. The characteristics of the interfering signal is further specified in annex C.

Table 10.9.2a-1: OTA in-channel selectivity requirement for SAN type 1-O operating above 10GHz

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 25, 35 | 15 | G-FR1-NTN-A1-1 | EISREFSENS_10M + ΔFR1_REFSENS | EISREFSENS_10M + 10 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 50 | 15 | G-FR1-NTN-A1-4 | EISREFSENS_10M + 7 + ΔFR1_REFSENS | EISREFSENS_10M + 17 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 10, 20, 25, 35 | 30 | G-FR1-NTN-A1-2 | EISREFSENS_10M + ΔFR1_REFSENS | EISREFSENS_10M + 10 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RB |
| 50, 70,100 | 30 | G-FR1-NTN-A1-5 | EISREFSENS_10M + 7 + ΔFR1_REFSENS | EISREFSENS_10M + 17 + ΔFR1_REFSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_10M = EISREFSENS_50M – 7, and EISREFSENS_50M is declared by the vendor as specified in clause 10.3.2a. |  |  |  |  |  |

==============End of change==============