3GPP TSG RAN WG4 Meeting #117		R4-2520188

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Further discussion on BS RF requirements for 6MHz channel bandwidth

Agenda Item:	7.1.5.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With respect to BS RF requirements for a 6 MHz channel bandwidth, significant progress was achieved at RAN4#116bis, with only a few remaining BS RF requirements requiring further discussion, i.e., ACS, NBB, Rx IMD, ICS [1]. These outstanding items are expected to be finalized during this meeting:

ACS Define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as: - Option 1: ±2.5125 MHz - Option 2: ±2.5050 MHz

NBB - Option 1: ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 - Option 2: ±(360+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

Rx IMD For intermodulation requirement: - CW offset: - Option 1: ±7.475 MHz - Option 2: ±7.52 MHz For narrowband intermodulation requirement: - CW offset and Offset for 5 MHz DFT-s-OFDM NR signal, 1 RB: - Option 1: ±425 kHz and ±1240 kHz - Option 2: ±630 kHz and ±1560 kHz

ICS For WA BS ICS requirement: - Interfering signal mean power (dBm) and type of interfering signal (RBs): - Option 1: -79.6 and 15 - Option 2: -80.6 and 12 - Option 3: -81.4 and 10

In this contribution, we present our views on these RF requirements.

# 2 Discussion

ACS

For ACS, TR 38.817-2 illustrates the method to calculate the interferer offset as following:

The required increment in frequency offset can be calculated by applying the following equation: where and is the nominal frequency offset (which is equal to half of the sum of the channel bandwidth of the wanted signal and the channel bandwidth of the interfering signal) and is the calculated offset increment.

In this case, x is 734, hence the offset of the interferer for ACS is 5.5125MHz – 6/2 MHz = 2.5125MHz.

Proposal 1: Set the interferer offset as 2.5125kHz, i.e., Option 1.

NBB

For narrowband blocking, the following table shows the current specs:

Of these two open options, interferer BASE values for 6MHz channel bandwidth are different, while m values are the same. As the same method to choose the base offset value in the early stage as in R4-1713293, the offset for 6MHz is the same as that for 7MHz, i.e., 355kHz.

offset= 0.5*BWchannel+350+7.515-0.5*BWchannel+350+7.515*15

Proposal 2: Set the base offset as 355kHz, i.e., Option 1, for narrowband blocking for 6MHz channel bandwidth.

Rx IMD

The CW offset and NR interferer offset has the following relationship:

∆CW=∆NR4+TBC4-CBW+TBCint4                    (1)

It is quite straightforward to calculate the CW offset according to the above Eq. (1) as the following table, which indicates 7.475MHz:

Proposal 3: Set CW offset as 7.475MHz for intermodulation requirement for 6MHz channel bandwidth.

For narrowband intermodulation, the offsets of CW and single RB interferer should also meet Eq. (1). Of the two options only Option 1 satisfies the equation.

Proposal 4: Set CW offset as 425kHz and offset of single RB interferer as 1240kHz, i.e., Option 1.

ICS

Of the three options for the interfering signal, Option 1 takes the same power level and number of RBs as 7MHz channel bandwidth as illustrated in the table below, and Option 3 the same as 5MHz channel bandwidth, while Option 2 goes in between these two options, which is a reasonable choice in our view.

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |

Proposal 5: Set the mean power and type of the interfering signal as -80.6dBm and 12 RBs for ICS, i.e., Option 2.

# 3 Conclusion

In this contribution, we have the following proposals for BS RF requirements for 6MHz channel bandwidth:

Proposal 1: Set the interferer offset as 2.5125kHz, i.e., Option 1.

Proposal 2: Set the base offset as 355kHz, i.e., Option 1, for narrowband blocking for 6MHz channel bandwidth.

Proposal 3: Set CW offset as 7.475MHz for intermodulation requirement for 6MHz channel bandwidth.

Proposal 4: Set CW offset as 425kHz and offset of single RB interferer as 1240kHz, i.e., Option 1.

Proposal 5: Set the mean power and type of the interfering signal as -80.6dBm and 12 RBs for ICS, i.e., Option 2.

# Reference

[1] R4-2515084, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel”, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm.