3GPP TSG RAN WG4 Meeting #117		R4-2520189

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Further discussion on UE RF requirements for 6MHz channel bandwidth

Agenda Item:	7.1.5.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With respect to UE RF requirements for a 6 MHz channel bandwidth, significant progress was achieved at RAN4#116bis,

Agreement: - Conduct A-MPR evaluation (if needed) for - NS_06 for n12 (PC3) - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable. - NS_06 for n85 (PC3 and PC2) - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable. - NS_14 for n5 (PC3 and PC2): A-MPR evaluation is not needed for 6MHz leveraging the experience of 7 MHz - NS_15 for n5 (PC3 and PC2) - NS_43/43U for n8: A-MPR evaluation is not needed as 6MHz is not demanded in Japan as confirmed by SoftBank

In this contribution, we present our A-MPR evaluation results on NS_15 for n5.

# 2 Discussion

For NS_15, the additional spectrum emission requirements is specified as the following table [2]:

##### 6.5.3.3.20 Requirement for network signalling value "NS_15" When "NS_15" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.20-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. Table 6.5.3.3.20-1: Additional requirements "NS_15" Frequency range (MHz); Channel bandwidth / Spectrum emission limit (dBm); Measurement bandwidth. 3 MHz, 5 MHz, 7 MHz, 10 MHz, 15 MHz, 20 MHz. 851 ≤ f ≤ 859; -53; 6.25 kHz. NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB..

In our evaluation, the following assumptions are used for simulations:

- LO leakage = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

- CIM5 = -70dBc.

And the PA model is calibrated with DFT-s-OFDM QPSK 20MHz with 1dB MPR.

The following figures illustrate the heatmaps for PC3 and PC2 with 6MHz channel bandwidth respectively.

- 6MHz is placed at the lower edge of band n5

- 6MHz is placed in the middle of band n5

- 6MHz is placed at the higher edge of band n5

Fig. 1, A-MPR heatmap for band n5 with 6MHz channel bandwidth for PC3

As shown in Fig. 1, for all three cases where the 6MHz carrier is placed at the lower edge, middle and the higher edge of band n5, the region where L_CRB is larger than 15 PRBs (i.e., 180kHz * 15 = 2.7MHz) but less than 19 PRBs (i.e., 180kHz * 19 = 3.42MHz) has a maximum A-MPR 2.5dB, and the region where L_CRB is larger than 19PRBs has a maximum A-MPR 8.5dB. Hence, we propose to define the following regions for NS_15 for 6MHz channel bandwidth:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 2.7 MHz ≤ 3.42 MHz | A11 |
|  |  | >0 | > 3.42 MHz | A12 |

The value A11 is 2.5dB , and A12 is 8.5dB for DFT-s-OFDM QPSK.

Proposal 1: RAN4 to define PC3 A-MPR regions for NS_15 for 6MHz channel bandwidth as the following table, where the value A11 is set as 2.5dB, and A12 as 8.5dB for DFT-s-OFDM QPSK.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 2.7 MHz ≤ 3.42 MHz | A11 |
|  |  | >0 | > 3.42 MHz | A12 |

Similarly, Fig. 2 illustrates the A-MPR heatmaps for PC2.

- 6MHz is placed in the lower edge of band n5

- 6MHz is placed in the middle of band n5

- 6MHz is placed in the higher edge of band n5

Fig. 2, A-MPR heatmap for band n5 with 6MHz channel bandwidth for PC2

From the above figures, we could observe that two regions, i.e., L_CRB is between 11 PRBs (i.e., 180kHz * 11 = 1.98MHz) and 20 PRBs (i.e., 180kHz*20=3.6MHz), or above 20 PRBs with a maximum A-MPR 4.5 dB and 11.5dB respectively, for NS_15 PC2. Accordingly, the table is shown below, and the value of A14 for DFT-s-OFDM QPSK is 4.5dB and A15 is 11.5 dB.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 1.98 MHz ≤ 3.6 MHz | A14 |
|  |  | >0 | > 3.6 MHz | A15 |

Proposal 2: RAN4 to define PC2 A-MPR regions for NS_15 for 6MHz channel bandwidth as the following table, where the value A14 is set as 4.5dB, and A15 as 11.5dB for DFT-s-OFDM QPSK.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 1.98 MHz ≤ 3.6 MHz | A14 |
|  |  | >0 | > 3.6 MHz | A15 |

# 3 Conclusion

In this contribution, we have the following proposals for UE RF requirements for 6MHz channel bandwidth:

Proposal 1: RAN4 to define PC3 A-MPR regions for NS_15 for 6MHz channel bandwidth as the following table, where the value A11 is set as 2.5dB, and A12 as 8.5dB for DFT-s-OFDM QPSK.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 2.7 MHz ≤ 3.42 MHz | A11 |
|  |  | >0 | > 3.42 MHz | A12 |

Proposal 2: RAN4 to define PC2 A-MPR regions for NS_15 for 6MHz channel bandwidth as the following table, where the value A14 is set as 4.5dB, and A15 as 11.5dB for DFT-s-OFDM QPSK.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 6MHz |  | >0 | ≥ 1.98 MHz ≤ 3.6 MHz | A14 |
|  |  | >0 | > 3.6 MHz | A15 |

# Reference

[1] R4-2515084, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel”, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm.

[2] 3GPP TS 38.101-1 v19.3.0