3GPP TSG RAN WG1 Meeting #117	 								                         R4-2520217

Dallas, Texas, U.S. , Nov 17th – 21th , 2025

Agenda Item:	8.4.3

Source:	Amazon

Title:	Consideration on 6GR TN and NTN co-existence

Document for:	Discussion and Decision

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In RAN4#116bis, the following is agreed:

R4-2514638	WF on NTN (section 1.6)

Way Forward:

Additional input from satellite companies is expected for RAN4#117:

- List of requirements to be further studied (using the same format proposed in 1.1.2)

- List of coexistence studies to be considered (using the format proposed in 1.3.1).

# Discussion

NTN-TN RF Coexistence

We summarize below key features to be considered in the integration/harmonization of both TN and NTN systems, such to improve/enhance and provide ubiquitous coverage:

  - NTN and TN RF coexistence scenarios

Scenario 1 - NTN and TN coverage overlap:

Proposal: 3GPP should start a study item to evaluate NTN to TN (and TN to NTN) co-existence interference requirements for adjacent and cochannel interference. The study should define different band combinations for TN and NTN systems and define RF cochannel and adjacent channels constraints such to minimize interference between both TN and NTN deployments.

        - Advantages: Extends to ubiquitous coverage; provides redundancy and reliability for wireless communications, increases capacity and load balancing capabilities and provides opportunity to enhanced mobility scenarios

        - Challenges:  Interference and spectrum management, latency variability, deployment and NTN/TN integration costs; technical complexity to manage mobility, regulatory and ecosystem hurdles

Scenario 2 - NTN and TN Coverage do not overlap:

Proposal: 3GPP should start a study item to define adjacent and co-channel interference requirements or a minimum operation safe distance such to allow both TN and NTN to operate efficiently while reducing the coverage gap between both systems.

  - F2 could be equal to F1 in some scenarios

        - Advantages: Extended coverage in areas where TN is not available with low cost of TN/NTN deployment integration and spectrum coordination.  NTN/TN interference between border cells of the TN system and NTN border cells can be controlled by defining a minimum separation distance or equivalently rigorous RF radiation for both systems.

        - Challenges:  No bidirectional offloading is possible; NTN does not provide resilience to TN and a coverage gap between TN and NTN may be necessary.ChallengesChallenges

## NTN-TN Inter-system Interference Management

  - Background: Section 2.1 defines RF requirement studies for the co-existence of NTN and TN deployments. These requirements do not assume the interaction of TN and NTN systems to effectively handle interference between both systems.  Since 6G is being designed with TN and NTN harmonization goal, it is natural to think there would be a good level of interworking between both TN and NTN systems and hence more dynamic and efficient ways to handle the inter-system interference.

  - Proposal:  3GPP should start a study item to define an NTN to TN (and TN to NTN) interference detection, reporting and management Framework:

    - TN network detects and reports interference coming up from NTN system:  The study should evaluate which TN network element (UE or Base station) detects and reports a level of interference coming from the NTN system. It should also define interference levels and measurement parameters where NTN signals are harmful to TN system. The study should define what to measure, when to report it, how to convey the interference levels to the NTN system and if possible what actions are to be taken by the NTN to mitigate the reported interference.

    - NTN network detects and reports interference coming up from TN systems:  The study should evaluate which NTN network elements (UE or NTN payload) detect and report a level of interference coming up from the TN system. It should define interference levels and measurement parameters where TN signals are harmful to the NTN system. It should define what to measure, when to report it, how to convey the interference levels to the TN system and if possible what actions are to be taken by the TN system to mitigate the reported interference.

## NTN-TN offloading and UE communications resilience

  - Background:  when considering NTN and TN coverage overlap as defined in Section 2.1; there are scenarios where offloading between the two systems may result beneficial.  Some examples:

    - Disaster recovery and Emergency Response (TN to NTN): In the event of TN infrastructure damage by natural disaster or cyberattacks; users can offload or reselect to NTN for immediate connectivity. Once TN is restored, traffic shifts automatically back to TN.

    - Peak hour Load Balancing in urban/rural edges (bidirectional):  offload routine data from TN to NTN in rush hours where TN coverage is spotty

    - High-Mobility applications (bidirectional)

  - Proposal 1: Since Interworking between TN and NTN must be considered harmonized 6G: 3GPP should study the benefits of having a bi-directional offloading capability between both TN and NTN systems in the situation where TN/NTN coverage overlap exists.

  - Proposal 2:  Whenever coverage overlap is available, besides network triggered off loading, 3GPP should study alternatives to automatically switch between NTN and TN systems in the absence of TN coverage or NTN coverage due to emergency conditions.

## NTN-TN Mobility

  - Background:  When considering NTN and TN coverage overlap (and non-coverage overlap); the 6G system should have clearly defined how to handle the inter-system mobility between both systems as follows:

  - Proposal - 3GPP should start a study defining NTN/TN IDLE Mobility:  It should be defined what the UE is to measure in both TN and NTN systems, when to start measuring, the measurement accuracy and how is the decision of changing from one system to the other for IDLE UEs roaming between both systems.

  - Proposal - 3GPP should start a study defining NTN/TN CONNECTED Mobility: Bidirectional handovers between both TN and NTN systems should be studied with the goal of minimizing service interruptions and supporting continuity of services across both systems. Measurements and reporting events for the UEs being covered by both systems should be defined.

# Conclusion

In this contribution, we propose the following items for further study:

  - NTN-TN RF Coexistence: includes two NTN-TN coverage co-existence deployment scenarios: Scenario 1:  NTN and TN with coverage overlap; Scenario 2: NTN and TN without coverage overlap

  - NTN-TN Interference Management:  proposes a study item dealing with TN and NTN inter-system interference detection, reporting and mitigation

  - NTN-TN Offloading and UE communications resilience:  When there is TN and NTN coverage overlap; two study item proposals: 1) TN/NTN network triggered offloading capabilities and 2) automatic UE service switch between NTN and TN systems in the event of absence of coverage of either TN or NTN due to emergency conditions

  - NTN-TN bidirectional mobility: the study of IDLE and CONNECTED mobility between TN and NTN is proposed including measurements and reporting events for service continuity while roaming between both systems