3GPP TSG-RAN WG4 Meeting #117                                    R4-2520225

Dallas, USA, November 17 – November 21, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on general MSD framework for 6G

Agenda item:	8.3

Document for:	Discussion

Introduction

In the last RAN4#116-bis meeting, RAN4 working group started 6G study items and one of the discussion sessions is general RF and UE RF topic which includes high level principles, UE reference architecture, Tx part with requirements, Rx part with requirements, frequency range and spectrum aggregation, etc. However, due to diverse inputs from companies and lack of time, some topics such as MSD framework couldn’t be discussed.

Therefore, we re-submit our discussion paper [1] on the high-level general MSD framework for 6G for discussion of RAN4#117 meeting.

Discussion

Current MSD analysis procedure

Until 5G era, MSD requirement for PC3 is derived case by case manner, which leads to increase of RAN4 workload. MSD analysis and calculation for CA/DC have been done whenever new CA/DC combinations are requested from operators, and this requires high RAN4 workload.

Mostly, several companies proposed the MSD based on their own architecture and parameters assumptions, and RAN4 determined the MSD by linearly averaging the MSD inputs from proponent companies. Sometimes, the MSD from the existing similar band combination is reused.

On the other hand, there are some cases where the MSD is slightly different from each other even though the band combination in terms of frequency and architecture would be very similar.

To make matters worse, there are too many MSD cases in the specification documents, it is not efficient to refer or check the requirement whenever it is needed.

Therefore, general MSD framework for 6G is expected to help simplify the MSD calculation as well as MSD test procedures, and several companies proposed the similar concept on MSD framework for 6G in the last RAN4#116-bis meeting in [1]-[8].

New MSD framework proposal for 6G

The MSD is highly dependent on the filter performance, linearity of RF devices, PA spectrum output and RF front end architecture. Of course, the aggressor and victim frequency play a major role determining the MSD.

The basic idea is that it might be possible to generate the general MSD equations if the above parameters are pre-defined depending on the RF front end architecture with UL and DL frequency information.

For example, same linearity performance (IIPn) and RF front end architecture can be used to derive MSD due to IMD for same band group combination as in Table 2 with band group definition in Table 1 used in 5G. The band group definition might be different in 6G if new band group concept is introduced for 6G.

Table 1 Band group definition in 5G FR1

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a (LB) | FR1-b (MB) | FR1-c (HB) | FR1-d (VHB) | FR1-e (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Table 2 PC3 MSD due to 2 UL IMD (example)

| Aggressor 1 | Aggressor 2 | Victim | IMD order | MSD |
| --- | --- | --- | --- | --- |
| LB | LB | MB | 2 | A |
| MB | VHB | MB | 4 | B |
| MB | HB | MB | 4 | C |

It should be noted that the MSD calculation mechanism is dependent on not only IMD order but also index. However, it can be simplified as one unified MSD number as HPUE MSD simplification rule with LUT approach.

Similarly, harmonic and harmonic mixing MSD can be derived as in Table 3.

Table 3 PC3 MSD due to harmonic and harmonic mixing (example)

| UL | DL | Harmonic Order | MSD |
| --- | --- | --- | --- |
| LB | MB | 3/1 | D |
| MB | VHB | 2/1 | E |
| VHB | HB | 2/3 | F |

For crossband isolation MSD, the filter attenuation and PA spectrum noise are dominant. In this case, the distance between UL-DL or fractional frequency information might be used such as LUT shown in Table 4.

Table 4 PC3 MSD due to crossband isolation (example)

| (FreqDL-FreqUL)/ (FreqDL+FreqUL) (%) | MSD |
| --- | --- |
| <1 | G |
| 1≤ <2 | H |
| 2≤ <4 | I |
| 4≤ <6 | J |
| 6≤ <8 | K |
| 8≤ <10 | L |
| ≥ 10 | M |

Conclusion

In this contribution, we proposed the general MSD framework for 6G to improve the work efficiency of RAN4.

Proposal 1: RAN4 to study the general MSD framework for 6G as proposed in Table 2, Table 3 and Table 4 as a starting point.

References

[1]	R4-2513289, “Discussion on general MSD calculation rule for 6G”, Murata Manufacturing Co Ltd, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[2]	R4-2513148, “Views on 6G UE RF”, Spreadtrum, UNISOC, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[3]	R4-2513022, “Qualcomm views on 6G spectrum”, Qualcomm Incorporated, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[4]	R4-2513059, “Discussion on 6G spectrum”, China Telecom, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[5]	R4-2513252, “Discussion on 6G Spectrum”, vivo, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[6]	R4-2513271, “Views on 6G spectrum aspect”, ZTE Corporation,Sanechips, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[7]	R4-2513273, “Considerations on 6G Spectrum and 6G Band Combinations”, Ericsson, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[8]	R4-2513307, “Consideration on spectrum for 6G”, Huawei, HiSilicon, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025