3GPP TSG-RAN WG4 # 117	R4-2520236

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	6.12.3

Source:	MediaTek Inc.

Title:	Discussion on NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

In the previous RAN4#116bis meeting NR MIMO Phase 5 was discussed and WF with several open issues was introduced for further work [1]. In this document we discuss remaining open issues with several observations and proposals. We also have separate simulation results document [2].

# Discussion

In this section we discuss remaining open issues.

Issue 1-2-2: Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: - Option 1A: X=5dB γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. - Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed Way forward: - FFS based on simulation results provided in following meetings

We observe that our simulated gamma values without additional offset range from 15 to 30 for TypeI-SinglePanel-r19 codebook and from 19 to 57 for eTypeII-r19 codebook. These are likely unreliable and unstable because the random-precoding reference operated at an extremely high BLER. Therefore, rather than using these high gamma values directly as requirements, we suggest identifying an alternative solution. We consider the proposed Option 1 as a potential candidate to address this issue. To help evaluate the feasibility of Option 1, we have provided gamma values with multiple offsets in our simulation paper [2]. In addition, we propose defining requirements at the SNR corresponding to 90% of the maximum throughput under PMI follow operation. We recommend comparing these two alternatives to determine which yields better-aligned results.

Observation #1: Our simulated gamma values range from 15 to 57.

Proposal #1: Consider Option 1 as a possible solution.

Proposal #2: Consider defining the requirement solely on SNR measured at 90% of maximum throughput when PMI is applied.

Proposal #3: Make the final decision on the test metric based on the simulation results.

Issue 1-2-9: round-off operation of MIMO correlation matrices Agreement: - Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements. - Further discuss and decide which option to be captured in the CR in next meeting - Option 1: Include round-off to four digits precision operation - Option 2: Not include round-off to four digits precision operation Issue 1-2-10: Scaling factors for MIMO correlation matrices Agreement: - For option 1 in Issue 1-2-9, as baseline, adopt the following scaling factors for 64 CSI-RS ports XP-Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability Array Type; Antenna Configuration; Correlation Level; Scaling Factor. Cross-polarized; 64 (8,4,2)×2; Medium; 0.00065. Cross-polarized; 64 (8,4,2)×4; Medium; 0.00092. - Further update on the scaling factor is not precluded.

Correlation matrices are positive semi-definite by definition. The diagonal loading scaling factor 'a' has been defined in TS 38.101-4 Annex B only for the purpose of keeping MIMO correlation matrices positive semi-definite after truncating the element-wise values to four decimal digits. These numerical matrices have then been printed in the spec for a few cases (but not all) with relatively small numbers of TX/RX antennas.

As the number of TX-antennas increases further, it is not possible or beneficial to print the numerical values of correlation matrices into spec. The composite matrix size for 64TX-4RX MIMO channel is 256x256, and for 128TX-8RX the size is 1024x1024. Furthermore, different N1xN2 gNB array geometries result in different numerical matrices. Since the numerical values will not be printed, there is no need for truncating. Truncation creates inaccuracy to the desired correlation properties and makes the matrix decomposition numerically more challenging. Thus, we should not instruct people to do truncation.

Observation #2: Since the numerical values will not be printed, there is no need for truncating correlation matrices.

Observation #3: Truncating correlation matrices to four decimal digits creates inaccuracy and makes matrix decomposition more challenging.

It is worth noting that a correlation matrix is valid even if it has zeros as eigenvalues. For example, all-ones matrix is a valid correlation matrix. There are numerically robust decomposition methods such as SVD that can be used instead of Cholesky decomposition. Thus, defining diagonal loading and scaling with factor 'a' is not necessary in general.

Observation #4: Robust matrix decomposition methods such as SVD do not require diagonal loading and scaling even if the correlation matrix has zeros as eigenvalues.

As the matrix size increases, it is not efficient or numerically stable to apply the correlations by using the huge composite correlation matrix at once. Luckily, correlations specified by 3GPP are separately defined for different matrix sub-dimensions (as α1, α2, β, and γ). Each sub-correlation can be applied one sub-dimension at a time by using smaller correlation matrices that are easy to decompose. Actually, specifying diagonal loading on top of the huge composite matrix gives a wrong impression of how to efficiently implement correlation generation. Thus, we believe that specifying scaling factors is not only unnecessary, but it also gives an incorrect message.

Proposal #4: Not to include round-off to four digits precision operation for MIMO correlation matrices.

Proposal #5: Not to specify or use scaling factors for MIMO correlation matrices.

# Conclusion

In this contribution we provide the view on the NR MIMO Phase 5 demodulation and CSI requirements issues. The following observations and proposals are made:

Observation #1: Our simulated gammas vary between 15 to 57.

Proposal #1: Consider Option 1 as a possible solution.

Proposal #2: Consider defining the requirement based solely on the SNR at 90 % of the maximum throughput when PMI is followed.

Proposal #3: Make the final decision on the test metric based on the simulation results.

Observation #2: Since the numerical values will not be printed, there is no need for truncating correlation matrices.

Observation #3: Truncating correlation matrices to four decimal digits creates inaccuracy and makes matrix decomposition more challenging.

Observation #4: Robust matrix decomposition methods such as SVD do not require diagonal loading and scaling even if the correlation matrix has zeros as eigenvalues.

Proposal #4: Not to include round-off to four digits precision operation.

Proposal #5: Not to specify or use scaling factors for MIMO correlation matrices.

# Reference

- R4-2514806, WF on NR_MIMO_demod, Samsung

- R4-2520237, Simulation results of NR MIMO Phase 5 demodulation and CSI requirements, MediaTek