3GPP TSG-RAN WG4 Meeting #117	R4-2520243

Dallas, United States, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on demodulation performance and CSI reporting requirements for MIMO_Ph5

Agenda item:	6.12.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, it was agreed to introduce requirements for eTypeII-r19 and not to introduce requirements for typeII-Doppler-r19 as well as SRS port grouping and CJT reporting enhancements.

In this contribution we will discuss the remaining issues and provide observations and proposals where needed.

UE demodulation/CSI reporting performance requirements

Test assumptions for typeI-SinglePanel-r19

Test metric for 64 CSI-RS ports

In RAN4#116bis it was discussed how to define the test metric for 64 CSI-RS ports (see [1]):

Issue 1-2-2: Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: - Option 1A: X=5dB γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. - Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed Way forward: - FFS based on simulation results provided in following meetings

The main reason for the discussion is that at the 90% throughput point normally used for comparison between follow PMI and random PMI, the simulation results already provided by other companies show that random PMI has extremely low throughput. Due to this, option 1 was proposed to artificially move the random PMI to a point where the simulations show more reasonable throughput.
For Option 1 we see an issue as the comparison is not a 1-to-1 comparison, hence it should be the last resort. Also introducing a new test metric which might still fail for future configurations where potentially random precoder will be having even higher SNR before reasonable throughput is seen, seems not an optimal choice.

Due to above concerns, we find the best way forward is to consider if requirements can be defined only based on SNR of 90% of the maximum throughput when PMI is followed, hence we support this to be the target solution. Of course, this is pending the alignment of simulation results.

We see the most reasonable way forward for defining the test metric for 64 CSI-RS ports to further analyse if it will be possible to define requirements only based on SNR of 90% of the maximum throughput when PMI is followed.

Target to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed for 64 CSI-RS ports.

Round-off operation of MIMO correlation matrices

In RAN4#116bis it was agreed that companies can provide simulation results using round-off to four digits or no rounding. It was however not agreed if rounding/scaling should be included in TS38.101-4 (see[1]):

Issue 1-2-9: round-off operation of MIMO correlation matrices Agreement: - Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements. - Further discuss and decide which option to be captured in the CR in next meeting - Option 1: Include round-off to four digits precision operation - Option 2: Not include round-off to four digits precision operation

Based on the discussions during RAN4#116bis we understand that companies have different pre-requisites for how the simulation results are achieved which includes both options from above. Since it has been agreed that both options will be taken into account for deriving the requirements, we do not see any reason to omit capturing the option of round-off as well as the scaling factors into TS38.101-4. In case companies choose to not use the round-off to 4 digits for providing simulation results that can still be accepted.

Include round-off to four digits precision operation to ensure all companies procedures are covered.

Change “scaling factor” to “regularization factor”

In TS38.101-4 it is incorrect to use the term “scaling factor” to regularize the values when rounding results in a non-semi positive definite correlation matrix. “Scaling factor” is normally used in normalization where as “regularization factor” is used to introduce regularization into a model. Based on our understanding we see the need to change the existing terms in TS38.101-4 from “scaling factor” to “regularization factor”.

The use of “scaling factor” is not correct when talking about regularizing the values in the correlation matrix to ensure positive semi-definite after round-off to 4 digits.

For TS38.101-4 section B.2.3.1.2 use “regularization factor” instead of currently used “scaling factor”.

Regularization factors for MIMO correlation matrices

In RAN4#116bis the regularization factors were discussed, and it was decided to adopt below regularization factors for 64 CSI-RS port with XP medium correlation level for 64 ports (see [1]):

Issue 1-2-10: Scaling factors for MIMO correlation matrices Agreement: - For option 1 in Issue 1-2-9, as baseline, adopt the following scaling factors for 64 CSI-RS ports XP-Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability Array Type; Antenna Configuration; Correlation Level; Scaling Factor. Cross-polarized; 64 (8,4,2)×2; Medium; 0.00065. Cross-polarized; 64 (8,4,2)×4; Medium; 0.00092. - Further update on the scaling factor is not precluded.

As the regularization factors were adapted from a larger table of proposed regularization factors, we would suggest to include the full table for future use. In our draftCR [2] we have included all the regularization factors with confirmation pending the draftCR review.

Adapt all regularization factors into the spec to include the following:

| Array Type | Antenna Configuration | Correlation Level | Regularization factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 |

BS demodulation performance requirements

General scope

Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

Issue 2-1-1: Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions Agreement: - RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions. - FFS on whether to introduce test applicability rule for 3Tx

During RAN4#116-bis it was discussed whether a test applicability rule should be introduced for 3Tx, however no other test applicability rules have been introduced for other UL MIMO codebooks, therefore Nokia strongly believes that no applicability rule should be introduced for 3Tx

Do not introduce test applicability rule for 3Tx.

Conclusion

This contribution has presented Nokia’s view on the topic MIMO_Ph5.

The following Observations and Proposals were made:

UE demodulation/CSI reporting performance requirements

Test assumptions for typeI-SinglePanel-r19

Test metric for 64 CSI-RS ports

- We see the most reasonable way forward for defining the test metric for 64 CSI-RS ports to further analyse if it will be possible to define requirements only based on SNR of 90% of the maximum throughput when PMI is followed.

- Target to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed for 64 CSI-RS ports.

Round-off operation of MIMO correlation matrices

Include round-off to four digits precision operation to ensure all companies procedures are covered.

Change “scaling factor” to “regularization factor”

The use of “scaling factor” is not correct when talking about regularizing the values in the correlation matrix to ensure positive semi-definite after round-off to 4 digits.

For TS38.101-4 section B.2.3.1.2 use “regularization factor” instead of currently used “scaling factor”.

Regularization factors for MIMO correlation matrices

Adapt all regularization factors into the spec to include the following:

| Array Type | Antenna Configuration | Correlation Level | Regularization factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 |

BS demodulation performance requirements

General scope

Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

Do not introduce test applicability rule for 3Tx.

- References

- R4-2514806 - WF on NR_MIMO_demod

- R4-2520246 - (NR_MIMO_Ph5-Perf) DraftCR on 38.101-4 for adding MIMO Channel Correlation Matrices