3GPP TSG-RAN WG4 Meeting #117	R4-2520247

Dallas, United States, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for LPWUS

Agenda item:	6.16.3

Document for:	Discussion

Introduction

In RAN4#116bis the final agreements on simulation assumptions were made. In the following we will provide our view on the remaining issues for introduction of MDR requirements for LPWUS.

We would also like to notice, that in RAN4#116bis there were no consensus to introduce FAR requirements, and it is still our view, that a UE which has no dedicated LP-WUR implemented will be able to pass any introduced MDR tests using the main receiver. Our view is based on the following:
From the RAN1 agreements a device supporting LP-WUS decoding can autonomously change from a specialized receiver to main radio without any signalling to the network. This means that when test is conducted in normal mode (no test mode) for the LP-WUR and the count of missed detection is based on the full wake up sequence from idle to connected mode the test equipment cannot distinguish between wake-up sequences there has been triggered by a correct decoding of the LP-WUS or triggered in the main radio (MR) by the Paging Occasion (PO) sequence. This is illustrated in Figure 1.

Figure 1 Illustration of the idle wakeup sequence

This means that a test in normal mode of the LP-WUR is based on MR results and can neither be used as a criterion for a functional LP-WUR as the test can be conducted by the MR only, nor can it be used for performance requirement.

Discussion

Testability Aspects

Whether to define separate test cases for idle/inactive and connected modes

In RAN4#116bis it was agreed to define one set of requirements and add test applicability rule based on UE supported capabilities (see [1]):

Issue 1-1-3: Whether to define separate test cases for idle/inactive and connected modes - Agreement: Define one set of requirements and add test applicability rule that the test is conducted in idle/inactive and/or connected modes based on UE supported capabilities. If UE support both RRC idle/inactive and connected modes, the test will be conducted: - Option 1: in connected mode - Option 2: in idle/inactive mode - Option 3: in both RRC idle/inactive and connected modes - FFS whether to capture the test applicability w.r.t. RRC state in 38.101-4 or leave it to RAN5.

In the agreement made, there are three options on how the UE supporting both idle/inactive and connected modes will have to be tested. It is our view, that as the wakeup signal will be used in different manor in idle/inactive and in connected mode it would be reasonable to have a test for both idle and connected mode.

Since the wakeup signal will be used in different manor in idle/inactive and in connected mode, we see it most reasonable that a UE supporting both would also need to be tested for both.

For UE supporting both idle/inactive and connected mode, the test shall be conducted in both idle/inactive and connected mode.

Demodulation Requirements

Consideration of one-shot and repetition transmission mode for OFDM-based receiver

In RAN4#116bis it was brought up if for OFDM based receiver, it should be considered if requirements are to be specified for both one-shot and repetition transmission mode (see [1]):

Issue 2-1-13: Consideration of one-shot and repetition transmission mode for OFDM-based receiver - Proposals - Option 1 - RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers.

In case of test to be performed in normal mode the test case definition should follow the RAN2 specification. In this case the requirements should follow the full repetition case for OFDM- based receiver.

Specify requirements at least for full repetition case for OFDM- based receivers.

Whether to define single requirement for OFDM and OOK receivers

In RAN4#116bis it was not yet decided if one or two sets of requirements would be defined. Effectively the decision was postponed until this meeting (see [1]):

Issue 2-1-16: Whether to define single requirement for OFDM and OOK receivers - Proposals - Option 1: Define two sets of requirements (one for each receiver type: OOK and OFDM). - Option 2: RAN4 to study the performance for both receiver types (OOK receiver and OFDM receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined

Based on the earlier proposals and agreements for simulation of the different receiver types where the timing and frequency offset are proposed to be different for the two receiver types. From our simulation results it can be seen a quite big difference (up to 5dB in SNR) in performance [2]

Based on our simulation results and simulation agreed settings a common set of requirements will be quite difficult to derive.

Define two sets of requirements (one for each receiver type: OOK and OFDM) (option 1).

Whether to define applicability rules for OFDM and OOK receivers

In RAN4#116bis it was discussed if applicability rules should be defined for the two receivers being discussed (see [1]):

Issue 2-1-17: Whether to define applicability rules for OFDM and OOK receivers - Proposals - Option 1: Don’t define any capability rules related to OFDM and OOK receivers for demodulation requirements. - Option 2: Defer the discussion of applicability rules until simulation results have been analysed for both receivers.

OFDM and OOK are two different receiver implementations, hence we do not see it reasonable to introduce applicability rules enabling the UE to be tested against only one of the receiver types.

Do not define any applicability rules related to OFDM and OOK receivers for demodulation requirements.

LP-WUS test cases for requirement definition

In RAN4#116bis the following was proposed as test cases for requirement definition, however agreements made during RAN4#116bis has reduced the scope (see [1]):

Issue 2-1-20: LP-WUS test cases for requirement definition - Proposals - Option 1: Consider defining requirements using the following testcases covering MDR Test #; Freq. range; Duplex; Receiver type; Detector; Channel model; M; L; Info bits. Test 1-1; FR1; FDD; OOK4; Envelope detector; TDLA30-10; M=1; L=12; Infobits=2. Test 1-2; M=4. Test 1-3; TDLC300-100; M=1; L=32; Infobits=5. Test 1-4; M=4. Test 1-5; Sequence detector; TDLA30-10; M=1; L=12; Infobits=2. Test 1-6; M=4. Test 1-7; TDLC300-100; M=1; L=32; Infobits=5. Test 1-8; M=4. Test 2-1; FR1; TDD; OOK4; Envelope detector; TDLA30-10; M=1; L=12; Infobits=2. Test 2-2; M=4. Test 2-3; TDLC300-100; M=1; L=32; Infobits=5. Test 2-4; M=4. Test 2-5; Sequence detector; TDLA30-10; M=1; L=12; Infobits=2. Test 2-6; M=4. Test 2-7; TDLC300-100; M=1; L=32; Infobits=5. Test 2-8; M=4. Test 3-1; FR1; FDD; OFDM; N/A; TDLA30-10; M=1; L=12; Infobits=2. Test 3-2; TDLC300-100; L=32; Infobits=5. Test 4-1; FR1; TDD; OFDM; N/A; TDLA30-10; M=1; L=12; Infobits=2. Test 4-2; TDLC300-100; L=32; Infobits=5. Test 5-1; FR2; TDD; OOK1; Envelope detector; TDLA30-10; M=1; L=12; Infobits=2. Test 5-2; L=32; Infobits=5.

Based on the agreements from previous meetings as well as our proposals earlier in this contribution, we now see it relevant to define tests for the following testcases for LP-WUS:

Introduce test cases for the following configurations:

| Test # | Freq. range | Duplex | Receiver type | Channel model | M | L | Info bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | FR1 | FDD | OOK4 | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 1-2 | FR1 | FDD | OOK4 | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 2-1 | FR1 | TDD | OOK4 | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 2-2 | FR1 | TDD | OOK4 | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 3-1 | FR1 | FDD | OFDM | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 3-2 | FR1 | FDD | OFDM | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 4-1 | FR1 | TDD | OFDM | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 4-2 | FR1 | TDD | OFDM | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 5-1 | FR2 | TDD | OOK1 | TDLA30-75 | M=1 | L=12 | Infobits=2 |
| Test 5-2 | FR2 | TDD | OFDM | TDLA30-75 | M=1 | L=32 | Infobits=5 |

Conclusion

This contribution has provided Nokia’s view on the topic of introduction of performance requirements for LP-WUS.

The following Observations and Proposals were made:

Testability Aspects

Whether to define separate test cases for idle/inactive and connected modes

- Since the wakeup signal will be used in different manor in idle/inactive and in connected mode, we see it most reasonable that a UE supporting both would also need to be tested for both.

- For UE supporting both idle/inactive and connected mode, the test shall be conducted in both idle/inactive and connected mode.

Demodulation Requirements

Consideration of one-shot and repetition transmission mode for OFDM-based receiver

Specify requirements at least for full repetition case for OFDM- based receivers.

Whether to define single requirement for OFDM and OOK receivers

Based on our simulation results and simulation agreed settings a common set of requirements will be quite difficult to derive.

Define two sets of requirements (one for each receiver type: OOK and OFDM) (option 1).

Whether to define applicability rules for OFDM and OOK receivers

Do not define any applicability rules related to OFDM and OOK receivers for demodulation requirements.

LP-WUS test cases for requirement definition

Introduce test cases for the following configurations:

| Test # | Freq. range | Duplex | Receiver type | Channel model | M | L | Info bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | FR1 | FDD | OOK4 | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 1-2 | FR1 | FDD | OOK4 | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 2-1 | FR1 | TDD | OOK4 | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 2-2 | FR1 | TDD | OOK4 | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 3-1 | FR1 | FDD | OFDM | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 3-2 | FR1 | FDD | OFDM | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 4-1 | FR1 | TDD | OFDM | TDLA30-10 | M=4 | L=12 | Infobits=2 |
| Test 4-2 | FR1 | TDD | OFDM | TDLA30-10 | M=4 | L=32 | Infobits=5 |
| Test 5-1 | FR2 | TDD | OOK1 | TDLA30-75 | M=1 | L=12 | Infobits=2 |
| Test 5-2 | FR2 | TDD | OFDM | TDLA30-75 | M=1 | L=32 | Infobits=5 |

- References

- R4-2514813 - WF for [116bis][220] NR_LPWUS_Demod

- R4-2520248 - Simulation results for UE demodulation performance requirements for LPWUS