3GPP TSG-RAN WG4 Meeting #117				                                　 R4-2520252

Dallas, USA, Nov. 17-21, 2025

Source: 	Huawei, HiSilicon

Title: 	MPR reduction for upper 6GHz by a new ACLR

Agenda Item:	10.2.2

Document for:	Approval

# Introduction

This contribution is a follow-up contribution of [1] based on the approved WF of [2].

# Discussion

## NS handling

There was an opinion that normally RAN4 introduces a new NS signalling when a new out of band emission which requires A-MPR e.g., NS_04. In fact, it is a valid point, while it can be said that the new NS was just technically needed because there is a country or a region to keep using e.g., the legacy general SEM.

Observation 1: RAN4 has introduced a new NS signalling when a new out of band emission which requires A-MPR such as NS_04. In fact, it is a valid point, while it can be said that the new NS was just technically needed because there is a country or a region to keep using e.g., the legacy general SEM.

When it comes to n104 ACLR, as far as we know, there has been no commercial UEs in the field so that it wouldn’t have clear benefit to keep holding the existing ACLR of 30 dB for n104. Thus, we think that it would be good to simply apply ACLR of 26 dB to n104 instead of adding 26 dB with a new NS on top of the existing 30 dB.

Observation 2: There has been no commercial n104 capable UEs so that it wouldn’t have clear benefit to keep the existing ACLR of 30 dB for n104. Simply applying 26 dB to n104 instead of adding 26 dB with a new NS on top of the existing 30 dB makes specification impact the least.

Proposal 1: Apply 26 dB to n104 without a new NS

## MPR handling

Single carrier operation

As elaborated in [1], a new ACLR would enable MPR to be reduced by 0.5~1.0 dB for QPSK and 16QAM for DFT-s-OFDM and those for CP-OFDM, respectively for single carrier operation. The detailed data is seen in Annex 1.

Observation 3: For single carrier, the new ACLR would enable MPR to be reduced by 0.5~1.0 dB as shown below.

For UL CA_n104C

Unlike MPR for PC2 Intra-band contiguous UL CA, PC3 intra-band contiguous UL CA has no differentiation between 1Tx and 2Tx.

Though we are open to discuss if RAN4 should differentiate the amount of MPR reduction between 1Tx and 2Tx or not, we performed MPR evaluation with the assumption of 1Tx and contiguous RB allocation. As a result, the new ACLR would enable MPR to be reduced by 0.5~1.0 dB for QPSK and 16QAM for DFT-s-OFDM and those for CP-OFDM, respectively depending on inner or outer allocation. The detail data is shared in Annex.

Observation 4: For contiguous RB allocation for UL CA_n104C, the new ACLR would enable MPR to be reduced by 0.5~1.0 dB as shown below.

For non-contiguous RB allocations, this is a bit complicated than contiguous RB allocation. Though we didn’t provide data on non-contiguous RB allocation, we are open to consider it if other companies provide data.

## MSD due to cross band isolation for CA_n78-n104

According to the past discussion to have derived MSD due to cross band isolation from n104 UL to n78 DL, companies considered that the frequency distance between aggressor (n104) and victim (n78) is sufficiently large and the noise due to PA is flat over n78 and the value of -130 dBm/Hz or -120 dBm/Hz was taken by companies’ preference [3, 4]. In our view, even if ACLR of 30 dB is replaced with 26 dB, the situation stays. Therefore, the same MSD can be kept.

Observation 5: When MSD due to cross band isolation from n104 UL to n78 DL was derived, noise floor due to PA over n78 was considered to be flat and the value of -130 or -120 dBm/Hz was used by some companies.

Proposal 3: Modification of MSD due to cross band isolation from n104 UL to n78 DL is not needed, even if ACLR of 30 dB is replaced with 26 dB.

# Summary

We obtained following observations and proposal. Companion CRs [5, 6] are also provided in this meeting.

Observation 1: RAN4 has introduced a new NS signalling when a new out of band emission which requires A-MPR such as NS_04. In fact, it is a valid point, while it can be said that the new NS was just technically needed because there is a country or a region to keep using e.g., the legacy general SEM.

Observation 2: There has been no commercial n104 capable UEs so that it wouldn’t have clear benefit to keep the existing ACLR of 30 dB for n104. Simply applying 26 dB to n104 instead of adding 26 dB with a new NS on top of the existing 30 dB makes specification impact the least.

Proposal 1: Apply 26 dB to n104 without a new NS

Observation 3: For single carrier, the new ACLR would enable MPR to be reduced by 0.5~1.0 dB as shown below.

Observation 4: For contiguous RB allocation for UL CA_n104C, the new ACLR would enable MPR to be reduced by 0.5~1.0 dB as shown below.

Observation 5: When MSD due to cross band isolation from n104 UL to n78 DL was derived, noise floor due to PA over n78 was considered to be flat and the value of -130 or -120 dBm/Hz was used by some companies.

Proposal 3: Modification of MSD due to cross band isolation from n104 UL to n78 DL is not needed, even if ACLR of 30 dB is replaced with 26 dB.

# References

[1] R4-2510200, MPR reduction for upper 6GHz by ACLR relaxation, Huawei, HiSilicon, RAN4#116

[2] R4-2511799, WF on MPR reduction for n104, Huawei, HiSilicon, RAN4#116

[3] R4-2405450, Requirements for CA_n78A-n104A, Qualcomm France, RAN4#110bis

[4] R4-2405325, Discussion on RF requirements for CA_n78A-n104A, Huawei, HiSilicon, RAN4#110bis

[5] R4-2521269, (NR_6GHz-Core) CR to TS 38.101-1: Correct the Single Carrier Tx requirements for n104, Huawei, HiSilicon, RAN4#117

[6] R4-2521272, (NR_6GHz-Core) CR to TS 38.101-1: Correct the intra-band CA Tx requirements for n104、Huawei, HiSilicon, RAN4#117

# Annex 1 for Single Carrier

## SC QPSK

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## SC 16QAM

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## SC 64QAM

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## SC 256QAM

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

For reference: Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. |  |  |  |  |

# Annex 2 for Carrier Aggregation

## CA QPSK (contiguous RB allocation)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## CA 16QAM (contiguous RB allocaion)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## CA 64QAM

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

## CA 256QAM

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

For reference: Table 6.2A.2.1-1 Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |