3GPP TSG RAN WG4 Meeting #117		                 R4-2520256

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on enhanced BS co-location requirements

Agenda Item:	7.2.3

Source: 	CATT

Document for:	Discussion

1 Introduction

Enhancement of co-location requirements - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any

According to the WF[1] of RAN4#116bis meeting, at this point, give equal priority to both option 1 and option 2 of enhance OTA co-location requirements. In this contribution, we present our views on the works for this objective.

2 Discussion

2.1 General

It is anticipated that the two potential approaches for deriving enhanced OTA co-location requirements—namely an improved CLRA-based method or one relying on existing OTA metrics—will be compared and investigated with respect to their impact on the four OTA co-location requirements. From the beginning, it should be clarified that the final four enhanced OTA co-location requirements must be defined using a single unified approach in order to reduce test costs; in other words, RAN4 should avoid a situation where some requirements are based on one approach while others are based on the other.

Proposal 1: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs.

Once the final enhanced OTA co-location requirements are defined, compliance may be declared based on the vendor’s BS design.

Proposal 2: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified.

CLTA is a specially designed test antenna aimed at accurately simulating the electromagnetic coupling effects between two BS antennas in close proximity during real network deployment within a laboratory environment, but there are some issues about CLTA,

- Definitions of CLTA may vary across different frequency bands.

- Specific CLTAs might only be available from a few suppliers, posing supply risks.

- The customization and production costs of customized CLTAs are ultimately passed on to equipment testing costs.

- Slight variations between CLTAs used in different labs can affect test result consistency.

- Calibrating CLTA is complex, increasing test uncertainty.

- The CLTA concept is difficult to implement effectively in high-frequency bands.

- CLTA testing typically requires complex setup and calibration processes.

- Maintaining a large inventory of CLTAs for different frequency bands and models is cumbersome.

1-O testing results incorporate the performance of the entire system, this is crucial for highly integrated AAS. In addition, the OTA test results are more accurate and reliable. Moreover, it's more suitable for high frequencies.

Based on above analysis, for upper frequencies of FR1, Option 2 can offer better performance in terms of accuracy, reliability, cost, test efficiency and high frequencies.

Proposal 3: For upper frequencies of FR1, Option 2 may offer better performance in terms of accuracy, reliability, cost, test efficiency and high frequencies.

3 Conclusion

In this contribution, we have the following proposals for enhanced BS co-location requirements:

Proposal 1: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs.

Proposal 2: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified.

Proposal 3: For upper frequencies of FR1, Option 2 may offer better performance in terms of accuracy, reliability, cost, test efficiency and high frequencies.

Reference

[1] RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei)

[2] R4-2515091, “Way-Forward for NR_BS_RF_Ph2”, Ericsson, RAN4#116bis

[3] 3GPP TR 25.942 v6.0.0, “Radio Frequency (RF) system scenarios”

[4] R4-2510227, “On improvements of co-location requirements”, Ericsson

[5] RP-252586, “CR to TR 37.941: Rel-19 CLTA-related agreements”, Huawei et., al.