3GPP TSG-RAN WG4 Meeting #117	R4-2520262

# Dallas, TX, US, Nov. 2025

Title:	On US 4.9 GHz UE RF requirements

Source:	Qualcomm, FirstNet, APCO, AT&T

Agenda item:	5.11.4

Document for:	Discussion

# Background

A new Rel-20 WI [1] was created to ‘allow for public safety operation in line with the framework identified by the FCC within the frequency range, 4940-4990 MHz, within 5G NR’, for which ‘a new NR TDD operating band needs to be defined’.

In a previous WF [8], some aspects of the FCC regulation were translated to 3GPP language, an important step to establish a common understanding in RAN4. In this contribution, we look at impact of the agreed emissions limits on UE configurations of channel bandwidth, by power class.

# Discussion

## Channel bandwidth and power class pairing restrictions

### 2.1.1	FCC Mask M reference level

RAN4 have agreed [8] that the 0 dB reference level associated with FCC mask M depends on the maximum output power for the UE’s power class:

This agreement is related to FCC regulation [4]:

“The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission”

The implication is that UE must demonstrate that its highest emissions-compliant Tx power can indeed equal the UE power class expectation for some condition. More formally:

Observation 2-1: The UE’s ‘highest average power of the fundamental emission measured across the designated channel bandwidth’ must equal the ‘power class max. output power’ for at least some allocation and modulation type. This is required to reconcile actual FCC regulation and the RAN4 agreement on FCC mask M reference level.

If the above condition cannot be guaranteed for any CBW and powerclass pairing, the RAN4 agreement on the 0 dB mask reference level is not valid, and therefore that CBW and powerclass pairing is invalid.

### 2.1.2	FCC PSD limit

Accordingly, RAN4 have agreed, based on regulation [8]:

For some power class and CBW pairings it is mathematically impossible for the UE to transmit at the ‘power class max. output power’ while staying under the PSD limit, as demonstrated earlier [7]. An example is PC1.5 (nominally 29 dBm) at the top of its output tolerance range (+2 dB) when configured for a 10 MHz channel (-9.5 dB). This problem condition is achieved even before real-world considerations like non-flat PSD is considered. While PC3 may be the focus of the WI as of this writing, it is necessary to at least prepare for introduction of other power classes in this work item.

Observation 2-2:  FCC part 90 PSD limit precludes certain combinations of power classes and configured channel bandwidths because the PSD even at full allocation and at ‘power class max. output power’ exceeds the FCC limit.

### 2.1.3	Summary

The implication of observations 1 and 2 is captured in the table below (exact boundaries will vary depending on assumption detail):

Both regulations, mask compliance and the PSD limit will be considered in future RAN4 A-MPR studies, because of agreements in [8]. Observations 1 and 2 can therefore be summarized as:

Proposal 2-1: A power class and channel bandwidth pairing is valid for US4.9 operation only if RAN4 can identify a waveform that is specified with both, 0 dB MPR as well as 0 dB A-MPR for at least one modulation.

### 2.1.4	Plenary facing issues

The general guidance from RANP and RAN4 leadership is to focus on PC3 and add HPUE through spectrum WI. The table above shows that any wording put in place based on PC3 alone (all green column for PC3) would be inadequate to accommodate HPUE and its associated validity boundaries (some red or pink cells).

Observation 2-3:  HPUE cannot be introduced for US4.9 via spectrum WI if core requirements are specified based on PC3 alone

Plenary guidance to use PC3 as the prototype UE power class for US4.9 was made prior to analysis of FCC regulation, and its implication on channel BW and power class pairing. It is logical to assume that plenary guidance may be optimized when presented with its ramifications. While this is not a RAN4 decision to make, the following proposal is made for awareness in RAN4, and perhaps to trigger discussion.

Proposal 2-2: Highlight at RAN plenary that HPUE cannot be introduced for US4.9 via a spectrum WI if core requirements are specified based on PC3 alone, and HPUE is better basis.

## 2.2	Frequency-offset for FCC Emission mask M

One regulation detail not captured by RAN4 for A-MPR analysis [8] pertains to the interpretation of ‘frequency removed from the assigned frequency’. The FCC Emission mask M regions defined by §90.210 states:

“On any frequency removed from the assigned frequency between 0-X% of the authorized bandwidth (BW)...”

For compactness, we refer to this quantity as mask frequency offset. In our view, the intent is for the mask to fit over the intended transmission of the channel, and for the roll-off to apply immediately outside the channel.

To reconcile the intent with the wording, the offset would have to apply to frequencies starting from the center frequency of configured UE channel bandwidth, unlike the 3GPP General SEM that applies to FOOB starting from channel edges.

Proposal 2-3: RAN4 interprets the FCC §90.210 mask parameter ‘frequency removed from the assigned frequency’ as frequency offset referenced to the center frequency of the configured UE channel bandwidth.

## 2.3	UE hardware assumption

The WID sets out this objective: ‘Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible’. From a UE perspective, since US4.9 is a subset of n79, reuse of hardware is possible, and a valid assumption. The transmit side requirements may impose new challenges (owing to different SEM) but that will be handled separately via NS. On the receive side, the implication of reusing n79 hardware is also adoption of n79 jammer requirements. While this assumption is equivalent to establishing a relaxation, it can be considered to enable relatively low-cost support on the UE side. Note that the relaxation only increases the susceptibility of the UE to interferers outside the channel, it does not cause more interference to neighbors.

Proposal 2-4: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

## 2.5	CA operation

In the US, n79 is not available for mobile use in its entirety. US4.9 is motivated by a desire to exploit spectrum overlapping with n79 for non-public safety operations. Consequently, there should be no occasion for the UE to support inter-band CA between n79 and US4.9.

Proposal 2-5: There is no need for UEs to support inter-band CA between n79 and US4.9.

The max. available spectrum in US4.9 is 50 MHz, which can be serviced by a single channel. There is hence no need to define intra-band CA in US4.9

Proposal 2-6: Intra-band CA in US4.9 is not enabled for Rel-20.

# 3. 	Simulation considerations

## 3.1	Simulation assumptions

The RAN4 agreements made on the FCC mask [8], and proposal 3 here on frequency offset, provide a good starting point for simulations. One aspect that needs further consideration is the FCC peak PSD limit which mandates that no device exceed 21 dBm/MHz. Real world UEs on the other hand must contend with imperfect power regulation, spectrum non-flatness, etc. To comply with the upper limit imposed by regulation, the UE must aim for a lower max. PSD. Now, 3GPP already provide for a tolerance on maximum UE output power (typically +2/-X dB). The +2 number is relevant and is proposed to be adopted.

The following simulation proposals can therefore be made for the A-MPR specification activity:

Proposal 3-1: For US4.9, and its channel BWs (10, 15, 20, 25, 30, 40 ,50 MHz), the following simulation parameters are assumed for study of additional required back-offs:

| Parameter | Value | notes |
| --- | --- | --- |
| Power class | (up to company preference) | PC3 may not be sufficiently representative of the challenges foreseen for other power classes |
| Emissions limits | See WF R4-2515103 [8] |  |
| Peak UL PSD target | 19 dBm/MHz | FCC PSD limit (21 dBm/MHz) – PSD tolerance (2 dB) |
| IQ Image | -28 dBc |  |
| LO leakage | -28 dBc |  |
| CIM3 | -60 dBc |  |
| PA Calibration point | 1dB MPR DFT-s-OFDM QPSK 20MHz CBW, 15k SCS, 100RB0 | Legacy method |
| Modulations | DFT-s-OFDM | To determine if a CBW/power class pairing is valid per FCC rules, RAN4 must identify waveforms that have both 0 dB MPR as well as 0 dB A-MPR |
|  | CP-OFDM | Legacy procedure to determine necessary back off |

## 3.2	Preliminary simulation results & AMPR formulation

### 3.2.1	Analysis strategy

Corner cases 1

Corner case 2

In this WI on enabling US4.9, it is prudent to be aware of any challenges that may show up for any power class of UE, even if the WI may focus on only one. While this strategy greatly increases the number of conditions to study, it is possible to get a high-level view by focusing on the corner cases.  Corner cases can be defined based on cases with the highest PSD (yellow) and with the lowest (purple). These are shown to the right, bordering the green areas.

### 3.2.2	Analysis of high PSD case (‘corner case 1’)

The table above shows multiple candidates (yellow cells) for study the high PSD cases. For this study, PC2 15 MHz was chosen. Preliminary projections on required back-off are shown in the figure below for DFT-s-QPSK and CP-OFDM:

Figure 3.2.2-1: Required back off (dB) for a ‘high PSD’ example (PC2, 15 MHz)

The DFT-s-QPSK exercise confirms by inspection that allocations exist where both MPR and any A-MPR can be zero (grey areas), validating the power class-CBW pairing.

Observation 3-1: PC2 15 MHz is a valid powerclass-CBW pairing that can meet FCC part 90 regulations.

The CP-OFDM backoff pattern for PC2 UEs with a configured BW of 15 MHz show three distinct motivating mechanisms:

Figure 3.2.2-2: Motivations for back off (dB) for a ‘high PSD’ example (PC2, 15 MHz)

3

1

2

The mechanisms are outlined below:

- PSD limit: When the UL allocation becomes narrower, the UE’s output power is concentrated in fewer RBs, and eventually it is unable to transmit at full power while staying under the FCC peak PSD limit. In the illustration above, approximately 7 dB back off is needed for the 3RB allocation to become PSD compliant.

- Insufficient guard band in channel: When the allocation is moved towards one of the channel edges, some RBs fall outside the part of the mask where no attenuation is needed (outside the ‘flat part’ in the middle 90% of the channel). The only option here is to back off the UL signal so it is below the mask limit line..

- General mask compliance: Even when conditions 1 and 2 do not apply, the distortion products generated by the PA can exceed the mask limit lines as illustrated.

### 3.2.3	Analysis of low PSD case (‘corner case 2’)

The figure below shows the pattern of required back off for PC3 50 MHz with CP-OFDM UL. Analysis of the pattern reveals a fourth motivating mechanism in addition to the three identified above for the high PSD case:

- Distortion products from LO and image: Mixing products between the desired signal and LO and image components can exceed the mask limit.

4

Figure 3.2.3-1: Additional motivation for back off (dB) for a ‘low PSD’ example (PC3, 50 MHz)

Observation 3-2: For US4.9, multiple different mechanisms motivate backoff, and each adds a unique pattern of additional back-off requirement in waveform space (for example: banding at the bottom, very high backoff for allocations with edge-most RBs, etc).

Figure 3.2.5-1: Example of ‘regions’ in waveform space used for specifying A-MPR

3.2.5	A-MPR strategy for US 4.9

A common legacy RAN4 method for specifying A-MPRs divides up waveform space into multiple regions and assigns a fixed value to each region. This strategy is simple but can often be conservative because the A-MPR for each region is determined by the worst-case allocation in the region. For US4.9 where the required back off has multiple steep gradients due to multiple physical mechanisms that manifest in different ways, this may be especially detrimental.

In parallel discussion on the 6G SI, there is desire to improve A-MPR specification. As a stepping stone to 6G, an alternative to the legacy method of region-wise constant A-MPR is a phenomenological approach that allows A-MPRs to vary in each region. The intent is for the specified A-MPR to better track the back-off needed by the UE, rather than the legacy method. If the phenomenological approach cannot significantly improve on the region-wise constant-AMPR approach, there is not much motivation to pursue.

In this phenomenological method, all the underlying mechanisms that require backoff are first identified, then a back-off assigned to each. The back off may be a constant value, or some simple function of the usual parameters (RBstart, RBend, LCRB, etc) and may apply to the whole triangle or only a region. The applicable back-off allowance for the UE would be from some combining method across the backoffs for all identified mechanisms. Specifically in this case where 4 independent mechanisms have been identified, the A-MPR could be specified as follows:

AMPR=MAX(M1,M2, M3, M4)

Where: Mx are ‘component’ backoffs assigned to each mechanism.

These component backoffs can be established based on the behavior of the underlying physical mechanisms. For example, by mechanism:

- PSD limit: The back off to comply with a PSD limit is expected to increase if the power class is higher and decrease if the allocation is wider. The expected contribution of this term (dB) is therefore of the form  M1=some constant+Ppowerclass-10log10(LCRB), which can be simplified further (at the expense of some tracking accuracy):

  - M1=k1-LCRB. (with lower bound of 0)

  - The parameter ‘k1’ depends on CBW and power class, as well as possibly modulation type.

- Insufficient guard band in channel: Since back off for this mechanism is driven mainly by RBs ‘falling off’ the flat part of the mask, the contribution for this term would apply in some regions, like in the legacy method. Specifically, for the left edge (right is similar due to symmetry):

  - M2 = (say) 20 dB, if RBstart < k2a (similar expression is needed for allocations against the right edge)

  - The parameter ‘k2a’ depends on CBW and power class.

  - The general idea is to discourage allocations that will have ‘spill over’ beyond +/- 45% CBW frequency offset from the center

- General mask compliance: The approach for this term is graphical, with detail in the annex. The expression is of the form (at least for allocations closer to the left edge, right edge would be similar):

  - M3=LCRBRBstart-k3b*k3a. (with lower bound of 0 and upper bound of parameter k3c)

  - The parameters ‘k3a’, ‘k3b’ and ‘k3c’ depend on CBW and power class

- Distortion products from LO and image: This term too benefits from a graphical approach, detail in the annex. The important difference here is that the back off in the region can vary rather than forced to adopt a constant value. The expression is of the form (at least for allocations closer to the left edge, right edge would be similar):

  - M4=k4a*{LCRBRBstart-k4b-k4c}. (with lower bound of 0)

  - The parameters ‘k4a’, ‘k4b’ and ‘k4c’ depend on CBW and power class, as well as possibly modulation type.

Proposal 3-2: RAN4 to study relative merits of specifying A-MPR for US4.9:

- Either using region-wise constant A-MPR (legacy method)

- Or allowing the A-MPR to vary in each region (to better track required back-off)

### 3.2.5	A-MPR by modulation type

The figure below shows relatively small change between DFT-s-OFDM variants, but a bigger jump from DFT-s-OFDM to CP-OFDM. To maximize UL system benefit, it would help to specify a lower set of A-MPRs for DFT-s-OFDM.

Figure 3.2.5-1: Back off (dB) by modulation type for a ‘high PSD’ example (PC2, 15 MHz).

DFT-s-QPSK, DFT-s-256QAM, CP-OFDM

Proposal 3-3: RAN4 to study relative merits of specifying different A-MPRs for US4.9, for DFT-s-OFDM and CP-OFDM.

# 4.	Conclusions

Observation 2-1: The UE’s ‘highest average power of the fundamental emission measured across the designated channel bandwidth’ must equal the ‘power class max. output power’ for at least some allocation and modulation type. This is required to reconcile actual FCC regulation and the RAN4 agreement on FCC mask M reference level.

Observation 2-2:  FCC part 90 PSD limit precludes certain combinations of power classes and configured channel bandwidths because the PSD even at full allocation and at ‘power class max. output power’ exceeds the FCC limit.

Proposal 2-1: A power class and channel bandwidth pairing is valid for US4.9 operation only if RAN4 can identify a waveform that is specified with both, 0 dB MPR as well as 0 dB A-MPR for at least one modulation.

Observation 2-3:  HPUE cannot be introduced for US4.9 via spectrum WI if core requirements are specified based on PC3 alone

Proposal 2-2: Highlight at RAN plenary that HPUE cannot be introduced for US4.9 via a spectrum WI if core requirements are specified based on PC3 alone, and HPUE is better basis.

Proposal 2-3: RAN4 interprets the FCC §90.210 mask parameter ‘frequency removed from the assigned frequency’ as frequency offset referenced to the center frequency of the configured UE channel bandwidth.

Proposal 2-4: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

Proposal 2-5: There is no need for UEs to support inter-band CA between n79 and US4.9.

Proposal 2-6: Intra-band CA in US4.9 is not enabled for Rel-20.

Proposal 3-1: For US4.9, and its channel BWs (10, 15, 20, 25, 30, 40 ,50 MHz), the following simulation parameters are assumed for study of additional required back-offs:

| Parameter | Value | notes |
| --- | --- | --- |
| Power class | (up to company preference) | PC3 may not be sufficiently representative of the challenges foreseen for other power classes |
| Emissions limits | See WF R4-2515103 [8] |  |
| Peak UL PSD target | 19 dBm/MHz | FCC PSD limit (21 dBm/MHz) – PSD tolerance (2 dB) |
| IQ Image | -28 dBc |  |
| LO leakage | -28 dBc |  |
| CIM3 | -60 dBc |  |
| PA Calibration point | 1dB MPR DFT-s-OFDM QPSK 20MHz CBW, 15k SCS, 100RB0 | Legacy method |
| Modulations | DFT-s-OFDM | To determine if a CBW/power class pairing is valid per FCC rules, RAN4 must identify waveforms that have both 0 dB MPR as well as 0 dB A-MPR |
|  | CP-OFDM | Legacy procedure to determine necessary back off |

Observation 3-1: PC2 15 MHz is a valid powerclass-CBW pairing that can meet FCC part 90 regulations.

Observation 3-2: For US4.9, multiple different mechanisms motivate additional backoff, and each adds a unique pattern of back-off requirement in waveform space (for example: banding at the bottom, very high backoff for allocations with edge-most RBs, etc).

Proposal 3-2: RAN4 to study relative merits of specifying A-MPR for US4.9:

- Either using region-wise constant A-MPR (legacy method)

- Or allowing the A-MPR to vary in each region (to better track required back-off)

Proposal 3-3: RAN4 to study relative merits of specifying different A-MPRs for US4.9, for DFT-s-OFDM and CP-OFDM.

# 5.	References

[1] RP-251887, “New WID on Introduction of NR TDD 4.9GHz Band for US Operation”, Fujitsu, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] R4-2510412, “On adjacent spectrum protection requirements for the 4940-4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

[3] R4-2510429, “Coexistence study for introduction of US 4.9GHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, 25th – 29th August, 2025

[4] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I/section-90.210

[5] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1215

[6] R4-2510411, “General aspects regarding the 4940–4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

[7]	R4-2514507, ‘On US 4.9 GHz UE RF requirements’, Qualcomm, RAN4 #116-Bis, Oct. 2025

[8]	R4-2515103, ‘Way Forward for [116bis][322] NR_TDD_4.9GHz_band’, Fujitsu, RAN4 #116-Bis, Oct. 2025

# 6.	Annex

## 6.1	Study on back-off requirements for ‘mechanism 3’

Mechanism 3 referenced in section 3.2.2 pertains to mask compliance for waveforms that do not have the more pressing concerns of PSD compliance or of allocated RBs ‘falling off’ the mask (i.e outside +/- 45% frequency offset). In the figure panel below, back off projections previously presented for the two corner cases studied are reproduced here for convenience.

1.4

2.5

6.0

1.2

2.2

4.0

Figure 6.1-1: Back off (dB) for a ‘high PSD’ example (PC2, 15 MHz) and a ‘low PSD’ example (PC3, 50 MHz).

DFT-s-QPSK, DFT-s-256QAM, CP-OFDM

Slope

Slope

A-MPR increases

The graphical constructs illustrate that for allocations closer to the left edge, a slope-based MPR specification suffices. A similar construct is expected to be feasible based on RBend and LCRB, due to symmetry. In general, the required back-off for mechanism 3 has the following trends:

- It seems to take on non-zero values for slopes greater than some cutoff value (for example 1.2 for PC3 50 MHz), and zero dB for lower slope values.

- It increases with increasing slope, although the actual value does not equal the slope (some scaling is required)

- It must be upper bounded since the slope can approach infinity for wide allocations, but back-off requirement remains reasonable (for example 6 dB range)

A simplified mathematical expression ‘M3’ for the back-off associated with mechanism 3 for allocations closer to the left edge would therefore use the slope value of an allocation beyond the cutoff value and then scaled to achieve the final value. In the PC3 50 MHz example, something like this would work:

M3_left=LCRBRBstart-1.2*2

More generally:

M3_left=LCRBRBstart-k3b*k3a

Where:

k3b = slope cutoff value, for example 1.4 for PC2, 15MHz CBW

k3a = scaling factor

A similar expression would apply to allocations closer to the right edge (not derived). M3 requires additional lower-bounding by 0 and upper-bounding by a third parameter ‘k3c’ which seems to be in the 6 dB range. Note that the procedure described above is illustrative, but specification-quality work will require a common set of simulation details.

## 6.2	Study on back-off requirements for ‘mechanism 4’

Mechanism 4 referenced in section 3.2.3 pertains to mask compliance for waveforms with prominent mixing products between the desired signal and LO leakage or between the desired signal and its image. In the figure panel below, back off projections previously presented for the low PSD corner case is reproduced here for convenience.

Figure 6.2-2: Example back off for mechanism 4

Slope

A-MPR increases

Slope = 1.3, A-MPR = 0 dB

The severity of the problem reduces if either the allocation is farther away from the channel edge or the allocation is wide enough. A detailed look however reveals that the back-off required for this mechanism (dB) is better captured as a function of slope to a point on the x-axis (RBstart = 45 in this example). In this example, any slope more negative than -1.3 needs back off, where ‘slope’ belongs to line through (45,0) and the allocation’s (RBstart, LCRB).

A simplified curve fit can be captured as:

M4=7*{LCRBRBstart-45+1.3} , for RBstart <= 44

A more general expression can be written as:

M4=k4a*{LCRBRBstart-k4b-k4c}

Where:

k4a = back-off scale factor (7.0 in this example)

k4b = Intercept on RBstart axis of ‘lines of constant backoff’ (45 in this example)

k4c = Slope of line where back off due to mechanism 4 is 0 dB (1.3 in this example)

applicability of this expression is limited to RBstart range of 0 to (k4b -1)

Note that the procedure described above is illustrative, but specification-quality work will require a common set of simulation details.